- Objective

To lay down the procedure for preparation of detergents and disinfectants solution.

- Scope

This SOP is applicable to for preparation of detergents and disinfectants solution in Area of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP.

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Procedure for Preparation of 0.2% v/v Teepol or Liquid Soap solution in Purified water (10.0 litres):

    - Take cleaned transparent graduated plastic 100 ml measuring cylinder.

    - Take approx 8.5 litres of purified water in a cleaned 15.0 litres capacity graduated stainless steel vessel.

    - Take 20 ml of Teepol or liquid soap solution in the 100 ml measuring cylinder.

    - Pour this 20 ml Teepol or liquid soap solution into the stainless steel vessel, which contains 8.5 litres purified water.

    - Rinse the measuring cylinder with purified water and pour into the stainless steel vessel.

    - Finally make up the volume up to 10 litres by adding purified water.

    - Stir the solution with stainless steel rod.

    - Close the lid of the container, labelled it and store for up to 24 hours.

    - Destroy the remaining quantity of solution which is not used within 24 hours.

    - Record the details of Preparation and destruction of Detergents in Annexure-I.

  - Procedure for Preparation of 2.5% v/v Dettol or Savlon solution in Purified water (10.0 litres)

    - Take cleaned transparent graduated plastic 1000 ml measuring cylinder.

    - Take approx 8.5 litres of purified water in a cleaned 15.0 litres capacity graduated stainless Steel vessel.

    - Take 250 ml of Dettol or Savlon in 1000 ml measuring cylinder.

    - Pour this 250 ml Dettol or Savlon into the stainless steel vessel, which contains 8.5 litres purified water.

    - Rinse the measuring cylinder with purified water and pour into the Stainless Steel vessel.

    - Finally make up the volume up to 10 litres by adding purified water.

    - Stir the solution with Stainless steel rod.

    - Close the lid of the container, labelled it and store for up to 24 hours.

    - Destroy the excess quantity of solution, if  not used within 24 hours.

    - Record the details of Preparation and destruction of Disinfectants in Annexure-I.

  - Procedure for preparation of 70 % v/v IPA in purified water (5.0 litters)

    - Take cleaned transparent graduated plastic 1000 ml measuring cylinder.

    - Take approx 1.0 litres of purified water in a cleaned 10.0 litres capacity-graduated stainless Steel vessel

    - Take 3.5 litres of IPA through 1000 ml measuring cylinder.

    - Pour this 3.5 litres IPA into the stainless steel vessel, which contains 1.0 litres purified   Water.

    - Rinse the measuring cylinder with purified water and pour into the stainless steel vessel.

    - Finally make up the volume up to 5.0 litres by adding purified water.

    - Stir the solution with stainless steel rod.

    - Close the lid of the container, labelled it and store for up to 24 hours.

    - Destroy the excess quantity of solution which is not used within 24 hours.

    - Record the details of Preparation and destruction in Annexure-I.

- Annexure (S)

Annexure – I 	:      Preparation of detergents and Disinfectants.

- Reference (S)

NA

- Glossary

SOP    :	  Standard Operating Procedure

No.     : 	  Number

IPA     :       Isopropyl Alcohol

V/V     :       Volume by Volume

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |