- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for operation and cleaning of Fogger Machine.

# 3.0    Scopes

# The Procedure is applicable for operation and cleaning of Fogger Machine in sterile area of

# C Block

# 4.0   Responsibility

Technician					:	Operation & Cleaning.

Officer and Above Production/ IPQA Officer	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

6.0   Procedure

  - Operation

6.1.1 Check the area and surroundings shall be clean.

6.1.2 Check the Fogger Machine shall be completely empty, if not then empty it, wash with WFI.

6.1.3 Fill the Fogger Machine completely with the desired Disinfectant.

6.1.4 Check the Speed button of the Fogger Machine and adjust it on the maximum speed if not adjusted.

6.1.5 Move the Fogger Machine in the areas to be sprayed/ fogged.

6.1.6 Switch ON the main switch to start the power supply.

6.1.7 Turn ON the Switch of Fogger Machine and do the fogging for 05 minutes and then switch OFF the Fogger Machine.

6.1.8 Move the Fogger Machine in the other areas of sterile side and do the fogging for 05 minutes in each room and two minutes for all change rooms.

6.1.9 When all the respective activity is over move the Fogger Machine to the buffer Zone of the Aseptic Area under LAF.

6.2    Cleaning procedure

6.2.1 After all the activities have been over, Switch OFF the Fogger Machine and main supply.

6.2.2 Clean the Fogger Machine with WFI and finally mop with 70% IPA solution.

    - Affix the CLEANED label on the machine.

7.0    Annexure (S)

NA

8.0  Reference (S)

NA

9.0   Glossary

SOP    : Standard Operating Procedure					.

LAF    : Laminar air flow.

IPA    : Iso-propyl Alcohol

NO    : Number.