- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To  lay down a procedure for operation and cleaning of Vial Washing M/c.

3.0    Scopes

This SOP is applicable for Vial Washing M/c installed in Vial Washing Area of C Block.

# 4.0	Responsibility

Technician					:	Operation & Cleaning.

Officer and Above Production/ IPQA Officer	:	Implementation Of Procedure.

5.0	   Accountability

Production Head.

6.0	  Procedure

  - Pre Startup

- Ensure that power supply is available.

- Ensure that cleanliness of machine and area.

- Ensure that supply of Purified water is available.

- Ensure that supply of Water for Injection is available.

- Ensure that supply of Compressed air is available.

  - Operation - For Line 1st:

6.2.1 Open the valve of purified water, Water for Injection and compressed air.

6.2.2 Turn ON the main supply, PLC will indicate water level low in circulated tank.

6.2.3  Press F11 to select manual mode.

6.2.4  Press F12 to by pass the water level sensor.

6.2.5  Press F1 to start the main drive. As main drive start moving press F3, F4, F8, to run the pump

of purified water and WFI tank and to collect the circulated water in recirculated tank.

6.2.6  Run the machine in manual mode and ensure that water is coming from every jet.

6.2.7  Press F11 to stop the machine.

6.2.8  Ensure working of Laminar airflow unit of vial washing machine and pressure differential is within the specified limit mentioned in the magnehelic gauge (8-15mm of wc ).

6.2.9  During operation ensure the pressure of purified water, WFI, Recirculated water and compressed air remains between 1.0 kg/cm2 to 2.5 kg/cm2.

6.2.10 Press F12 to detect the water level in all the tanks of vial washing machine.

6.2.11 Push the vials from empty vial inspection table towards vial washing machine allow them to come on the conveyor.

6.2.12 Press F15 to select AUTO mode.

6.2.13 Press F1 to start the machine in auto mode.

6.2.14 Wash all the vials of the batch in Auto mode.

6.2.15 On Completion of vial washing close the valve of purified water, WFI and compressed air line.

  - Operation - For Line 2nd:

6.3.1 Open the valve of purified water, Water for Injection and compressed air lines.

6.3.2 Press water level sensor by pass to detect the water level in all tanks of vial washing machine.

6.3.3 Press the manual mode on MMI.

6.3.4 Press Pump- I, II, III buttons for Purified water, WFI, Re-circulated water respectively and Air 	solenoid button for compressed air

6.3.5 After filling of tank, Press auto mode start button.

6.3.6 Press main drive to start the vial washing machine.

6.3.7 Wash all the vials of the batch in Auto mode.

6.3.8 On Completion of vial washing close the valve of purified water, WFI and compressed air line.

6.4   Change Over

6.4.1 Set the SS curtain of vial washing machine according to size of vial.

    - Set the guide plate of in feed conveyor according to size of vial.

6.5    Cleaning Procedure

6.5.1  At the end of operation clean the vial washing machine.

6.5.2 Turn OFF the main power supply. Open the side covers of the machine and remove the fallen glass pieces, vials and discard them.

6.5.3  Open the drain valve of all the tanks to drain out the water in storage tank of vial washing machine, clean them with freshly prepared 70% IPA (use within 72hrs) with lint free mops.

6.5.4  Clean the outer body in feed, out feed conveyor and pipelines of the vial washing machine with freshly prepared 70% IPA (use within 72 hrs) with lint free mops.

6.5.5  Clean the drain with mops and disinfectant solution used in the schedule of washing and   sterilization area.

6.5.6   Fix all the side covers of vial washing machine.

7.0     Annexure (S)

NA

8.0    Reference (S)

NA

9.0   Glossary

SOP  	: Standard Operating Procedure.

LAF		: Laminar airflow.

IPA		: Isopropyl Alcohol

NO     	: Number

PC	: Production (C–Block)