- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for operation and cleaning of Pyroklenz De- pyrogenation tunnel.

# 3.0    Scopes

This SOP is applicable for the sterilization and de-pyrogenation of washed vials of aseptic area     C Block.

4.0	   Responsibility

Technician					:	Operation & Cleaning.

Officer and Above Production/ IPQA Officer	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

6.0    Procedure

  - Prestart Up

6.1.1  Ensure the cleanliness of area.

6.1.2 Ensure the cleanliness of machine.

6.1.3 Ensure the power supply is available.

6.1.4 Ensure the AHU of washing and sterilization area is operational and differential pressure is

within the specified limit (4.0±0.5 mm of wc).

6.2	   Start –Up

    - Turn ON the main power supply.

6.2.2 Enter the batch no. press down arrow key four times and then press enter to enter the batch

no. place the cursor on batch no. and press right side arrow key once and then press enter. Now

enter the batch no. by using the key provided in MMI, after entry is over press enter. Press main

to return in main menu.

6.2.3 Enter the validated speed of conveyor of tunnel according to size of vials.

6.2.4 Set the height of diaphragm of sterilization zone as per height of the vials to be de-pyrogenated.

  - Operation

6.3.1  Press F1 to start the De-pyrogenation tunnel.

6.3.2 All the blower and turbine of the De-pyrogenation tunnel starts along with heaters. Check the   reading of maghenelic gauge of the de-pyrogenation tunnel. Differential pressure limits for dry zone (8 to 15 mm of wc), sterilization zone (18 to 25 mm of wc), cool zone (8 to 15 mm of wc), stabilization zone (8 to 15 mm of wc).

6.3.3  As the temperature of tunnel reaches to set value, conveyor starts moving. Start vial   washing machine to start the washing of the vials and allow the vial to get inside of the tunnel for sterilization and de-pyrogenation.

6.3.4 After all the vials of batch are De-pyrogenated and all the vials are passed on the turn table of aseptic area the diaphragm of the tunnel in aseptic area to be closed press F2 to stop the tunnel. Turbine and blower of LAF will stops automatically as temperature of the sterilization zone comes to 110C.

6.4    Change Over

6.4.1 Set the height of diaphram in both side of tunnel sterilization in & sterilization out according to

vial size.

6.4.2 Enter the new validated speed of tunnel according to size of vials.

6.4.3 Press key F5 PLC shows password………..

6.4.4 Enter the password, PLC shows LEVEL 1.

6.4.5 Press down the arrow key 7 times; at the stage of program press enter.

6.4.6 Press down the arrow key 2 times & press enter. Enter the validated speed by using the key provided in MMI.

6.4.7 On Completion of the entry, press enter. Press main to return in main menu.

6.4.8 Press F4 to logout.

6.5    Cleaning Procedure

6.5.1  At the end of daily operation clean the De-pyrogenation tunnel.

6.5.2 Turn OFF the main power supply. Open the side covers of the De-pyrogenation tunnel remove

any fallen vials and discard them.

    - Clean the conveyor belt and bottom of conveyor of the tunnel using 70% IPA with lint free mop.

    - Clean the outer body, exhaust duct using 70% IPA with lint free mop.

    - Fix all the side covers.

7.0    Annexure (S)

NA

8.0   Reference (S)

NA

9.0   Glossary

SOP   	: Standard Operating Procedure.

LAF		: Laminar air flow.

IPA		: Isopropyl Alcohol

MMI		: Man Machine Interface

PLC 		: Programmable logic control

MM		: Millimeter

PC		: Production (C-Block)