- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. Specification of the magnehelic gauge limit of LAF. 2. Increase hold time study of Sterilized Machine Parts And Accessories. | Due to magnehelic gauge limit of LAF was not Specified in old SOP’S. Due to change hold time study from 48 hours to 72 hours of Sterilized Machine Parts and Accessories. |

2.0    Objective

# To lay down a procedure for operation and cleaning of vial filling & bunging machine

# 3.0    Scopes

This SOP is applicable for operation and cleaning of vial filling & bunging machine in aseptic   area of C Block.

4.0	   Responsibility

Technician					:	Operation & Cleaning.

Officer and Above Production/ IPQA Officer	:	Implemented the Process.

5.0	   Accountability

Production Head.

6.0   Procedure

  - Prestart Up

6.1.1 Ensure cleanliness of filling area and machine.

6.1.2 Ensure that swab of machine and area is released in case of any product change over.

6.1.3 Ensure that power & Nitrogen/Co2 supply available and pressure NLT 1.5kg/cm2.

6.1.4 Ensure that Vacuum supply available and pressure NLT 500 mm of Hg.

6.1.5 Ensure that LAF is operational and the pressure differential is within the specified limits

(8 to 15 mm of wc).

6.1.6 Ensure that the temperature and RH of the filling room is controlled as per the product requirement.

6.1.7 Ensure that the sterilized Machine parts are kept in moveable LAF and use within 72 hrs, the pressure   differential of moveable LAF is within the specified limits (8 to 15 mm of wc).

6.1.8 Ensure that the sterilized Rubber stopper are available & kept under LAF and use within 72hrs,the pressure  differential of LAF is within the specified limits (8 to 15 mm of wc).

6.1.9 Ensure that the sterilized Aluminium seals are available & kept under LAF and use within 72 hrs, the   pressure differential of LAF is within the specified limits (8 to 15 mm of wc).

6.2	   Operation

6.2.1 Switch ON the main supply. The PLC will ask for password, enter the correct password and enter in main Menu.

6.2.2  Run the empty machine in inching mode to check the proper functioning.

6.2.3  Stop the machine and place the moveable LAF near the filling machine and unload the machine parts under LAF.

6.2.4  Remove the moveable LAF from the filling room to the buffer zone and ensure that the moveable LAF is working.

6.2.5  Fix the Piston in both of the Port Wheel.

6.2.6  Fix the Port Wheels on the machine, now run the machine in inching mode to check free movement of port wheel.

6.2.7  Fix hoppers with felts and mount them on the studs provided on the vial-filling machine. Affix the front felt with the help of screw.

6.2.8   Fix the side guards of port wheel with the help of bolts.

6.2.9   Mount the bowl of rubber stopper hopper on vibrator plate with the help of bolt, and then fix the chute of rubber stoppering assembly.

6.2.10  Run the empty machine to check the free movement of port wheel and other parts of vial filling machine.

6.2.11  Charge the rubber stopper bowl with sterile rubber stopper. Start the vibrator of the rubber stopper and allows the rubber stopper to fill the chute.

6.2.12  Open the diaphragm of de-pyrogenating tunnel as soon as vial reaches allow the vials to travel up to scrambler.

6.2.13  Start the scrambler to feed the channels of vial filling machine as both of the channels get  feeded, stop the scrambler.

6.2.14  Start the vial-filling machine to check the free movement of vials through the channels and check the plugging of the empty vials. Discard the vials in waste bin.

6.2.15  Open the valve of vacuum pump and open the valve of nitrogen/CO2 gas.

6.2.16  Mop the outer surface of the powder container with freshly prepared & filtered 70% IPA (use  within 72hrs) using sterile lint free mop, allow it to dry, open the lid of the container and charge the sterile powder in powder hopper of vial filling machine.

6.2.17  Run the machine in inching mode to fill all of the port of the wheel.

6.2.18  Select the auto mode and start the machine. Collect the vials from one to eight stations from both of the port wheel, stop the vial-filling machine. Weigh them on weighing balance note down the withdrawal content of each vial. Adjust the weight of each port, and to increase the weight rotate the piston anti-clockwise and to decrease the weight rotate the piston clockwise.

6.2.19  After adjustment again start the vial-filling machine in auto mode and collect the vials from one to eight stations from both the ports and stop the vial-filling machine. Check the withdrawal content of each vial and record. If all the vials fall with in the range start the vial filling in auto mode.

6.2.20  Feed the powder hopper with the sterile powder and bowl of rubber stopper with sterilized rubber stopper to get continuous supply of material for filling operation.

6.2.21  Run the machine till raw material of the batch gets completely filled.

6.3      Change Over

6.3.1   Set the SS guides cover of machine according to diameter of vial.

6.3.2   Set the port wheels and pistons according to dose of powder.

6.3.3   Set the height of filling & bunging assembly-according height of vials.

6.3.4   Change the vial separator according to dosing of powder (single & double dosing).

6.3.5   Set the dosing lever as per the No. of dosing required.

  - Cleaning Procedure

6.4.1   On Completion of the batch, switch OFF the vacuum pump and close the valve of nitrogen/Co2.

6.4.2   Unscrew the rubber stopper hopper discard the remaining rubber stopper in the bowl.  Remove the chute, place them on the trolley.

6.4.3   Remove the lid of the powder hopper remove the felt of the powder hopper unscrew both of the powder hopper one by one, place them on the trolley.

6.4.4   Unscrew both of the port wheels and place them on trolley. Send all the machine parts through dynamic exit pass box for cleaning and sterilization to unit preparation area.

6.4.5   Open the side covers of the machine and clean the machine with the help of vacuum cleaner to remove the powder on the surface of the machine and conveyor belt.

6.4.6   Clean the machine with 0.22 micron filtered water for injection, mop the surface to remove the traces of powder, use fresh filtered water for injection for second moping.

6.4.7  Finally mop the machine surface with freshly prepared 70% IPA solution.

7.0     Annexure (S)

NA

8.0    Reference (S)

NA

9.0   Glossary

SOP	   	: Standard Operating Procedure.

LAF		: Laminar air flow.

IPA		: Iso-propyl Alcohol

NO.     	: Number

NLT		: Not less than

mm		: Millimeter

RH		: Relative humidity

PLC		: programmable logic control

Co2		: Carbon di-oxide

PC		: Production (C-Block)