- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. Increase hold time study of Sterilized Machine Parts And Accessories. | Due to change hold time study from 48 hours to 72 hours of Sterilized Machine Parts and Accessories |

2.0    Objective

To lay down a procedure for operation and cleaning of Vial Sealing machine.

# 3.0    Scopes

This SOP is applicable for the operation and cleaning of Vial Sealing machine of aseptic area

C  Block.

# 4.0   Responsibility

Technician			:	Operation & Cleaning.

Officer and Above Production	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

6.0    Procedure

  - Prestart Up

6.1.1 Ensure that the cleanliness of the machine and vial sealing area.

6.1.2 Ensure that power supply available.

6.1.3 Ensure that the LAF is in working condition and pressure differential is within the specified limits

8-15 mm of water.

6.1.4 Ensure that the temperature and RH of the sealing room is to be monitored and start only if the temperature as per product requirement.

6.1.5 Ensure that the correct sterilized Aluminum seals are available for sealing & kept under LAF.

6.1.6 Ensure that the correct change parts are replaced and guides are set as per the container size (vial).

6.2    Operation

6.2.1 Mop the surface of vial sealing machine with freshly prepared & filtered 70% IPA (use within 72 hrs).

6.2.2 Switch ON the main supply, PLC will start.

6.2.3 Run the empty machine in inching mode to check the proper functioning.

    - Start the vibrator from MMI of the machine to check the proper functioning.

6.2.5 Charge the sterilized seals in the hopper of vial sealing machine. Start the vibrators of the vial-sealing machine, allow the seals to fill the chute then stop the vibrator.

6.2.6 Take sufficient stoppered empty vials and pass through the vial-sealing machine to ensure proper sealing and send sealed vials of each head to IPQA for leak test.

6.2.7 Discard the remaining vials after breaking the flip off portion in waste bin.

6.2.8 As filling and bunging operation starts and sufficient quantity of vials are present on scrambler, start the swing conveyor of vial filling and bunging room as the vials get feeded, start the vibrator and then start the sealing machine by pressing the ON switch provided on control panel of vial sealing machine.

6.2.9 Adjust the speed of sealing machine from speed port and also the feed rate of seals from the bowl

by adjusting the vibrator.

6.2.10 Feed the vibrator hopper with sterilized aluminum seal during operation for continuous running of the machine.

6.2.11 Continuous checking of sealed vials must be done for any defects till batch completion.

6.2.12 Perform the sealing operation till all the filled vials get sealed.

  - Change Over

6.3.1  Place the correct star wheel and guide plate according to vial size.

6.3.2  Place the feed worm according to vial size.

6.3.3  Adjust the height of machine according to vial height.

6.4   Cleaning Procedure

6.4.1 On Completion of the batch remove the remaining seals from the hopper of the vial sealing machine.

6.4.2 Remove any fallen seals from the machine surface; discard in waste bin, clean the machine surface

and conveyor belt with the help of vacuum cleaner.

6.4.3 Clean the machine with 0.22 micron filtered water for injection, mop all the surface of the machine

to remove the traces of powder and use freshly filtered water for injection for moping second time.

6.4.4 Finally mop the machine surface with disinfectant solution as per schedule and allow it to air dry.

7.0    Annexure (S)

NA

8.0   Reference (S)

NA

9.0   Glossary

SOP   	: Standard Operating Procedure.

LAF		: Laminar air flow.

IPA		: Isopropyl Alcohol

IPQA	: In Process quality assurance

PC		: Production (C-Block)