- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. Cassette of basket drives contains NMT 5500 bungs of 20 mm diameter. 2.104000 Aluminium seals distributed in nylon bags. 3. No. of Repeat wash-3 is zero. | Due to change in Qty. of bungs from 5000 to NMT 5500 in Cassette of basket drives. Due to change in Qty. 85000 to 104000 aluminium seals distributed in nylon bags. After validation, validation team found wash-2 is sufficient in bung processor cycle so wash-3 is not required. |

2.0    Objective

To lay down a procedure for operation and cleaning of bung processor.

3.0    Scopes

This SOP is applicable for the sterilization of articles and washing of rubber plugs to be used in    aseptic area C Block of Alkem Laboratories Ltd. Baddi.

# 4.0	Responsibility

Technician			:	Operation & Cleaning.

Officer and Above Production	:	Implementation of Procedure.

- Procedure

5.1 		Prestart Up

5.1.1 Ensure that the cleanliness of washing Area.

5.1.2 Check Laminar Air Flow device is operational and pressure differential is within the limit                                            (8-15 mm of water) mentioned in the magnehelic gauge.

5.1.3 Switch on the main and switch on the control switch also switch on the printer.

5.1.4 Select the mode from auto /manual mode selector switch.

5.1.5 Now press NST door open switch to open the NST door.

5.1.6 Clean the bung processor cum autoclave using 70% IPA and lint free mop before start of process.

5.1.7 Ensure that all the utilities are available.

5.1.8 Ensure that availability of material to be sterilized.

5.1.9 Ensure that availability of strip chart paper in strip chart recorder & Steam Clox.

5.2 	Start Up

5.2.1 To start the process follows the step mention below; PLC shows date, time and bung Processor.

5.2.2 Press 7 numeric key (down arrow) 5 times.

5.2.3 Now PLC will show: -

M/s alkem  labs ltd.

F4: main menu

Press F4   on  MMI PLC will show –

F1: SET  VIEW DATA

F3:  LOGIN, F4:  MENU 2

Press F4. -- PLC will show batch data.

F1: MONITOR  	F2: START

F3: BATCH          F4: MAIN >>

Now press F3 to enter in batch details before starting the operation.

5.2.4   Press down arrow numeric key 7, 5 times then press 9, then press enter the batch no by pressing no key continuously by pressing required nos. keys to enter the batch no. Now press enter key to enter the numbers.

5.2.5   Now press no 9 twice and enter the Qty of material loaded in similar was mention in above   point. Now press F4 to return in main menu.

5.3 	   Bung Process Cycle

5.3.1   Press F4 twice --- PLC will show

F1: MONITOR     F2: START

F3: BATCH         F4: MAIN >>

5.3.2   Press F 2 --- PLC will show

XXX  PROCESS.

F3: SELECT PROCESS

5.3.3   Now press F3 till the desire process will appears on the MMI screen. Press 7 down arrow key twice

Will show:

F3:  select process

F 2: print parameter

5.3.4   To print the parameter press F2, now press down arrow key 7 once PLC Screen will show

F2: Print parameter

F1: Start Process

5.3.5    Load the cassette of basket drives each unit with NMT 5500 bungs of 20 mm diameter.

5.3.6    Place the cassette in the basket after loading the cassette, lock the cassette with lock provided

in basket, move the trolley and align the trolley to move in bung processor cum autoclave.

5.3.7   Press basket drive switch to check the movement of the basket in side the unit.

5.3.8   Now press the NST door close switch to close the NST door and ensure that the precondition of

door, door precondition starts glowing.

5.3.9   Dissolve 75 gms. SLS in 3.0 liter of purified water and charge it in detergent port.

5.3.10 Open the valve of process air, compressed air purified water WFI and plant steam.

5.3.11 Before pressing F1 the press the power on switch of strip chart recorder and the mode selection          switch to auto mode. Now press F1 to start process. Process will start automatically and following cycle taken place.

WASH – 1

MACHINE WASH				3.0 MIN

DETERGENT IN 				1.0 MIN

FLUIDIZATION				10.0 MIN

STABILIZATION				2.0 MIN

PURIFIED WATER OVER FLOW 		3.0 MIN

DRAIN					8.0 MIN

MACHINE WASH				2.0 MIN

DRAIN					2.0 MIN

NO. OF RPTS 				1.0

Followed by wash – 2 with purified water.

WASH – 2

FLUIDIZATION				10.0 MIN

STABILIZATION				5.0 MIN

PURIFIED WATER OVER FLOW 		4.0 MIN

DRAIN					8.0 MIN

MACHINE WASH				2.0 MIN

DRAIN					2.0 MIN

NO. OF RPTS 				1.0

5.3.12    PLC will ask for repeat of wash – 2. Check the wash water for particulate matter and

Detergent from sampling point and if found absent then press F2 to escape the repeat it

as in wash –3.

WASH – 3

DLY. BEFORE DRAIN			6.0 MIN

DRAIN					8.0 MIN

MACHINE WASH				2.0 MIN

DRAIN					2.0 MIN

NO. OF RPTS 				0.0

5.3.13    Fill the silicon port with 80 ml silicon oil in silicon port before completion of drain in wash-2. Then siliconization phase will start, process follows as: -

SILICONIZATION

SILICON IN				1.0 MIN

SILICON SOAKING			15.0 MIN

DRAIN					7.0 MIN

MACHINE WASH				2.0 MIN

DRAIN					4.0 MIN

NO. OF RPTS 				1.0

5.3.14  Then the sterilization phase starts at 121.40c for 30 min, drying phase starts for 30 min after process   end the chamber pressure reaches to –0.060 bar and process on /end indicator starts glowing. Now select the manual mode for unloading operation.

5.3.15 To unload the washed, siliconized, sterilized and dried rubber plugs, press the OPEN switch towards sterile side to open ST door. As the door opens, place the trolley and remove the basket on the trolley under LAF, allow it to cool.

5.3.16  Remove the cassette and open the lid and unload the sterilized bung in presterilised SS container under LAF After unloading operation of bungs is over than place the basket in the bung processor with air in lock the basket, press the close switch, to close the ST door.

5.3.17 To remove the basket from the unit press the open switch control penal to open the door, place the trolley properly, lock it and remove the basket on the trolley, unlock the trolley and remove it. Clean the chamber of bung processor cum autoclave WFI and lint free mop.

  - Vacuum Leak Test

5.4.1  To start the Leak test cycle follows instruction up to 5.3.4.

5.4.2  Ensure that no material present in the chamber of autoclave during the process.

5.4.3  PLC screen will show     XXX PROCESS

F3: SELECT PROCESS

5.4.4	  Now press F3 till the vacuum leak test cycle appears on the screen. Press 7 down arrow key three times screen will show

F2: print parameter

F1: Start process

5.4.5 To print the parameter press F2. Ensure that mode selector switch is in auto mode and switch on the strip chart recorder.

5.4.6 Now press F1 to start the process. Press 7 down arrow key once MMI screen will show

F1: start process

F4: main screen

Press F4 to enter in main screen.

5.4.7  Now to monitor the process press F4 two times then press F1 to monitor the data.

5.4.8  To view data press 7 and 6 to scroll the cursor to view the data.

5.4.9  Vacuum hold for 10 minutes at –0.750 bar and process on /end indicator starts glowing indicate that the   process is completed.

    - Press the OPEN switch towards non-sterile side to open NST door. As the door open go for next process.

5.5     Bowie Dick test

5.5.1 To starts Bowie Dick test cycle follow below instruction up to 5.3.4.

5.5.2 Ensure that Bowie Dick paper present in the chamber of autoclave during the process.

5.5.3 Load the chamber with desire load along with the Bowie Dick kit. Press the NST door close switch to close the door. Now ensure that precondition of door achieved.

5.5.4 PLC screen will show ---

XXX process

F3: select Process

5.5.5 Press F3 till the Bowie Dick process appears on the MMI screen.

5.5.6 Press 7 down arrow key 3 times. MMI screen will show.

F2: Print parameter

F1: start process

5.5.7 To print the parameter press F2 and ensure that made selector switch is in auto mode and switch on the strip chart recorder. Now press F1 to start the process, press 7 down arrow key once MMI screen will show

F1: Start process

F4: Main Screen

Press F4 to enter in main screen.

5.5.8 Now monitor the process Press 7 down arrow key 5 times the press F4 Twice then press F1 to monitor the data, to view the data press 7 and 6 to scroll the cursor to view the data and process on /end indicator starts glowing indicate that the process is completed.

5.5.9 Press the OPEN switch towards non-sterile side to open NST door. As the door open go for next process.

5.6      Standard Cycle

5.6.1	To starts standard cycle follow below instruction up to 5.3.4.

5.6.2	PLC screen will show ---

XXX process

F3: select Process

5.6.3 Press F3 till the Standard process appears on the MMI screen.

5.6.4 Press 7 down arrow key 3 times. MMI screen will show.

F2: Print parameter

F1: start process

5.6.5 Press F2 to print data and then press F1 to start the process.

5.6.6 Press 7 down arrow key once, PLC screen will show

F1: start process

F4: Main Screen

Press F4 to enter in main screen.

5.6.7   Now Monitor the process Press 7 down arrow key 5 times the press F4 Twice then press F1 to monitor the data, to view the data press 7 and 6 to scroll the cursor to view the data.

5.6.8   Then the sterilization phase starts at 121.40c for 30 min, the chamber pressure reaches to –0.050 bar and process on /end indicator starts glowing.

5.6.9   In case of any alarm acknowledge the alarm as per the following, Press F5, followed by F4, and followed by F2.

5.6.10 Blinking of process ON/OFF indication will indicate cycle completion.

5.6.11 Turn the AUTO/ MANUAL switch in MANUAL mode.

    - Inform the Aseptic area personnel to unload the autoclave.

    - Sterile area personnel will do the following steps.

    - Press the sterile door open push button.

5.6.15 Wait for the door opening and unlock the trolley.

5.6.16 Unload the trolleys on the floor trolleys.

5.6.17 Unload the sterilized articles in mobile LAF.

5.6.18 Check for the colour change of the steam clox indicator and hand it over to washing area                      personnel and that will be attached to the concerned document.

5.6.19 In case colour of steam clox indicator was not changed repeat the sterilization cycle.

5.6.20 Load the empty trolleys in the guide rails of the steam sterilizer.

5.6.21 Lock the trolleys on the rails.

5.6.22 Close the sterilizer using door close push button.

5.6.23 Switch OFF the printer and strip chart recorder.

5.6.24 Remove the strip chart from the recorder and check for correctness of the cycle and attach in the concerned document.

5.6.25 In case of any discrepancy in the cycle repeat the cycle.

5.6.26 Remove the printout from the printer and check for correctness of the cycle and attach in the concerned document.

5.7       HPHV Cycle

5.7.1   To starts HPHV cycle follows instruction up to 5.3.4.

5.7.2   PLC screen will show ---

XXX process

F3: select Process

5.7.3   Press F3 till the HPHV cycle on the MMI screen.

5.7.4   Press 7 down arrow key 3 times. MMI screen will show.

F2: Print parameter

F1: start process

5.7.5   Press F2 to print data and then press F1 to start the process.

5.7.6   Press 7 down arrow key once, PLC screen will show

F1: start process

F4: Main Screen

Press F4 to enter in main screen.

5.7.7   Now monitor the process Press 7 down arrow key 5 times the press F4 Twice then press F1 to monitor  the data, to view the data press 7 and 6 to scroll the cursor to view the data.

5.7.8   Then the sterilization phase starts at 121.40c for 30 min, drying phase starts for 30 min after process    end the chamber pressure reaches to –0.060 bar and process on /end indicator starts glowing.

5.7.9   Now follows the instructions from 5.6.9 to 5.6.26 as in standard cycle.

5.7.10 Load pattern details of HPHV cycle.

Load pattern no. A	  :	104000   Aluminium seals in distributed in nylon bags.

Load pattern no. B   :	30 packs of garments with 10 packs in each rack, 1 pair of bootie in

1 pack with 10 packs in each rack.

Load pattern no. C	  :	Two Hoppers, two port wheels, 16 piston sets, felts, 4 scoops

Two buckets, membrane filter holder, SS container for storage of                               rubber plugs and aluminum seal, Blender Bin (as per requirement.)

5.8      Cleaning Procedure

5.8.1   At the end of daily operation sterilizer to be clean.

5.8.2   Remove the trolley from the steam sterilizer on floor trolley.

5.8.3   Clean the trolley with 70% IPA with of lint free mops.

5.8.4   Clean the outer body and pipeline of the sterilizer with 70 % IPA and lint free mops.

    - Clean the drain with mops and disinfectant solution used in the schedule of aseptic area.

6.0 	Annexure (S)

Annexure – I – Log Card for Bung Processor

7.0     Reference (S)

NA

9.0      Glossary

SOP   	: Standard Operating Procedure.

LAF	: Laminar air flow.

IPA	: Isopropyl Alcohol

NO.     	: Number

WFI	: Water for injection

ML		: Milliliter

GM    	  : Gram

NST	: Non Sterile

ST		: Sterile

SS		: Stainless Steel

SLS	: Sodium Lauryl Sulphate

PLC	: Programmable Logic Control

MMI	: Man Machine Interface

PC		: Production C-Block