- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To  lay down a procedure for operation and cleaning of aseptic area blender.

# 3.0    Scopes

This SOP is applicable for the operation and cleaning of aseptic area blender of aseptic area C Block.

# 4.0	Responsibility

Technician			:	Operation & Cleaning.

Officer and Above Production	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

6.0    Procedure

6.1	Prestart Up

- Ensure the cleanliness of main body of blender and area.

- Ensure the Rinse Water/swab of area and blender is released in case of any product 	change over.

- Ensure the blender is sterilized (use within 72 hrs.).

- Ensure the LAF of blending area is in working condition and reading of magnehelic gauge are within the specified limits (8-15 mm of wc).

- Ensure the environmental conditions are as per specified in BPCR.

- Ensure the raw material is available for blending.

- Ensure the weighing balance is in working condition and calibrated.

6.2      Start Up

    - Switch on the main switch.

    - Adjust the shaft of the main body so that the bin of the blender can be attached to the main body.

    - Fix the bin to the shaft of the main body of the blender with the help of clamp and tight it firmly.

    - Fix the butterfly valve of the bin with the help of clamp and tight it firmly, checks the movement of the valve and close it.

    - Close the top lid of blender with the help of clamp.

    - Start the switch for 5 minutes to check the free movement of the bin.

    - Stop the blender in up right position; Open the top lid of blender so that material could be charged in the bin.

    - After blending is over, unload the raw material in aluminium containers.

    - Switch OFF the main switch.

  - Cleaning Procedure

- Switch OFF the main supply.

- Open the clamp to dismantle the bin from main body.

- Place the bin in the stand provided for it and send it to the unit preparation area for cleaning through change room.

- Clean the main body of the blender and area first with the help of vacuum cleaner.

- Mop the main body with Filtered WFI followed by filtered disinfectant solution then clean the area afterwards.

- Dismantle the butterfly valve by opening the clamp remove the gasket, open the lid of the bin and remove the gasket.

- Clean the cage of the bin first using purified water.

- Clean the gaskets, butterfly valve, and lid with the help of purified water.

- Finally rinse all parts of blender including blender bin with WFI.

- Ensure that after final rinsing machine components should not be kept more than 4 hours. If the hold time is more than 4 hours then again go for rinsing with WFI before sterilization.

7.0       Annexure (S)

NA

8.0       Reference (S)

NA

9.0	   Glossary (S)

SOP   	: Standard Operating Procedure.

LAF		: Laminar air flow.

WFI		: Water for injection

IPQA		: In Process quality assurance

BPCR	: Batch production & control record

RPM		: Rotation per minute

BOM		: Bill of material.

mm		: millimeter

PC		: Production (C-Block