- Objective

To lay down a procedure for operation & calibration of Moisture analyzer.

2.0       Scope

This SOP is applicable to Production area of Alkem laboratories limited, Baddi.

3.0	Responsibility

Production Officer/Executive	:	Implementation of procedure

:	Documentation and data entry

4.0	Accountability

Head Production		:	System Compliance

- Operation

- Switch ON the main power plug of the “Moisture Balance”

- Switch on the Moisture Balance by pressing “On/off” button on the instrument.

- After some time main screen menu displayed.

- Open the sample pan holder by pressing “↨” open/close auto sample chamber and it C   V     automatic opens.

- Clean the sample chamber and pan holder properly with lint free cloth.

- Place the empty and cleaned sample pan in the sample pan handler.

- Place the sample panhandler in the automatic sample chamber. Ensure that the tongue of the pan handler fits exactly in its place.

- Press the “ O / T  ” key for tarring, the automatic sample chamber closes briefly then open again.

- After tarring, the status display flashes and prompts you to place the sample in the samp disk.

- Place the approximate 1g sample in the sample pan with spreading.

- The status display flashes and prompts you to ready for start.

- Press the ‘Start’ key and automatic sample chamber closes. The instrument starts the drying   and measurement.

- The status appears current temperature, sample weight, total time and % result on the    display.

- At the end of drying time, give an audio signal sounds and sample chamber is Automatically opened and result print out automatically comes out.

- You can read off moisture content of your sample in the display.

- Remove the sample panhandler carefully from the sample chamber.

- Clean the sample chamber and pan holder properly with lint free cloth.

- Close the sample pan holder by pressing “↨” key.

6     	Cleaning:

- Open the sample chamber by pulling it out. And remove the sample pan holder.

- Use a lint free cloth for cleaning

- Disconnect the instrument from power supply before cleaning.

- Open the sample chamber by pulling it out and remove the sample pan holder.

- Clean the exterior of the instrument and the compartment with that lint free cloth.

7.0       Annexure (S)

NA

# 8.0       Reference (S)

NA

9.0       Abbreviation (S)

SOP		: Standard Operating Procedure

No.		: Number