- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | Increase hold time study of Sterilized garments. | Due to change hold time study from 48 hours to 72 hours of garments. |

2.0    Objective

To lay down a procedure for Operation and cleaning of vial lifting table and Inspection table.

3.0    Scopes

This SOP is applicable for Operation and cleaning of vial lifting table and Inspection table in vial washing area of C Block.

# 4.0    Responsibility

Operation & Cleaning				:	Technician.

Officer and Above Production/ IPQA Officer	:	Implemented the Procedure.

5.0	   Accountability

Production Head.

6.0    Procedure

6.1	   Prestart up

Before starting of vial lifting table ensure the following:

6.1.1 Power Supply shall be available.

6.1.2 Compressed air Supply shall be available.

6.1.3 Vials shall be available.						.

6.1.4 Rejected vial or broken vial container shall be available.

6.2	   Operation

    - Area and surrounding shall be clean.

6.2.2 Vial lifting table and inspection table must be completely empty and must be clean before starting of the operation, if not then empty it and clean.

6.2.3 Turn the switch ON of the power supply.

6.2.4 Tube lights of the Inspection table shall be working.

6.2.5 Adjust the height of the lifting table using the level provided in the side of the table.

6.2.6 Load the decartoned vials on the lifting table and push them towards inspection table using SS plate.

6.2.7 Inspect carefully the vials, if any defect found reject them from washing and keep in the rejection box which is duly labeled.

6.2.8 The good vials shall be allowed for washing.

  - Cleaning Procedure

6.3.1 After all activities are over, switch OFF the power supply.

6.3.2 Clean the Vial lifting table and inspection machine with lint free duster.

    - Mop the Vial lifting table and inspection table with freshly prepared 70% IPA (use before 72hrs).

7.0     Annexure (S)

NA

8.0     Reference (S)

NA

9.0     Glossary

SOP   	: Standard Operating Procedure.

IPA		: Iso-propyl Alcohol

NO    	: Number

SS		: Stainless Steel

PC		: Production (C-Block)