- Objective

To lay down a procedure for Operation and Cleaning of Automatic Capsule Filling Machine (AF-150)

- Scope

The SOP is applicable for Operation and Cleaning of Automatic Capsule Filling Machine (AF-150) in of Alkem Laboratories Unit-II.

- Responsibility

Officer/ Executive: Concerned Department

- Accountability

Head Production

- Procedure

  - PRE-START UP

    - Remove the “STATUS” Label and affix “CLEANED” Label duly signed by Production Officer/ Executive.

    - There shall be seven digits alphanumerical numbering system for every room in plant.

    - Ensure proper functioning of machine lubrication system.

    - Ensure that all moving parts are free by rotating the machine clockwise manually

    - Ensure correct setting of Dosing Disk and pallet assembly.

    - Ensure correct setting of powder hopper.

    - Ensure correct setting of capsule separation vacuum.

    - Ensure that the gap between dosing disk and scraper plate shall be 0.05 to 0.1 mm

    - Ensure that the gap between cap bush and body bush shall be 0.1 mm.

    - Ensure that the stirrer of a powder hopper is rotating freely in the powder hopper.

    - Ensure that the empty capsule Sorter and elevator shall be working properly.

    - Ensure that the Filled Capsule Sorter and elevator shall be working properly.

    - Ensure that the DP machine shall be working properly.

    - Ensure that the area around the AF 150T machine shall be cleaned as per SOP No xxxx.

  - STARTUP

    - Bring the HDPE container along with double poly bags having “STATUS LABEL” Label duly signed by Production Officer/Executive.

    - Bring the IPC/BIN filled with granules to the cubicle which shall have proper status label and Approved label on it duly signed by IPQA Officer/ Executive.

    - Load the IPC/BIN with the Lifting and Positioning device and place it to the Hopper.

    - Open the butterfly valve of IPC/BIN and fill the hopper.

    - Set the hopper valve to allow the materials to flow into the Dosing disk.

    - Bring the EHG capsules to be filled to the cubicle and load into Empty capsule hopper.

  - OPERATION:

    - Open the compressed air valve.

    - Turn On the PLC button.

    - Press loading ON

    - Select mode depending on type of filled material. Select 1 for powder and 2 for pallets.

    - Press ESC to come out from the menu.

    - Set the speed of machine by rotating the tuning knob.

    - Capsule separations pressure shall be in between 15-25 inch of Hg.

    - Set the weight by adjusting the weight adjustment screw.

    - Clockwise rotation  -  Increases the weight

    - Anticlockwise rotation  - decreases the weight

    - Set the locking length by adjusting the screw located at station no. 8

    - Clockwise rotation -  increases the length

    - Anticlockwise rotation - decreases the length

    - Rotate the machine manually to ensure the smooth running.

    - Run the machine in auto mode.

    - Press Drive ON to run the machine.

    - Take the samples of capsules and check the parameters as per BPCR.

    - After getting satisfactory results collect the filled capsules in S.S container and then placed the capsule in HDPE drum and put “CONTAINER” Label on it.

  - SHUTDOWN:

    - Press drive OFF to stop the machine.

    - Dry clean the machine with lint free duster.

    - Close the valve of IPC.

    - Remove the powder/pallets from hopper.

    - Remove the empty capsules from hopper.

    - Destroy the rejected capsules by putting them into water.

    - Record the same in BPCR.

    - Clean the cubicle as per SOP NO. XXX

  - CLEANING PROCEDURE

    - At the end of the shift/batch changeover of same product.

    - Put OFF the mains.

    - Remove all the dust using vacuum cleaner.

    - Remove powder hopper by loosening the knob.

    - Remove the dosing disk.

    - Remove the brush of DP 100 and clean with vacuum cleaner.

    - Remove powder level adjuster by loosening the screw.

    - Clean above parts with dry lint-free duster.

    - After five ‘Type A’’ cleaning ‘Type B’’ cleaning shall be performed.

  - Cleaning during changeover of product.

    - Put OFF the mains and put 'TO BE CLEANED' Label on the machine.

    - Remove all dust using a vacuum cleaner.

    - Remove hopper by loosening the knob.

    - Remove doing assembly.

    - Dismantle the magazine and finger projection.

    - Dismantle the pins.

    - Dismantle the pallets filling assembly.

    - Remove the brush of DP 100.

    - Dismantle the cap bush and body bush.

    - Take the above parts to washing area with the help of trolley having 'TO BE CLEANED' Label.

    - Clean all the parts using nylon scrubber with potable water.

    - Finally rinse the parts with purified water.

    - Dry the parts with lint-free duster.

    - Dry the DP brush under Infra red Drier.

    - Wipe the surface of machine with dry lint-free cloth.

    - Assemble all the above parts on the AF 150 machine.

    - Remove the 'TO BE CLEANED' Label and put 'CLEANED' Label duly signed by Production Officer/Executive.

    - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

    - After preventive maintenance, all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

  - PRECAUTIONS:

    - Ensure that the compressed air pressure shall not be less then the limit.

    - Do not run the machine without closing the safety door.

    - Ensure that the vacuum cut off valve shall be working properly.

  - ENCLOSERS:

Annexure –I Cleaning check list

Change History Log

SOP Training Record

- ABBREVIATIONS:

SOP		:	Standard Operating Procedure

No.		:	Number

IPQA		:	In process quality Assurance

BPCR		:	Batch Production and control Record

IPC		:	In process container

S.S		:	Stainless Steel

DP Machine	:	Dedusting and Polishing machine

PRO		:	Production Officer

PRE		:	Production Executive

QAO		:	Quality Assurance Officer

QAE		:	Quality Assurance Executive

CAPSULE FILLING MACHINE (AF-150)

| Equipment No |  | Date |  |
| --- | --- | --- | --- |
| Previous Product |  | Next Product |  |
| Batch no. |  | Batch No. |  |

| Sr. No. | CLEANING STEPS | STATUS |
| --- | --- | --- |
| TYPE –A CLEANING CHECKLIST |  |  |
|  | All loosely stuck powder removed from the machine with vacuum cleaner and duster. |  |
|  | Hopper, machine guard window, empty capsule sorter and polishing unit cleaned with duster. |  |
|  | Collection box, dosing disk, dosing pins, powder level sensing device cleaned with nylon brush and duster. |  |
|  | Empty capsule hopper, powder hopper, base plate, capsule rejection chamber cleaned with nylon brush and vacuum. |  |
|  | External and internal surface of machine cleaned with duster. |  |
|  | Turret, Magazine, finger projection Exit chute, platform, acrylic door and non-contact parts cleaned with duster and vacuum. |  |
|  | Dust extraction tube and nozzles and dust collection pipes cleaned with duster. |  |
|  | Empty capsule hopper, empty capsule loader and base of the machine and other non contact parts cleaned with duster |  |
|  | Room cleaned as per SOP No XXX |  |

| TYPE –B CLEANING CHECKLIST |  |  |
| --- | --- | --- |
|  | All loosely stuck powder removed from the machine with vacuum. |  |
|  | Machine interior, ceiling and outer surface cleaned with duster dipped in purified water and then with dry duster. |  |
|  | Empty capsule hopper and base of the machine and other non-contact parts cleaned with duster dipped in purified water and then with dry duster. |  |
|  | Collection box, feed channel, powder hopper, Dosing disk. Tapping blocks, tamping pin clean with purified water and then dry duster. |  |
|  | Capsule ejection box, locking plate, locking pin, bush, cleaning assembly and filled capsule exit chute cleaned with nylon brush, duster and purified water. |  |
|  | Guide plate, cap bush, body bush, EHG capsule loading assembly cleaned with nylon brush, duster and purified water. |  |
|  | External and internal surface of machine cleaned with nylon brush, duster and purified water. |  |
|  | Connecter joint, empty capsule loader and machine guard cleaned with purified water. |  |
|  | Room cleaned as per SOP No XXX |  |

NOTE: Tick mark ( ) - Completion of jobs as per instruction,    Cross mark (x) –Incomplete or incorrect job done

Strike out the checklist which is not applicable

|  | Cleaned By Operator | Checked By PRO/PRE | Checked By QAO/QAE |
| --- | --- | --- | --- |
| NAME |  |  |  |
| SIGN |  |  |  |
| DATE |  |  |  |