- Objective

To lay down a procedure for Operation of ERH meter.

- Scope

The SOP is applicable for check the amount of free water vapour present in a hygroscopic product employed in manufacturing Area of Alkem Laboratories Limited Unit-II Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation of machine is done as per SOP.

- Accountability

Head Production

- Procedure

  - Ensure that, machine and room is cleaned before machine set up.

  - Connect the AC adaptor to the power connector of Hygrolab and connect a prob1.

  - Press MENU key to activate the functions, the different Menu items appears on the LCD message line.

  - Menu Selection:

MENU

CALCULATE

SYS-STATUS

SETTINGS

PROBE

ADJUST REF

ADJUST 1 PT

ADJUST MPT

DISPLAY

MODE

ENTER

STANDARD

EAwE

QAW QUICK

  - Select mode selection of Hygrolab 3 by navigating UP/DOWN keys and select standard mode followed by ENTER.

  - Then select Q Aw Quick, now the equipment set up is ready. Place the tablet / blend (adequate) into the sample holder above that keep the AW-DIO water activity station and ENTER. Now the test starting.......

  - Initially LCD display shows Q1 00:01 :00D then the reading will be continued to maximum time limit of upto 5.0 min when the analyser over there must be ‘beep sound followed by machine stops automatically with the final display. Example as follows 3.52% rh and 69.64 degree F with the reading Q1 00:04:55S.

  - Note down the reading in the batch manufacturing record then press switch off and disconnect the AC adaptor to the power connector of Hygrolab.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary

SOP		:	Standard Operating Procedure

AW		:	water activity

ERH		:	equilibrium Relative Humidity

Q AW-DIO	:	water activity measurement station

Q Aw Quick	:	water activity measurement with Aw quick algorithm for accelerated

AC		:	Alternate current

LC		:	Liquid crystal

LCD		:	Liquid Crystal Display

1pt		:	1 Point Adjustment

%		:	Percentage

SYS-STATUS	:	System Status

BMR		:	Batch Manufacturing Record

IPC		:	In process container

S.S.		:	Stainless Steel

- Revision History:

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |