- Revision History

| Rev. No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. change in SOP format 2 2.change in font type from time roman to Tahoma & font size from 12 to 10 | 1. Periodic Review 2. to make the document user friendly. |
| 03 | NA | Periodic Review |

- Objective

To lay down a procedure for operation of on line camera on sticker labeling machine

3.0    Scope

This SOP is applicable for for operation of on line camera on sticker labeling machine in C Block.

4.0     Responsibility

Officer and Above Production (production/IPQA)	:	Implementation of Procedure.

Operation and cleaning                                            :      technician

5.0    Accountability

Production Head

6.0   Procedure

6.1 operation

6.2   switch on the power supply of computer.

6.3   enter relevant password. The desktop menu will be displayed.

6.3   Enter relevant password. The desktop menu will be displayed.

6.4 Click ON presence PLUS, to start camera.

6.5 For set up parameter click on SET UP, then FOCUS and TRIGGER option will be displayed.

6.6 Click on FOCUS option, AUTO EXPOSURE then EXPOSURE TIME will be displayed.

6.7 Enter the required value in displayed parameter.

6.8   Click on TRIGGER option, minimum trigger width and polarity will be displayed.

6.9	Enter the required value in displayed parameter.

6.10	For tool parameter setting, click on tool parameter, then BUILD, SENSOR and LIBRARY will be displayed.

6.11 Click on BUILD and enter in GC count, GC tool will be displayed. Draw required ROI and click on teach and apply respectively.

6.12 Click on advance, Rotary Range, Extra Edges, Missing Edges Acceptance Label, result threshold

and Scaling Range will be displayed respectively.

6.13 Enter the required value in displayed parameter.

6.14 Then return to INPUT and click on next then tool will be displayed.

6.15 Enter file name and match count to show the number of good and rejected lables by paas/fail

Counter.

6.16 Create and name a file and save all settings.

6.17 Then after select the camera in RUN mode.

6.18 Click on reset and followed by start button and start labeling machine.

6.19 In case of over printing defect, the camera will automatically stopped and hooter will sound with

red glowing light.

Note: (A) Adjust proper light on label for proper camera functioning.

(B) Appropriate area to be drowns for ROI.

(C) The setting data should be saved in defined file along with name.

7.0   Annexure (S)

NA

8.0 Reference (S)

NA

9.0 Glossary

SOP   	: 	Standard Operating Procedure.

NO.      :	 Number

PC        :	 Production (C-Block)

NA         :         Not applicable

IPQA      :       In Process Quality Assurance

ROI        :     Region of Interest