- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for preparation and filtration of disinfectants solution.

# 3.0    Scopes

This SOP is applicable for preparation and filtration of disinfectants solution used in C Block.

# 4.0	Responsibility

Technician Preparation		:	Preparation & Filtration.

Officer and Above Production	:	Implementation of Procedure.

5.0	    Accountability

Production Head.

  - Procedure

  - Preparation Of Disinfectant Solution

6.1.1 Ensure the availability of cleaned pressure vessel, disinfectant as per schedule, measuring

cylinder, WFI.

6.1.2  Take less than measured quantity of WFI in pressure vessel then add required quantity of                 disinfectant by using measuring cylinder in to pressure vessel containing WFI.

6.1.3 Rinse the measuring cylinder with WFI, add it in to pressure vessel.

6.1.4 Add WFI in to pressure vessel to quantity sufficient.

6.1.5 Close the lid of pressure vessel properly and go for filtration

6.2     Filtration Of Disinfectant Solution

6.2.1 Ensure that availability of pre sterilized 0.22 membrane filter with filtration assembly, pressure

vessels , silicon tubes and SS clamps under LAF.

6.2.2 Ensure that the availability of freshly prepared disinfectant solution in pressure vessel.

6.2.3 Connect the silicon tube to up stream of filtration assembly and then connect the other end of   tube to out let of pressure vessel containing disinfectant solution.

6.2.4 Connect the silicon tube to down stream of the membrane filtration assembly and connect the   other end to the filtrate collection pre sterilized pressure vessel.

6.2.5 Connect compressed air tube to inlet of the pressure vessel containing disinfectant solution to be filtered.

6.2.6 Tie the clamp at the joints to avoid spoilage during processing.

6.2.7 Ensure that all the connection clamps are firmly tight.

6.2.8 Open the valve of compressed air line and increase the pressure from pressure regulator up to      1.5 kg/cm2 and build up the air pressure in pressure vessel.

6.2.9 Open the air vent valve of membrane filtration assembly to remove the air bubbles so that the rate of filtration remains satisfactory.

6.2.10 Carry out the operation till all the disinfectant solution gets filtered.

6.2.11 Check the filter integrity as per SOP No. PC/095 before and after filtration.

6.2.12 Filtration activity should be performed under LAF.

7.0       Annexure (S)

NA

8.0       Reference (S)

SOP NO : PC/095

9.0      Abbreviation (S)

SOP	   : Standard Operating Procedure.

LAF	: Laminar air flow.

NO       : Number

SS	: Stainless Steel.

PC	: Production (C-block)