# Design QUALIFICATION

# NAME OF THE ITEM: BQS (Blister Quick Servo)

# FUNCTIONAL AREA:  C- BLOCK

# PROTOCOL No. :        DQ/

ALKEM HEALTH SCIENCE

SAMARDUNG, KAREK BLOCK,

PO. NAMTHANG,

SOUTH SIKKIM-737137

CONTENT

Sr. No.	  Description

- Protocol Approval

- Objective

- Responsibility

- Equipment Description & Identification

- User Requirements

- Complementary Aspects

- Safety and Environmental Protection

- Cleaning maintenance and service

- Rules and Regulation

- Scope of Delivery

- Installation ,Commissioning and Tests

- Qualification /Validation

- Guarantee/Warrantee

- Deviation

- Annexure

- Summary and Conclusion

- Approval of Design Qualification

- Acceptance by vendor

- PROTOCOL APPROVAL:

Protocol Prepared By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |

DQ Reviewed By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |
| Production |  |  |  |
| Quality Assurance |  |  |  |

DQ Approved By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

- Objective:

The purpose of this document is to ensure that all the critical aspects of the Equipment,

cGMP & Safety features have been considered in designing the equipment/instrument and

is properly Documented

- Responsibilities:

In accordance with the document, following functions shall be responsible for initiation and finalization of Equipment user requirement specification. When the work is carried by contract/ consulting staff, all the work is to be performed under the oversight of Alkem.

3.1 Preparation of Document

- User department to prepare the DQ

- Ensures that the document is in compliance with current policies and procedures of cGMP regulations.

- Ensures that the content is sufficient, clearly defined, technically sound and accurate.

- It is a Guidance document to prepare the DQ.

# 3.2 Review of Document

- To be reviewed by Head of the user department and functional department ( Engineering & Quality assurance)

# 3.3 Approval of Document

- Approval of document by Head Manufacturing/Head Engineering/Head Quality.

# 4.0	Equipment Description & Identification:

# 4.1       Scope:       BQS (Blister Quick Servo).

# 4.2      Purpose:   Purpose of BQS (Blister Quick Servo) in production area to pack the tablets/ capsules in the pvc/pvdc- Alu and Alu- Alu blisters.

  - Process Equipment Description: This equipment used in production packing area for the packing of tablets/capsules in the different pack styles in the form of Pvc/Pvdc-Alu and Alu-Alu blisters. It is economically used in the packing area for production.

  - USER REQUIREMENTS

  - System Requirements:

| Sr. No. | SYSTEM COMPONENTS | SYSTEM REQUIREMENTS |
| --- | --- | --- |
| 1 | Identification (In case of Equipment /Instrument) | BQS (Blister Quick Servo) |
| 2 | Model/Type | BQS (Blister Quick Servo) |
| 3 | Capacity | Max. 45 cycle/min. for pvc blisters and 35 cycle/min. for alu-alu blisters. Max. 200 punching cycle/min. in punching station. |
| 4 | Potential Suppliers | Pam-Pack machines Pvt. Ltd. |
| 5 | Contact parts (In case of Equipment) | SS 316 L with mirror finish |
| 6 | Non contact parts (In case of Equipment) | SS 304 with matt finish |
| 7 | Non metallic contact parts (In case of Equipment /Instrument) | - Any material with food grade quality having no Potential impact on the products. 2. Durable. 3. Must be easily cleanable. |
| 8 | Motor & Electrical installations (In case of Equipment /Instrument) | Machine should be operated through PLC mounted on separate electrical control panel. |
| 9 | Machine assemblies (In case of Equipment /Instrument) | Must be covered with SS 304 with mirror finish. |
| 10 | Machine adjustments (In case of Equipment /Instrument) | Setting with Zero clearance with good accuracy. |
| 11 | Packaging & Transport | Should be packed and transported in such a way to avoid any damage during transportation. |
| 12 | No. of requirements | 01 |
| 13 | Requirements for any power failure backup’s (In case of Equipment /Instrument) | To be backed up by installed in-house DG set. After any power failure PLC show continuous counting of all filled blisters. |
| 14 | Gear box specifications(In case of Equipment /Instrument) | As per cGMP model |
| 15. | Machine specification | - Basic machine with dedicated feeding system consisting of Hopper, Linear Vibrator, and Rotary Vibrator (Dedicated feeding system) - Providing Servo System (Make M/s. B & R Automation, Austria) 5 servo system. - Acrylic guarding. - Overprinting unit. - Alu-Alu attachment - Drum type feeding system. - Embossing station and Perforation Station. - Film presence sensor for lidding foil. - Wiper Brush Assembly. - Print registration system - Provision of Link-up with Cartoning Machine. - Provision for camera and NFD. |
|  | Material of Construction | - All contact parts made of SS316L, Nylon & all other non-contact components are of SS304 or as per the machine requirements - Exterior easy to clean & non-corrosive. |
|  | Safety features | Adequate safety features for men and material are provided along with the equipment. |
|  | Qualifications/ Documentations | The manufacturer shall complete and provide the documents pertaining to Design, Installation, and Operation Qualification. |
|  | Operating System | The equipment shall be operated through PLC system. |
|  | Control System | - The electrical system of the equipment shall be housed as per cGMP and cGEP standards, with adequate safety. - Electrical panel and electro pneumatic panel to be installed on the base structure inside the machine. - Control panel to be housed inside the machine. |

Verified By & Date:

  - Technical Description

    - Specification of components.

| Sr. No. | Components | Descriptions |
| --- | --- | --- |
| 1 | Name | BQS (Blister Quick Servo) |
| 3 | Specification | Contact parts - SS316 L and Non contact parts – SS 304 |
| 5 | Output | Max. 45 cycle/min. for pvc blisters and 35 cycle/min. for alu-alu blisters. Max. 200 punching cycle/min. in punching station. |
| 6 | Forming Area | Max. forming area 240 X 180 mm |
| 7 | Forming Depth | Max. up to12 mm |
| 8 | Lidding foil un-winding station | Max. roll dia. 250 mm with 72-76 core dia. |
| 9 | PVC unwinding geared motor | Make : Technocraft HP- 0.05 HP RPM – 1440 RPM Voltage- 230 V AC Phase – 1 Ph Quantity- 1 No. |
| 10 | PU Roller | Material- PU coated with Alu. Base Quantity-1 No. |
| 11 | Base foil joint detection sensor | Make : Visolux / P&F Model- PK-20 Voltage-10 to 30 VDC Quantity- 1 No. |
| 12 | Sensor for dancing Lever | Type- Inductive Specification- M12 Quantity-2 No. |
| 13 | Heating plate ( Bottom ) | Material- Al. Casting (LM 9) Treatment- Teflon coating Watt- 2.5 kW Quantity- 1 no. |
| 14 | Heating plate top | Material- Al. Casting (LM 9) Watt-2.5 kW Quantity- 1 no |
| 15 | Pneumatic cylinder (Tandom) Bottom | Make : Pneumax Size- Bore Dia.32 (25 mm. stroke + 5 mm. stroke) Quantity- 1 No. |
| 16 | Pneumatic cylinder (Top) | Make: Pneumax Size- 32 x 25 Quantity- 1 No. |
| 17 | Top Plate | Material- H 30 (T6) Treatment- GBB and Anodising Quantity-1 no. |
| 18 | Bottom plate | Make : Material- Alu. Alloy Treatment- GBB and Anodising Quantity- 1 No. |
| 19 | Power pack(Common for forming and sealing station) | Make: Zeon Make Working pressure-100 Bar Motor make- Siemens/ Crompton Greaves/ Bharat Bijlee. Motor HP-5 HP Phase- 3 Ph Voltage- 415 V AC RPM-1440 RPM Quantity-1 No. |
| 20 | Hydraulic valve | Make: CETOP/ Parker/ Hyto Voltage-24 V DC Quantity- 1 No. |
| 21 | Hydraulic cylinder | Make : IPH Size- Dia. 80 Bore x 15 stroke Quantity- 1 No. |
| 22 | Hopper | Material- SS 316 L Finish- Polished inside surface Capacity-50 Liters Quantity- 1 No. |
| 23 | Linear vibrator | Material- SS 316 L Finish- Polished inside surface Quantity-1 No. |
| 24 | Vibrator controller ( Linear ) | Make : Sai Control Voltage-230 V, 3 A Quantity- 1 No. |
| 25 | Vibrator controller (Rotary) | Make : Sai Control voltage-230 V, 6 A Quantity-1 no. |
| 26 | Vibrator coil (Linear) | Make : Sai Control Voltage-230 V Quantity-1 No. |
| 27 | Vibrator coil ( Rotary ) | Make : Sai Control Voltage-230 V Watt-180 W Quantity-3 Nos. |
| 28 | Distributor plate | Material- SS 316 L Finish- Matt Quantity- 1 No. |
| 29 | Vibrator strips | Material-58 CRV 4/ EN47 Hardness-40 - 45 HRC Treatment- Blackodising Quantity- 4 Nos. |
| 30 | Geared Motor (For Alu. Unwinding station) | Make : Elscient Watt-0.18 W RPM-1370 RPM Voltage-415 V AC Phase-3 Ph. Quantity-1 No. |
| 31 | Gear for unwinding (For Alu. Unwinding station) | Material- Hylum Quantity- 1 No. |
| 32 | Sealing Station (Top Plate) | Material- H 30 (T6) Treatment- GBB and anodising Quantity- 1 No |
| 33 | Sealing Station (Bottom plate) | Material- H 30 (T6) Treatment- GBB and anodising Quantity- 1 No |
| 34 | Sealing Station (Hydraulic valve) | Voltage-24 V Quantity-1 No |
| 35 | Sealing Station (Heater plate) | Material- LM-9 Hardness-120 BHN Treatment- Glass bead blasting Quantity- 1 No |
| 36 | Sealing station (Hydraulic cylinder) | Size- Dia. 80 Bore x 15 stroke Quantity- 1 No. |
| 37 | Cooling station(Bracket) | Material- LM 16 Hardness-90-100 BHN Treatment- Painting Quantity-1 No. |
| 38 | Cooling station(Top Plate) | Material- H9 T6 Treatment- GBB and anodising Quantity-1 No |
| 39 | Cooling station(Bottom plate) | Material- H9 T6 Treatment- GBB and anodising Quantity- 1 No. |
| 40 | Overprinting station (Pressure roller) | Material- M. S. Treatment- BNP Quantity- 1 No. |
| 41 | Web Pulling station(Pulling disc) | Material- HC HCR/ EN41B Hardness-45 - 50 HRC Treatment- Blackodizing Quantity-2 Nos |
| 42 | Web Pulling station(Pulling disc intermediate) | Material- HC HCR Hardness-45 - 50 HRC/ EN41B Treatment- Blackodizing Quantity- 1 No. |
| 43 | Web Pulling station(Gear Box) | Make : Bonfiglioli Ratio-1:30 Quantity- 1 No. |
| 53 | Web Pulling station(Servo motor) | Make : B&R Automation Torque-5.3 Nm Quantity- 1 No. |
| 54 | Punching and perforation station(Gear Box) | Make : Technocraft Ratio-7.5:1 Quantity- 1 No. |
| 55 | Punching and perforation station(Servo motor) | Make: B&R Automation Torque-7.5 Nm Quantity- 1 No. |
| 56 | Punching and perforation station-Cutting plate (change part) | Material- HCHCr Hardness-58-60 HRC Treatment- Blackodising Quantity- 1 No. |
| 57 | Punching and perforation station-Punch (change part) | Material- HCHCr Hardness-58-60 HRC Treatment- Blackodising Quantity- 1 No.(Depends) |
| 58 | Punching and perforation station- Change gear (change part) | Material- EN8 / SAE 8620 Hardness-55-60 HRC Treatment- Blackodising Quantity- 1 No. |
| 59 | Punching and perforation station- Moving plate (change part) | Material- EN-8 Treatment- Blackodising Quantity-1 No |
| 60 | Punching and perforation station-Sliding plate (change part) | Material- M.S. Treatment- Blackodising Quantity- 1 No. |
| 61 | Punching and perforation station- Support plate (change part) | Material- H9 T6 Treatment- Anodised Quantity-2 Nos |
| 62 | Pick up station(Vacuum pump) | Make : Indovac Capacity-500 LPM Pressure-26" of Hg. Quantity- 1 No. |
| 63 | Pick up station(Vacuum pump motor) | Make : Siemens/ Bharat Bijli/ Hindustan Motor Supply voltage-415 V AC HP- 2 HP RPM- 1440 RPM Phase- 3 Ph Quantity-1 No. |
| 64 | Pick up station(Suction cup) | Suction cup : IWKA Material- Polyurethane Quantity-4 No |
| 65 | Pick up station(Good bad sorting chute) | Material- SS 304 Quantity- 1 No. |
| 66 | Pick up station(Suction wheel) | Material- Nylon Quantity- 2 Nos. |
| 67 | Structure and drive(Main frame) | Material- M.S. Fabrication Painted |
| 68 | Structure and drive(Cladding) | Material-SS 304 Finish- Mat |
| 69 | Structure and drive(Machine mounts) | Make : AVMEC Type- AV 20 Quantity-4 Nos |
| 70 | Controls(Power supply) | Make : NHP/ Meanwell Operating Voltage-24 V DC– 20A, Quantity-1 Nos |
| 71 | Controls(Transformer) | Make : Chaitanya Voltage-415 V AC +/- 10% Phase-1 Ph Output-220 to 240 Volts Quantity-1 No. |
| 72 | Controls(Print Mark check Scanner) | Make : Visolux / P&F Model- PK-20 Quantity- 1 No. |

Verified By & Date:

  - Utility Requirment:

| Sr no | Specification | Requirment |
| --- | --- | --- |
| 1 | Electrical supply | 3 Phase ,415 V AC ,50 HZ |
| 2 | BQS Compressed Air #### ( Dry ) | Pressure – 6 bar / cm2 Capacity - 30 to 35 CFM |
| 3 | BQS (Electrical supply) | 415 V, 50 Hz. Average 7 kW Peak 14 kW |
| 4 | BQS (Chilled water) | Cooling Capacity –9000 Kcal/Hr. Temp. – PVC -100 C PVDC -150 to 200 C Flow rate – 15-20 lit/ min. |

Verified By & Date:

  - Material of Construction

  - 5.5.1 Contact and Non contact surfaces.

| # Sr. No |  | # Component Description | # MOC |
| --- | --- | --- | --- |
| 1 | Main frame | M. S. Fabricated |
| 2 | All structural plates | M. S. |
| 3 | Cladding covers | SS 304 |
| 4 | Product contact SS parts | SS 316 |
| 5 | Distributor plate | SS 316 |
| 6 | Guarding over moving parts | Transparent acrylic |
| 7 | Guarding sections | Alu. |
| 8 | Punch tool | H C H Cr. Tool Steel |
| 9 | Cutting plate | T6 160 Cr12 HCHCr T6 Steel |
| 10 | Vibrating strip | 58 CRV 4 |
| 11 | Forming and sealing top plate | H 30 (T6) / EN47 |
| 12 | Forming and sealing bottom plate | H 30 (T6) |
| 13 | Heater plate | LM-9 |
| 14 | Cooling top and bottom plate | H9 T6 |
| 15 | Change gear | EN8 / SAE 8620 |
| 16 | Good/ Bad sorting chute | SS 304 |

# Verified By & Date:.

6.0      COMPLEMENTARY ASPECTS

6.1       Training

| Sr. No. | Specification |  | SYSTEM REQUIREMENTS |
| --- | --- | --- | --- |
| 1 | The vendor Shall supply all available information for the adequate exploitation of equipment. For the Compliance of this purpose at the Job site and/ or at the Vendors Shop. Vendor’s technical staff shall train customer’s personnel. The scope of the Training will be agreed during the contract signature. |  | YES |
| 2 | The supplier is to include the personnel training activities. The supplier is to specify the foreseen time for: - Operator/Supervisor training - Manager Training - Electrical maintenance training - Mechanical Maintenance training |  | YES |
| 3 | The supplier is to specify the personnel background needed for each of the operators maintenance. |  | YES |
| 6.2 Pre Delivery Qualifications (FAT) |  |  |  |
| Sr. No. |  | Specification | SYSTEM REQUIREMENTS |
| 1 |  | The System or its parts as provided for in the scope of supply shall be pre-installed at the vendors shop prior to delivery to customer site. Installation will be completed and documented including mechanical parts as well as electrical connections of all parts to facilitate taking over tests at Vendors shop prior to delivery. | YES |

6.3 Supplier Technical Documentation Requirements:

| Sr. No. | COMPONENTS | REQUIREMENTS |
| --- | --- | --- |
| 1. | Technical Documents | FAT,IQ,OQ Electrical Drawing P & ID diagram GA diagram Operation manual Calibration certificates Bought out components certificates. MOC certificates |

6.4    Technical Manuals

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | Operation manual -02 copies | YES |

Verified By & Date

7.0 SAFETY AND ENVIRONMENTAL PROTECTION

| Sr No. |  | Specification |  |  | Requirements |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Environment |  |  | NA |
|  | 7.1 Safety and Interlocking |  |  |  |  |
| Sr. no | Alarm Messages. |  | Fault | Description |  |
| 1 | Sensor for forming foil |  | Foil end | Machine stops at zero position |  |
| 2 | Air pressure |  | Low air pressure | Machine stops at zero position |  |
| 3 | Water pressure |  | Low water pressure | Machine stops at zero position |  |
| 4 | Machine guard |  | Guard open | Machine stops at zero position |  |
| 5 | Vacuum pump |  | Vacuum pump off | Machine stops at zero position |  |
| 6 | Low temperature- forming |  | Temperature is low | Machine stops at zero position |  |
| 7 | Low temp.- sealing |  | Temperature is low | Machine stops at zero position |  |
| 8 | High temperature- forming |  | Temperature is high | Machine stops at zero position |  |
| 9 | High temperature- sealing |  | Temperature is high | Machine stops at zero position |  |
| 10 | Eye mark |  | Eye mark missing | Machine stops at zero position |  |
| 11 | Sealing thermocouple |  | Thermocouple open | Machine stops at zero position |  |
| 12 | Sensor for lidding foil |  | Foil end | Machine stops at zero position |  |
| 13 | Emergency off |  | Emergency off | Machine stops instantly |  |
| 14 | Water pressure |  | No heating, No water | Machine stops at zero position |  |
| 15 | Overload |  | Machine overload | Machine stops at zero position |  |
| 16 | MCB |  | MCB trip | Machine stops at zero position |  |
| 17 | Air blowing – sealing |  | ----- | Air blows while machine is in stopped condition |  |
| 18 | Air blowing – Punching |  | ----- | Air blows while machine is in running condition |  |
| 19 | Indexing |  | Indexing overload | Machine stops at zero position |  |

Verified By & Date:

8.0 CLEANING MAINTENANCE AND SERVICE

| Sr No. | Specification |
| --- | --- |
| 1. | In accordance with cGMP guidelines the units must be easy to clean. |
| 2. | The Supplier should guarantee that, if required, a service team can be on site within one working day. |
| 3 | The design should be such as to allow mechanical cleaning of the surface and that the cleanliness of the surface can be checked easily. |
| 4 | All machine parts, in particular instrumentation, should be constructed so that they can be easily removed and calibrated. |
| 5 | All special tools required for running and maintenance should be best. |
| 6 | A spare parts delivery guarantee with in time. |

Verified By & Date:

9.0 RULES AND REGULATION :

These standards, recommendation and requirements are considered  the minimum. Specifications that are more stringent or expansive take the precedence. In case of conflict between published requirements, final determination is the responsibility of the Owners Representative.

10. SCOPE OF DELIVERY

| Sr No. | Specification | Requirements |
| --- | --- | --- |
| 1 | Units described in the specific system requirements including all necessary controls and instrumentation. | YES |
| 2 | The complete mechanical and electrical installation. | YES |
| 3 | The Connections to all the necessary utilities, exhaust, and waste lines necessary for its operation. | YES |
| 4 | All piping and cabling of the units itself. | YES |
| 5 | Wiring and cable run: all wiring and cable run is part of the supply. Alkem Health Science will supply the main power switches to be located in correspondence to the electrical and control cabinets delivered by the equipment supplier. | YES |
| 6 | All internal contacts of the supplied equipment for the required utilities. | YES |
| 7 | Unload on site of the equipment: the supplier is required to define all the necessary handling devices required to the unloading operation. The supplier will inform at least 4 weeks in advance the day of delivery and the list of required handling devices. | YES |
| 8 | Assembling operation: the required consumable, the internal transportation, the assembling tools and the required personal are part of the supply. | YES |
| 9 | A complete set of commissioning spare parts. | YES |
| 10 | All special tools necessary for use and maintenance of the supplied equipment. | YES |
| 11 | A complete set of two years spare parts should be listed quoted and offered as option. | YES |
| 12 | All test activities as specified in this document. | YES |
| 13 | Training in the use and maintenance of the equipment. | YES |
| 14 | A complete set of documentation as specified In this document. | YES |

Verified By & Date:

11.0	 INSTALLATION, COMMISSIONING AND TESTS

11.1    General

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The Supplier must specify for each piece of equipment the Guaranteed performance and the guaranteed system performance. These values will be tested during the acceptance tests. | YES |
| 2 | In addition the functionality described in the user requirements and detailed in the system specifications will be tested. | YES |

11.2 INSTALLATION , COMMISSION

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1 | The commissioning tests will be carried out in accordance with a written test plan developed by the supplier with clearly stated test procedures and acceptance criteria. | YES |
| 2 | The supplier will approve successfully completed tests and will specify items requiring additional work. Representatives from Alkem Health Science. Will attend and participate in the commissioning tests as required. | YES |
| 3 | The installation and commissioning of the system will be performed at the Alkem Health Science. Facility by the supplier. | YES |
| 4 | The commissioning can only start once all the foreseen documents have been delivered by the supplier to Alkem Health Science. | YES |
| 5 | All equipment should be properly installed, adjusted, leveled, tagged, and connected with utilities. | YES |
| 6 | Point to point checks on wiring and pneumatic should be performed. | YES |
| 7 | All instruments should be properly calibrated. | YES |
| 8 | A equipment ( instrument) used for qualification must be listed and approved by Alkem Health Science. | YES |
| 9 | The calibration equipment must have all the necessary documents to demonstrate their maintenance & use. | YES |
| 10 | The last calibration of all this equipment must be less than 6 months old, and evidenced by certificate. | YES |
| 11 | Verification that the interior surfaces of equipment are free of practices and dirt and all points of product contact meet the specified material requirements. | YES |
| 12 | All the clearances and tolerances specified in the drawing or recommended by component manufacturers are correct. | YES |
| 13 | On site verification that valves and other equipment with moving parts are in their normal position if in a power down condition and move in the correct direction with the correct speed and precision. | YES |
| 14 | Verification that all the Input and Output points are connected and labeled according to the documentation and that all the along the input values have been scaled in accordance with the system specification and process requirements. That all equipment components requiring configuration | YES |
| 15 | The commissioning should demonstrate that the system supplied by the supplier has been properly installed and that the functions are in accordance with Alkem Health Science. User Requirements specifications, Vendors System specifications Manuals and other Documentation. | YES |

11.3 Site Acceptance Test   (SAT)

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | This test will be carried out once the commissioning will be completed. The scope will be to verify the performance and the functionality of the system integrated with the other factory systems | YES |
| 2 | The test will be carried out to verify the system response with the expected productivity of the system. | YES |
| 3 | Details on the test realization will be defined during the project Phase. The supplier is asked to specify the proposed duration for SAT and the standard procedure proposed. | YES |
| 4 | During SAT the required functionality, performances and system reliability are met. | YES |
| 5 | The Functionality described in the User Requirements Specification and in the System Specifications are verified and met. | YES |
| 6 | All the documentation agreed has been delivered. | YES |

Verified By & Date:

12.0 QUALIFICATION / VALIDATION

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The maintenance Qualification is responsibility of the customer. However, the supplier is responsible for delivering the basic documents for maintenance qualification. | YES |
| 2. | This includes all side costs such as : calibration measuring equipment and instruments: manpower (IQ and OQ will take place completely on Alkem Health Science.) | YES |
| 3 | Time Schedule for IQ/OQ execution will be developed by Alkem Health Science. With the supplier. | YES |
| 4 | Suppliers personnel used for IQ/OQ must be well trained and experienced. This should be documented. | YES |
| 5 | The onsite test run performed by the supplier might become part of the IQ. | YES |
| 6 | Main IQ/OQ steps such as calibration must be performed and documented in accordance to a SOP approved by Alkem Health Science. | YES |
| 7 | All equipment used for qualification must be listed and approved by Alkem Health Science. The calibration equipment should be well documented. | YES |
| 8 | The last Recalibration of all this equipment should be less than 06 month old. Proofed by Certificate. | YES |
| 9 | OQ can only start after IQ approved by Alkem Health Science. | YES |
| 10 | IQ will be carried out by Alkem Health Science. During Installation phase. IQ will include the tests performed by the supplier. | YES |
| 11 | Part of the OQ will be carried out by Alkem Health Science. During commissioning and SAT phase. OQ will include the tests performed by the supplier. | YES |
| 12 | After installation of the equipment at customers site. Complementary IQ & OQ tests will be performed by the Customer and may be supervised by a member of Technical staff. | YES |
| 13 | Qualification documents (In case of equipments/Instruments) | DQ, IQ, OQ |

Verified By & Date:

13.0 GAURANTEE/WARRANTEE

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The System must be guaranteed including all the sub- system and components for a period of 12 months from the date of the system acceptance for a 03- shift operation. | YES |
| 2 | The servicing companies involved for the Sub- systems maintenance must be declared and the maintenance group organization described. Furthermore, the supplier will be directly responsible of the system assistance and the required operation will be co- ordinate by him. | YES |
| 3 | In case of failures, the intervention will be guaranteed by the supplier within a maximum time limit. The supplier is asked to specify the maximum time limit. | YES |
| 4 | The supplier is asked to propose as option maintenance and assistance contract after the guarantee expiration. | YES |

Verified By & Date:

# 14.0 Deviation

# 15.0 Annexure

# 16.0 Summary and Conclusion

# 17.0 Approval of Design Qualification.

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

# 18.    Acceptance By vendor

# Name of Vendor:

# Sign/Date: