# Design QUALIFICATION

# NAME OF THE ITEM: COMPRESSION MACHINE -35 ST

# FUNCTIONAL AREA:C- BLOCK

# PROTOCOL No. :        DQ/

ALKEM HEALTH SCIENCE

SAMARDUNG, KAREK BLOCK,

PO. NAMTHANG,

SOUTH SIKKIM-737137

CONTENT

Sr. No.	  Description

- Protocol Approval

- Objective

- Responsibility

- Equipment Description & Identification

- User Requirements

- Complementary Aspects

- Safety and Environmental Protection

- Cleaning maintenance and service

- Rules and Regulation

- Scope of Delivery

- Installation ,Commissioning and Tests

- Qualification /Validation

- Guarantee/Warrantee

- Deviation

- Annexure

- Summary and Conclusion

- Approval of Design Qualification

- Acceptance by vendor

- PROTOCOL APPROVAL:

Protocol Prepared By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |

DQ Reviewed By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |
| Production |  |  |  |
| Quality Assurance |  |  |  |

DQ Approved By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

- Objective:

The purpose of this document is to ensure that all the critical aspects of the Equipment,

cGMP & Safety features have been considered in designing the equipment/instrument and

is properly Documented

- Responsibilities:

In accordance with the document, following functions shall be responsible for initiation and finalization of Equipment user requirement specification. When the work is carried by contract/ consulting staff, all the work is to be performed under the oversight of Alkem.

3.1 Preparation of Document

- User department to prepare the DQ

- Ensures that the document is in compliance with current policies and procedures of cGMP regulations.

- Ensures that the content is sufficient, clearly defined, technically sound and accurate.

- It is a Guidance document to prepare the DQ.

# 3.2 Review of Document

- To be reviewed by Head of the user department and functional department ( Engineering & Quality assurance)

# 3.3 Approval of Document

- Approval of document by Head Manufacturing/Head Engineering/Head Quality.

# 4.0	Equipment Description & Identification:

# 4.1       Scope:       Compression Machine CPD-IV 45 Station.

# 4.2      Purpose:   Purpose of compression machine to compress the powder granules with high

# Compaction force and convert into tablet form.

  - Process Equipment Description:

The lower punches, which always remain in the die, hold the powder that is fed into the die, onto it. Further, due to the rotation of the turret, these punches move on the dozer assembly where only required quantity of the powder is held back and the rest is scrapped out. Punches then glide on to the pressure roll. At this stage the upper punch moves down the Upper cam track – Lowering, towards the Main pressure roll after passing under the tamping roll assembly. Both the punches get pressurized under the main pressure rolls. At this instant, due to the heavy compression force, the particles in the formulated powder get bonded to each other and form the tablet. The excessive backpressure generated in this process is absorbed by the hydraulic cylinder, which is suspended onto the lower pressure roll carrier. After the tablet is formed inside the die, it needs to be ejected out. The lower punch moves further on to the ejection cam track and the tablet gets ejected out. At this stage, the upper punch moves onto the Upper cam track – Lifting and glides idly on the upper cam piece. After the tablet is ejected out, the lower punch moves in the lower cam track and comes beneath the feeder and the cycle continues. In one revolution, there is double ejection of tablets.

Complete machine can be divided in following sub sections

- Powder feeding unit (LHS & RHS)

- Powder Transfer unit (LHS & RHS)

- Drive unit, Turret and Compression unit

- Weight Dozer (LHS & RHS)

- Hydraulic Power Pack Assembly.

- Lubrication Unit (Intermittent + Continuous).

- Tablet discharge unit (LHS & RHS)

- Powder Level Sensors. (LHS & RHS)

- Overload safety Unit.

  - USER REQUIREMENTS

  - System Requirements:

| Sr. No. | SYSTEM COMPONENTS | SYSTEM REQUIREMENTS |
| --- | --- | --- |
| 1 | Identification (In case of Equipment /Instrument) | Compression machine CPD-IV 45 ST. |
| 2 | Model/Type | CPDIV-45 station “D” tooling high speed double Rotary Tablet compression machine is controlled through PLC system |
| 3 | Capacity | Min. = 64800 / Max.=324000 (Single Layer) Min. = 32400 / Max.= 67500 @25 RPM (Bi-Layer) Depend on the Product BD |
| 4 | Potential Suppliers | - Cadmach |
| 5 | Contact parts (In case of Equipment) | SS 316 |
| 6 | Non contact parts (In case of Equipment) | SS 304 |
| 7 | Non metallic contact parts (In case of Equipment /Instrument) | - Any material with food grade quality having no Potential impact on the products. 2. Durable. 3. Must be easily cleanable. |
| 8 | Motor & Electrical installations (In case of Equipment /Instrument) | Machine should be operated through PLC mounted on separate electrical control panel. |
| 9 | Machine assemblies (In case of Equipment /Instrument) | Must be covered with SS 304 with mirror finish. |
| 10 | Machine adjustments (In case of Equipment /Instrument) | Setting with Zero clearance with good accuracy. |
| 11 | Packaging & Transport | Should be packed and transported in such a way to avoid any damage during transportation. |
| 12 | No. of requirements | 01 |
| 13 | Requirements for any power failure backup’s (In case of Equipment /Instrument) | To be backed up by installed in-house DG set. After any power failure PLC show continuous counting of all filled & sealed vials. |
| 14 | Gear box specifications(In case of Equipment /Instrument) | As per cGMP model |
| If any other please specify |  |  |
| 15. | Machine specification | CPDIV-45 station “D” tooling high speed double Rotary Tablet compression machine is controlled through PLC system. Machine can be divided into following sub sections. - Powder feeding unit (LHS & RHS) - Powder Transfer unit (LHS & RHS) - Drive unit, Turret and Compression Unit. - Weight Dozer (LHS & RHS) - Hydraulic Power Pack Assembly. - Lubrication Unit (Intermittent + Continuous). - Tablet discharge unit (LHS & RHS) - Powder Level Sensors. (LHS & RHS) - Overload safety Unit. |
|  | Material of Construction | - All contact parts made of S.S.316L, Nylon & all other non-contact components are of S.S.304 or as per the machine requirements - Exterior easy to clean & non-corrosive. |
|  | Safety features | Adequate safety features for men and material are provided along with the equipment. |
|  | Qualifications/ Documentations | The manufacturer shall complete and provide the documents pertaining to Design, Installation, and Operation Qualification. |
|  | Operating System | The equipment shall be operated through PLC system. |
|  | Control System | - The electrical system of the equipment shall be housed as per cGMP and cGEP standards, with adequate safety. - Electrical panel and electro pneumatic panel to be installed on the base structure inside the machine. - Control panel to be housed inside the machine. |

Verified By & Date:

  - Technical Description

    - Specification of components.

| Sr. No. | Components | Descriptions |
| --- | --- | --- |
| 1 | Name | CPDIV-45 stn. Double Rotary |
| 2 | Tooling | “D” tooling |
| 3 | Specification | Contact parts - SS316 (Inclusive of the Die pot table) and Non contact parts – SS 304 |
| 4 | Turret RPM | Min = 12 / Max = 60 |
| 5 | Output (Tablet / Hour.) | Min. = 64800 / Max.=324000 (Single Layer) Min. = 32400 / Max.= 67500 @25 RPM (Bi-Layer) Depend on the Product BD |
| 6 | Hopper capacity | 20 1 Liters |
| 7 | Upper Punch Penetration (Main) | Min = 3 mm to Max = 6 mm |
| 8 | Upper Punch Penetration (Tamp) | Min = 2 mm Max = 5 mm |
| 9 | Tablet thickness | Min= 2 mm Max=8mm 8 mm Thickness is for Flat Tablet. In case your Tablet is Beveled Edge or Concave, the cup depth for the same will be considered as extra. For example: Flat Tablet Thickness = 8 mm, and Concavity of the Tablet is 1 mm each side, then the total thickness will be 10 mm. |
| 10 | Max. Operating pressure (Main) | 100 kN |
| 11 | Max. Tamping Pressure | 10 kN |
| 12 | Max. Tablet Dia. | 25 mm |
| 13 | Max. Depth Of Fill | 20 mm |
| 14 | Approx. Overall dimensions (in mm, with guards open) | 1460 x 1610 x 1960H |
| 15 | Type of Feeding | Forced Feeder |
| 16 | Main Drive Motor | 10 HP / 7.5 kW, 415V, 50 Hz, 3 Phase, 1455 RPM. Make – C & G / BBL |
| 17 | Feeder Drive Motor (02 Nos.) | 0.25 HP / 0.18 kW, 1350 RPM, 415V, 50 Hz, 3 Phase., Make – Bharat Bijlee Ltd |
| 18 | Hydraulic Motor (01 No.) | 1 HP / 0.75 kW, 1410 RPM, 50 Hz, 3 Phase, Make – C & G / BBL |
| 19 | ACVFD for Main Motor | Make – Allen Bradley Specification: 10 HP, 415 V, 3 Ph Model – Power flex 40, |
| 20 | ACVFD for Feeder Motor | Make – Allen Bradley Specification: 1 HP, 415 V, 3 Ph Model – Power flex 4 |
| 21 | PLC | Make – Allen Bradley Model – 1762 L 40 BXB 1200 Micrologix |
| 22 | Powder Level Sensor | Make – IFM Electronics Type: KI5023-KIE3015 |
| 23 | HMI | Make – Proface Specs: 24 V DC, 6” Colour Model: AGP 3301 S1-D-24, Rating Class: 2 |
| 24 | Lubrication Pump | Make – MiniHyd Model – PP1 |
| 25 | Accumulator | Make – Cadmach Type – Mechanical Spring Loaded. |
| 25 | Electromagnetic Clutch | Make – Golden 20 EBA, 24VDC |
| 26 | MCB – (All in Control Panel) | Make – Indokopp Model: Goldline |
| 27 | Contactor – Hydraulic Motor | Make – Sprecher & Schuh Specs: 12 Amps. / 24VAC Model: CA3-12-10, |
| 28 | Overload Relay for Feeder Motor (02 Nos) | Make – SAMWHAEOCR LTD Specs: 0.5 A to 6.0 A |
| 29 | Overload Relay for Hydraulic Motor | Make – Sprecher & Schuh Specs: 1.6 A to 2.5 A Model: CT3-12 |
| 30 | SMPS | Make – OMRON Model: S8JX-050-24CD |
| 31 | Main Isolator Switch | Make – Salzer Model: LB 232 32309 B 33 RDYR |
| 32 | Contactor for Magnetic Clutch | Make – L & T Specs: 24 A/ 240 V AC Model: MR 11 |
| 33 | Bridge Rectifier | Make – Semicron Specs: 5 A / DC Model: MD 5 BU 1016 |
| 34 | Control Relay & Sockets | Make – PLA Specs: 24 V AC, 11 Pin, 3 C/O Model: MPC |
| 35 | Control Relay | Make – Phoenix Specs: 24 V DC, 6A |
| 36 | Control Relay | Make – Phoenix Specs: 24 V AC, 6A |
| 37 | Cooling Fan | Make – Rexonard Specs: 230 V AC, 0.1A |
| 38 | Filter grill for 4” Fan | Make – Rexonard |
| 39 | Push Button | Make – Teknic Specs: 24 v AC Model: 2LHBR 24 |
| 40 | Push Button Contact Ele Green | Make – Teknic Specs: 24 v AC, 4 A Model: Teknic (S2) |
| 41 | Push Button Contact Ele Red | Make – Teknic Specs: 24 v AC, 4 A Model: Teknic (S1) |
| 42 | Emergency Push Button Red | Make – Teknic Specs: 24 v AC |
| 43 | Magnetic Micro Switch for guard | Make – B + K Engg Specs: 24 v AC |
| 44 | Delay Timer | Make – BCH Model: BAR |
| 45 | Pilot Lamp Red (for tower) | Make – Teknic Model: 2LHBR 24 |
| 46 | Micro Switch | Make – Essen Model: MS – 7 |
| 47 | Proxy Switch | Make – Pepperl + Fuchs Model: NBB5 – 18GM50 – E2 |
| 48 | Filter Regulator for Main Incoming Air Supply | Make – FESTO LFR-D-MINI P1 (max) – 16 bar / P2 (max) – 12 bar Gauge – 0 to 16 bar |
| 49 | Lubricator for Main Incoming Air Supply | Make – FESTO, LOE-D-MIDI, P1 (max) – 16 bar |
| 50 | Intermediate regulator for Tablet Take off plate | Make – FESTO LR-D-MINI P1 (max) – 16 bar / P2 (max) – 12 bar |
| 51 | Air Solenoid Valve | Make – FESTO CPE14-M1BH-5L-1/8 |
| 52 | Rotary Actuator for gate operation in the tablet discharge chute | Make – FESTO DSR-12-180-P |
| 53 | Filter Regulator for Lower Punch Lubrication System | Make – FESTO LFR-D-MINI P1 (max) – 16 bar / P2 (max) – 12 bar Gauge – 0 to 16 bar |

Verified By & Date:

  - Specification of PLC

| Make | ALLEN BRADLEY BXB (1200 Micrologix) |
| --- | --- |
| Number of Input / Output | 16 Inputs / 12 Outputs |
| Line Power | 20.4 to 30 VDC |
| Power supply inrush | 24VDC = 4A for 150 ms |
| Input Circuit type | 24VDC, sink / source |
| Output circuit type | 6 Relay output – 6 FET |
| Operating Temperature | +0C to +55C (+32F to +131F) ambient |
| Storage Temperature | -40C to +85C (-40F to +185F) ambient |
| Operating Humidity | 5% to 95% RH (Non – condensing) |
| Vibration | Operating 10 to 500 Hz, 5G, 0.015-in. peak to peak. Relay Operation: 2G |
| Shock (Without data access tool installed) | Operating: 30G Panel mounted (15G DIN rail mounted) Relay Operation: 7.5G Panel mounted (5G DIN rail mounted) Non – Operating: 40G Panel mounted (30G DIN rail mounted) |

  - Utility Requirment:

| Sr no | Specification | Requirment |
| --- | --- | --- |
| 1 | Electrical supply | 3 Phase ,415 V AC ,50 HZ |
| 2 | Compressed Air | 6 kg/cm2 |

Verified By & Date:

  - Material of Construction

  - 5.5.1 Contact and Non contact surfaces.

| # Sr. No |  | # Component Description | # MOC |
| --- | --- | --- | --- |
| 1 | Turret (Special 3 Piece) |  |
| 2 | Upper / Lower piece | SGI 600/3 |
| 3 | Die Table | SS 316 |
| 4 | Press roll - Main (Upper & Lower) | OHNS AISI 01 |
| 5 | Feeder Housing | Gun Metal |
| 6 | Paddle for Force Feeder | SS 316 |
| 7 | Hopper | SS 316 |
| 8 | Butterfly Valve | SS 316 |
| 9 | Discharge Chute | SS 316 |
| 10 | Lower Guard | SS 304 |

# Verified By & Date:.

6.0      COMPLEMENTARY ASPECTS

6.1       Training

| Sr. No. | Specification |  | SYSTEM REQUIREMENTS |
| --- | --- | --- | --- |
| 1 | The vendor Shall supply all available information for the adequate exploitation of equipment. For the Compliance of this purpose at the Job site and/ or at the Vendors Shop. Vendor’s technical staff shall train customer’s personnel. The scope of the Training will be agreed during the contract signature. |  | YES |
| 2 | The supplier is to include the personnel training activities. The contractor is to specify the foreseen time for: - Operator/Supervisor training - Manager Training - Electrical maintenance training - Mechanical Maintenance training |  | YES |
| 3 | The contractor is to specify the personnel background needed for each of the operators maintenance. |  | YES |
| 6.2 Pre Delivery Qualifications (FAT) |  |  |  |
| Sr. No. |  | Specification | SYSTEM REQUIREMENTS |
| 1 |  | The System or its parts as provided for in the scope of supply shall be pre-installed at the vendors shop prior to delivery to customer site. Installation will be completed and documented including mechanical parts as well as electrical connections of all parts to facilitate taking over tests at Vendors shop prior to delivery. | YES |

6.3 Supplier Technical Documentation Requirements:

| Sr. No. | COMPONENTS | REQUIREMENTS |
| --- | --- | --- |
| 1. | Technical Documents | FAT,IQ,OQ Electrical Drawing P & ID diagram GA diagram Operation manual Calibration certificates Bought out components certificates. MOC certificates |

6.4    Technical Manuals

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | Operation manual -02 copies |  |

Verified By & Date

7.0 SAFETY AND ENVIRONMENTAL PROTECTION

| Sr No. | Specification |  | Requirements |
| --- | --- | --- | --- |
| 1 | Environment |  | NA |
| 7.1 Safety and Interlocking |  |  |  |
| Sr. no | Alarm Messages. | Description |  |
| 1 | Turret Motor Overload | The turret motor stops, if the current drawn by the motor is more than 10% of the rated value. |  |
| 2 | LH Feeder Motor Overload | Motor stops if the current drawn by the motor is more than 10% of the rated value |  |
| 3 | RH Feeder Motor Overload | Motor stops if the current drawn by the motor is more than 10% of the rated value |  |
| 4 | LH Dozzer Assembly Not In Position | Machine shall not start. |  |
| 5 | RH Dozzer Assembly Not In Position | Machine shall not start. |  |
| 6 | Batch Size Completed | Machine will not start |  |
| 7 | Emergency Push Button Operated | Machine does not start |  |
| 8 | Machine Guards Open | Machine will not start and will stop if in motion. |  |
| 9 | Guards Selected In Bypass Mode | Machines runs but with red-indication in the MMI |  |
| 10 | Hydraulic Motor Overload | Motor stops if the current drawn by the motor is more than 10% of the rated value |  |
| 11 | Lubrication Oil Level Low | Machine will not start. Machine stops, if in motion. |  |
| 12 | Tablet Counting Proxy Not In Position | - Machines runs, but with red-indication in the MMI - No tablet counting. |  |
| 13 | Main Set Pressure LH Overload | Machine runs with excessive vibration at lower roll carrier |  |
| 14 | Main Set Pressure RH Overload | Machine runs with excessive vibration at lower roll carrier |  |
| 15 | Main Drive In Manual Mode | The machine shall not start. |  |
| 16 | Air Pressure Low. | The machine shall not start. |  |
| 17 | LHS / RHS Powder Level Low. | The machine shall not start. |  |

Verified By & Date:

8.0 CLEANING MAINTENANCE AND SERVICE

| Sr No. | Specification |
| --- | --- |
| 1. | In accordance with cGMP guidelines the units must be easy to clean, to disinfect, and where |
| 2. | The Supplier should guarantee that, if required, a service team can be on site within one working day. |
| 3 | The design should be such as to allow mechanical cleaning of the surface and that the cleanliness of the surface can be checked easily. |
| 4 | All machine parts, in particular instrumentation, should be constructed so that they can be easily removed and calibrated. |
| 5 | All special tools required for running and maintenance should be best. |
| 6 | A spare parts delivery guarantee with in time. |

Verified By & Date:

9.0 RULES AND REGULATION :

These standards, recommendation and requirements are considered  the minimum. Specifications that are more stringent or expansive take the precedence. In case of conflict between published requirements, final determination is the responsibility of the Owners Representative.

10. SCOPE OF DELIVERY

| Sr No. | Specification | Requirements |
| --- | --- | --- |
| 1 | Units described in the specific system requirements including all necessary controls and instrumentation. | YES |
| 2 | The complete mechanical and electrical installation. | YES |
| 3 | The Connections to all the necessary utilities, exhaust, and waste lines necessary for its operation. | YES |
| 4 | All piping and cabling of the units itself. | YES |
| 5 | Wiring and cable run: all wiring and cable run is part of the supply. Alkem Health Science will supply the main power switches to be located in correspondence to the electrical and control cabinets delivered by the equipment supplier. | YES |
| 6 | All internal contacts of the supplied equipment for the required utilities. | YES |
| 7 | Unload on site of the equipment: the supplier is required to define all the necessary handling devices required to the unloading operation. The supplier will inform at least 4 weeks in advance the day of delivery and the list of required handling devices. | YES |
| 8 | Assembling operation: the required consumable, the internal transportation, the assembling tools and the required personal are part of the supply. | YES |
| 9 | A complete set of commissioning spare parts. | YES |
| 10 | All special tools necessary for use and maintenance of the supplied equipment. | YES |
| 11 | A complete set of two years spare parts should be listed quoted and offered as option. | YES |
| 12 | All test activities as specified in this document. | YES |
| 13 | Training in the use and maintenance of the equipment. | YES |
| 14 | A complete set of documentation as specified In this document. | YES |

Verified By & Date:

11.0	 INSTALLATION, COMMISSIONING AND TESTS

11.1    General

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The Contractor must specify for each piece of equipment the Guaranteed performance and the guaranteed system performance. These values will be tested during the acceptance tests. | YES |
| 2 | In addition the functionality described in the user requirements and detailed in the system specifications will be tested. | YES |

11.2 INSTALLATION , COMMISSION

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1 | The commissioning tests will be carried out in accordance with a written test plan developed by the supplier with clearly stated test procedures and acceptance criteria. | YES |
| 2 | The contractor will approve successfully completed tests and will specify items requiring additional work. Representatives from Alkem Health Science. Will attend and participate in the commissioning tests as required. | YES |
| 3 | The installation and commissioning of the system will be performed at the Alkem Health Science. Facility by the contractor. | YES |
| 4 | The commissioning can only start once all the foreseen documents have been delivered by the supplier to Alkem Health Science. | YES |
| 5 | All equipment should be properly installed, adjusted, leveled, tagged, and connected with utilities. | YES |
| 6 | Point to point checks on wiring and pneumatic should be performed. | YES |
| 7 | All instruments should be properly calibrated. | YES |
| 8 | A equipment ( instrument) used for qualification must be listed and approved by Alkem Health Science. | YES |
| 9 | The calibration equipment must have all the necessary documents to demonstrate their maintenance & use. | YES |
| 10 | The last calibration of all this equipment must be less than 6 months old, and evidenced by certificate. | YES |
| 11 | Verification that the interior surfaces of equipment are free of practices and dirt and all points of product contact meet the specified material requirements. | YES |
| 12 | All the clearances and tolerances specified in the drawing or recommended by component manufacturers are correct. | YES |
| 13 | On site verification that valves and other equipment with moving parts are in their normal position if in a power down condition and move in the correct direction with the correct speed and precision. | YES |
| 14 | Verification that all the Input and Output points are connected and labeled according to the documentation and that all the along the input values have been scaled in accordance with the system specification and process requirements. That all equipment components requiring configuration | YES |
| 15 | The commissioning should demonstrate that the system supplied by the contractor has been properly installed and that the functions are in accordance with Alkem Health Science. User Requirements specifications, Vendors System specifications Manuals and other Documentation. | YES |

11.3 Site Acceptance Test   (SAT)

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | This test will be carried out once the commissioning will be completed. The scope will be to verify the performance and the functionality of the system integrated with the other factory systems | YES |
| 2 | The test will be carried out to verify the system response with the expected productivity of the system. | YES |
| 3 | Details on the test realization will be defined during the project Phase. The supplier is asked to specify the proposed duration for SAT and the standard procedure proposed. | YES |
| 4 | During SAT the required functionality, performances and system reliability are met. | YES |
| 5 | The Functionality described in the User Requirements Specification and in the System Specifications are verified and met. | YES |
| 6 | All the documentation agreed has been delivered. | YES |

Verified By & Date:

12.0 QUALIFICATION / VALIDATION

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The maintenance Qualification is responsibility of the customer. However, the supplier is responsible for delivering the basic documents for maintenance qualification. | YES |
| 2. | This includes all side costs such as : calibration measuring equipment and instruments: manpower (IQ and OQ will take place completely on Alkem Health Science.) | YES |
| 3 | Time Schedule for IQ/OQ execution will be developed by Alkem Health Science. With the supplier. | YES |
| 4 | Suppliers personnel used for IQ/OQ must be well trained and experienced. This should be documented. | YES |
| 5 | The onsite test run performed by the supplier might become part of the IQ. | YES |
| 6 | Main IQ/OQ steps such as calibration must be performed and documented in accordance to a SOP approved by Alkem Health Science. | YES |
| 7 | All equipment used for qualification must be listed and approved by Alkem Health Science. The calibration equipment should be well documented. | YES |
| 8 | The last Recalibration of all this equipment should be less than 06 month old. Proofed by Certificate. | YES |
| 9 | OQ can only start after IQ approved by Alkem Health Science. | YES |
| 10 | IQ will be carried out by Alkem Health Science. During Installation phase. IQ will include the tests performed by the contractor. | YES |
| 11 | Part of the OQ will be carried out by Alkem Health Science. During commissioning and SAT phase. OQ will include the tests performed by the contractor. | YES |
| 12 | After installation of the equipment at customers site. Complementary IQ & OQ tests will be performed by the Customer and may be supervised by a member of Technical staff. | YES |
| 13 | Qualification documents (In case of equipments/Instruments) | DQ, IQ, OQ |

Verified By & Date:

13.0 GAURANTEE/WARRANTEE

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The System must be guaranteed including all the sub- system and components for a period of 12 months from the date of the system acceptance for a 03- shift operation. | YES |
| 2 | The servicing companies involved for the Sub- systems maintenance must be declared and the maintenance group organization described. Furthermore, the contractor will be directly responsible of the system assistance and the required operation will be co- ordinate by him. | YES |
| 3 | In case of failures, the intervention will be guaranteed by the contractor within a maximum time limit. The contractor is asked to specify the maximum time limit. | YES |
| 4 | The supplier is asked to propose as option maintenance and assistance contract after the guarantee expiration. | YES |

Verified By & Date:

# 14.0 Deviation

# 15.0 Annexure

# 16.0 Summary and Conclusion

# 17.0 Approval of Design Qualification.

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

# 18.    Acceptance By vendor

# Name of Vendor:

# Sign/Date: