# Design QUALIFICATION

# NAME OF THE ITEM: TABLET COATING EQUIPMENT

# FUNCTIONAL AREA:  C- BLOCK

# PROTOCOL No. :        DQ/

ALKEM HEALTH SCIENCE

SAMARDUNG, KAREK BLOCK,

PO. NAMTHANG,

SOUTH SIKKIM-737137

CONTENT

Sr. No.	  Description

- Protocol Approval

- Objective

- Responsibility

- Equipment Description & Identification

- User Requirements

- Complementary Aspects

- Safety and Environmental Protection

- Cleaning maintenance and service

- Rules and Regulation

- Scope of Delivery

- Installation ,Commissioning and Tests

- Qualification /Validation

- Guarantee/Warrantee

- Deviation

- Annexure

- Summary and Conclusion

- Approval of Design Qualification

- Acceptance by vendor

- PROTOCOL APPROVAL:

Protocol Prepared By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |

DQ Reviewed By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Engineering |  |  |  |
| Production |  |  |  |
| Quality Assurance |  |  |  |

DQ Approved By:

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

- Objective:

The purpose of this document is to ensure that all the critical aspects of the Equipment,

cGMP & Safety features have been considered in designing the equipment/instrument and

is properly Documented

- Responsibilities:

In accordance with the document, following functions shall be responsible for initiation and finalization of Equipment user requirement specification. When the work is carried by contract/ consulting staff, all the work is to be performed under the oversight of Alkem.

3.1 Preparation of Document

- User department to prepare the DQ

- Ensures that the document is in compliance with current policies and procedures of cGMP regulations.

- Ensures that the content is sufficient, clearly defined, technically sound and accurate.

- It is a Guidance document to prepare the DQ.

# 3.2 Review of Document

- To be reviewed by Head of the user department and functional department ( Engineering & Quality assurance)

# 3.3 Approval of Document

- Approval of document by Head Manufacturing/Head Engineering/Head Quality.

# 4.0	Equipment Description & Identification:

# 4.1       Scope:  TABLET COATING EQUIPMENT GC SMART 500.

# 4.2      Purpose:   Purpose of Tablet coating machine is to provide the film on the compressed

# Tablets.

Process Equipment Description:

Coaters are used mainly in the pharmaceutical industry to coat tablets. This machine requires special knowledge and skills to operate. The parts of this Coater have been constructed and built according to state of the art technology and safety regulations. It is nevertheless possible that the operator or a third person may be exposed to danger to life and limb if the equipment is improperly used and operated, or that damage to the coater and other installations may occur. The tablets to be coated are loaded into the Pan of the GCS 500. The Pan consists of two conical end pieces and a cylindrical central part fully perforated with 3mm dia. There is central opening at the front and solid flanges welded onto the back to attach the pan to the Pan bearings. Hot, filtered air is passed at high velocity from the inlet of the Coater through the Tablets. Because of the high temp of the air, the tablets are heated. The Pan is rotated for mixing of the tablets. The spraying of solution is done through the Spray nozzles. The solution is pumped through the peristaltic pump. The processed air inside the pan is taken out with the help of exhaust blower.

# The machine can be divided into the following sub sections:

• Exhaust air handling unit

• Coater (Main Unit)

• Spraying System

  - USER REQUIREMENTS

  - System Requirements:

| Sr. No. | SYSTEM COMPONENTS | SYSTEM REQUIREMENTS |
| --- | --- | --- |
| 1 | Identification (In case of Equipment /Instrument) | Tablet Coating equipment GCS 500 |
| 2 | Model/Type | Tablet Coating equipment GCS 500 , Tablet Coating machine is controlled through PLC system. |
| 3 | Capacity | 500 ltr |
| 4 | Potential Suppliers | Pam Glatt Pharma Technologies Pvt. Ltd. |
| 5 | Contact parts (In case of Equipment) | SS 316 L |
| 6 | Non contact parts (In case of Equipment) | SS 304 |
| 7 | Non metallic contact parts (In case of Equipment /Instrument) | - Any material with food grade quality having no Potential impact on the products. 2. Durable. 3. Must be easily cleanable. |
| 8 | Motor & Electrical installations (In case of Equipment /Instrument) | Machine should be operated through PLC mounted on separate electrical control panel. |
| 9 | Machine assemblies (In case of Equipment /Instrument) | Must be covered with SS 304 with mirror finish. |
| 10 | Machine adjustments (In case of Equipment /Instrument) | Setting with Zero clearance with good accuracy. |
| 11 | Packaging & Transport | Should be packed and transported in such a way to avoid any damage during transportation. |
| 12 | No. of requirements | 01 |
| 13 | Requirements for any power failure backup’s (In case of Equipment /Instrument) | To be backed up by installed in-house DG set. After any power failure. |
| 14 | Gear box specifications(In case of Equipment /Instrument) | As per cGMP model |
| 15. | Machine specification | Tablet coating equipment GCS 500 for the film coating of compressed tablets. Through PLC system. Machine can be divided into following sub sections. • Exhaust air handling unit • Coater (Main Unit) • Spraying System |
|  | Material of Construction | - All contact parts made of SS 316L, Nylon & all other non-contact components are of SS 304 or as per the machine requirements - Exterior easy to clean & non-corrosive. |
|  | Safety features | Adequate safety features for men and material are provided along with the equipment. |
|  | Qualifications/ Documentations | The manufacturer shall complete and provide the documents pertaining to Design, Installation, and Operation Qualification. |
|  | Operating System | The equipment shall be operated through PLC system. |
|  | Control System | - The electrical system of the equipment shall be housed as per cGMP and cGEP standards, with adequate safety. - Electrical panel and electro pneumatic panel to be installed on the base structure inside the machine. - Control panel to be housed inside the machine. |

Verified By & Date:

  - Technical Description

    - Specification of components.

| Sr. No. | Components | Descriptions |
| --- | --- | --- |
| 1 | Name | GC SMART 500 |
| 2 | Specification | Contact parts - SS316 L and Non contact parts – SS 304 |
| 6 | Capacity | 500 liters |
| 7 | Coater (Main Housing) | AISI 316L & 304 |
| 8 | Pan | Make : Pam Glatt Pharma Tech. Pvt. Ltd AISI 316L Punched perforations: interior dia 3mm Free area: 32%. Outside pan : pickled and brushed Inside pan: mirror polished 420 Grit(Ra 0.2 µm max.) |
| 9 | Mixing Baffles | Make: Pam Glatt Pharma Tech. Pvt. Ltd AISI 316L Reversed Fischer Baffles- 4 Nos. |
| 10 | Pan Motor | Make : M/s Hindustan Motors Pan Drive (motor) Flame proof motor. Pan Drive (gear).reduction gear box. Toothed flywheel ring for pan drive |
| 11 | Exhaust Air Fan & Motor | Make : Pam Glatt Pharma Tech. Pvt. Ltd Blower motor shall be in FLP of in general shall comprise of ducting. The blower shall be in spark resistant material i.e. MS casing and Al Impeller. Flameproof motor with overload protection will be provided Fan housing will be single walled painted Impeller material will be Aluminium and statically & dynamically balanced. SS Bellow (AISI304) will be provided between the blower and duct Sound relaxation silencer will be provided. ACVF Drive is provided for blower control. |
| 12 | Operating Control Panel | The control panel is made of SS304 along with Schneider PLC based HMI Contec make & Samsung make CLP 321n printer. All the transmitters are to be mounted inside the panel. All interfacing tubing/cables will be passed through SS conduits in manufacturing area. All the major parameters like flap, temperature, atm air pressure, process time, machine setting will be monitored through control panel. One panic push button (On Coater Housing) for switching off the machine in emergency will be provided. One Emergency stop push button for Stopping the machine on operating panel. Atomising air pressure adjustment through PLC. Four level password protections are provided (for operator, supervisor, recipe & maintenance). |
| 13 | PLC | Process On / Off Spray Test, Spray On/Off Exhaust air temperature – set & actual DP HEPA & Wet Scrubber with Alarm Set Air Flow measurement in m/sec or CFM. Different Safety Alarms & interlock as per list |
| 14 | Colour Samsung CLP 321N Printer | Real time & date, Product Name , Batch Info, Set parameters (Inlet temp) Actual Parameters (Inlet, exhaust, RH, Pan D.P., ATM Air) Air Velocity/Flow Alarms and error messages |
| 15 | Pneumatic & Electrical Panel | Electro pneumatic panel is to be installed in service area |
| 16 | Coater Housing | Make : Pam Glatt Pharma Tech. Pvt. Ltd Material-AISI 316L (except front sheet and door covers, i.e.SS304) Surface finish- Matt Finishing Pan illumination position-Top side of coater housing Drain-Bottom side of housing |
| 17 | Coating Pan | Make: Pam Glatt Pharma Tech. Pvt. Ltd Material-AISI 316L Punched perforations-Hole dia 3mm Punched perforations-Interior dia 3mm Outside dia.- 1570 mm Max. working volume- 500 L Surface finish-Mirror polish |
| 18 | Mixing baffles, Qty: 5 Nos. & Reduced volume -5 Nos. | Make-Pam Glatt Pharma Tech. Pvt. Ltd Type-Fischer Reversed Material-AISI 316L Surface Finish-Mirror (Grit 420) |
| 19 | Drum Assembly (Pan Motor) | Make: M/s Hindustan Motors Motor Power-2.2 kW Motor Speed-1430 RPM Phase-3 Phase Supply-415 ± 10 % Volts Frequency-50 Hz Motor Starters-ACVF Drive |
| 20 | Drum Assembly (Gear Box) | Make: M/s SEW Eurodrive Reduction Ratio- 64.75 Type- KF77AM112/RS Output Flange size- 300mm |
| 22 | Spray Medium Nozzles(Spray Nozzles) (GCSD 1.1), Qty : 5 Nos. | Make : Glatt GmbH Material- Stainless Steel 316L Nozzle Size- 1.0 mm. |
| 23 | Spray Medium Nozzles (WIP troll ball on spray arm for inside Pan) | Make : Siddharth Material- AISI 316L Capacity- 70 lpm Connection- ¾'' BSP size |
| 24 | Spray Medium Nozzles (WIP Nozzles on Roof for outside Pan (Flat Jet Sprinkler nozzle) - 8 Nos. | Make: Siddharth Material- AISI 316L Capacity- 5 lpm |
| 25 | Spray Medium Nozzles (Manifold Assembly for spray liquid) | Make: Pam Glatt Pharma Tech. Pvt.Ltd. Material- AISI 316L Nos. of ports- 5 Clamping- TC end clamping |
| 25 | Spray Medium Nozzles (Spray Pump) | Make : Flowtech Model- FP-04 Flame proof Pump Type - Single Head |
| 26 | Spray Medium Nozzles (Spray pump motor) | Make : Hindustan Motors Power- 0.75kW Speed- 1410 RPM |
| 27 | Side door assembly | Make : Pam Glatt Pharma Tech. Pvt.Ltd Material- AISI 304 |
| 28 | Top railing assembly | Make: Pam Glatt Pharma Tech. Pvt.Ltd. Position- Coater Housing Bottom |
| 29 | Glass Door Assembly with ‘D’ type door | Make : Pam Glatt Pharma Tech. Pvt. Ltd MOC -Toughened Glass Glass Size -OD 719mm x Thk.15mm Type- “D” Type Sampling Door |
| 36 | Structure assembly | Make: Pam Glatt Pharma Tech. Pvt.Ltd. Material -M.S. (IS 2062) Position -Coater Housing Bottom |
| 37 | Exhaust System (Exhaust Air Control Flap) | Make: Pam Glatt Pharma Tech. Pvt. Ltd Housing -Material Aluminium Material - Flap Disc. -Stainless Steel 304 Material - Drive Shaft -Stainless Steel 304 Material Sealing -Silicone Drive -Pneumatic Actuator, Rotex make |
| 38 | Exhaust System(Exhaust Blower assembly) | Make: NADI Industrial Fans. Impeller Material- Aluminium Housing Material -MS Air Flow Capacity- 6000 m³/hr Static Diff. Pressure -900 mmWGP (At 200C) Motor FLP / Non FLP- Flame Proof Motor Power- 30kW Motor RPM- 2960 Phase -3 Phase |
| 39 | Exhaust System (Exhaust Blower assembly) | Make: Supply -415+/-10% Volts Frequency -50 Hz Motor Starters -ACVF Drive Silencer for Blower -Provided. |
| 40 | Booster Pump Assembly | Make : Pam Glatt Pharma Tech. Pvt. Ltd Fittings and interconnections MOC -AISI 316 |
| 41 | Booster Pump Assembly (PUMP) | Make : Grundfos Model -CM 10-3 ARGV AQQV FAAN Rated Flow -6m3 / hr Material -AISI 316 |
| 42 | Booster Pump Assembly(BALL VALVE WITH ACTUATOR AT INLET AND OUTLET) | Make : Micro pneumatics Type - Pneumatically operated ball valve Working Pressure- 5-8 kg/cm2 Material- AISI 316 Inlet / outlet connectionsize-1 1/2” TC end connection Gland body packing- PTFE |
| 43 | PC Control | Make : Contec Model- PT1561E Resolution-15” TFT touch-screen display, 1024 x768 Degree of Protection -IP65 front Wide DC Supply- 9-36VDC RAM -1GB , 160GB shockproof HDD |
| 44 | Printer | Make: Samsung Model -CLP 321N |
| 45 | PLC | CPU make : Schneider Electric Model -M340 series |
| 46 | PLC (Analog Input Card (Analog to Digital), Qty: 1 Nos.) | Make : Schneider Electric Model -BMX AMI 0800 Channels- 8 |
| 47 | PLC (Analog Output Card (Digital to Analog), Qty : 2 No. | Make : Schneider Electric Model -BMX AMO 0410 Channels -4 |
| 48 | PLC (Digital Input – 1 Nos.) | Make : Schneider Electric Model -DDI 1602 Channel -16 |
| 49 | PLC (Digital Output – 2 Nos.) | Make : Schneider Electric Model- DRA 1605 Channel -16 |

Verified By & Date:

  - Specification of PLC

The main function of a PLC is to translate the instructions into the digital or analogue codes needed to operate the device or machine. PLC system collects data from field instrumentation & displays the information on the operator station. The instruments are connected to the system Equipment.

  - Utility Requirment:

| Sr no | Specification | Requirment |
| --- | --- | --- |
| 1 | - Electrical Phase Voltage frequency Total Electric consumption | 3 Ph and Neutral 415±10 V%,50 Hz. 40 KW |
| 2 | - Compressed Air Consumption Flow pressure Quality Line Size | 170 m3/h at 6 bar Min. 6 bar (Kg) oil, water & dust free Min 1 inch with T connection near panel |
| 3 | - Steam consumption Steam Pressure Steam Consumption Steam Inlet line Condensate Line | Min 3.5 bar and max 4.5 bar 283 kg/hr 1 ½ “ size 1” size |

Verified By & Date:

  - Material of Construction

  - 5.5.1 Contact and Non contact surfaces.

| # Sr. No |  | # Component Description | # MOC |
| --- | --- | --- | --- |
| 1 | Coater Housing | AISI 316L |
| 2 | Coating Pan | AISI 316L |
| 3 | Mixing baffles | AISI 316L |
| 4 | Spray Medium Nozzles | Stainless Steel 316L |
| 5 | Glass Door Assembly | Toughened Glass |
| 6 | Structure assembly | M.S. (IS 2062) |
| 7 | Booster Pump Assembly | AISI 316 |

# Verified By & Date:.

6.0      COMPLEMENTARY ASPECTS

6.1       Training

| Sr. No. | Specification |  | SYSTEM REQUIREMENTS |
| --- | --- | --- | --- |
| 1 | The vendor Shall supply all available information for the adequate exploitation of equipment. For the Compliance of this purpose at the Job site and/ or at the Vendors Shop. Vendor’s technical staff shall train customer’s personnel. The scope of the Training will be agreed during the contract signature. |  | YES |
| 2 | The supplier is to include the personnel training activities. The contractor is to specify the foreseen time for: - Operator/Supervisor training - Manager Training - Electrical maintenance training - Mechanical Maintenance training |  | YES |
| 3 | The contractor is to specify the personnel background needed for each of the operators maintenance. |  | YES |
| 6.2 Pre Delivery Qualifications (FAT) |  |  |  |
| Sr. No. |  | Specification | SYSTEM REQUIREMENTS |
| 1 |  | The System or its parts as provided for in the scope of supply shall be pre-installed at the vendors shop prior to delivery to customer site. Installation will be completed and documented including mechanical parts as well as electrical connections of all parts to facilitate taking over tests at Vendors shop prior to delivery. | YES |

6.3 Supplier Technical Documentation Requirements:

| Sr. No. | COMPONENTS | REQUIREMENTS |
| --- | --- | --- |
| 1. | Technical Documents | FAT,IQ,OQ Electrical Drawing P & ID diagram GA diagram Operation manual Calibration certificates Bought out components certificates. MOC certificates |

6.4    Technical Manuals

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | Operation manual -02 copies |  |

Verified By & Date

7.0 SAFETY AND ENVIRONMENTAL PROTECTION

| Sr No. | Specification |  | Requirements |
| --- | --- | --- | --- |
| 1 | Environment |  | NA |
| 7.1 Safety and Interlocking |  |  |  |
| Sr. no | Alarm Messages. | Description |  |
| 1 | Guards open | Machine stop. |  |
| 2 | Air pressure low | Machine stop. |  |
| 3 | Temp. low | Machine stop |  |
| 4 | Temp. High | Machine stop |  |
| 5 | Nozzle Jam | Machine stop |  |
| 6 | Pan is not moving | Machine stop & Trip the spray process. |  |
| 7 | Emergency stop | Machine stop & Trip the process |  |
| 8 | AHU Door Open | Machine stop & Trip the process |  |
| 9 | M/C Drain Water Level High | Machine stop & Trip the process |  |
| 10 | M/C Drain Valve not Opened | Machine stop & Trip the process |  |
| 11 | Process Time End | Machine stop & Trip the process |  |

Verified By & Date:

8.0 CLEANING MAINTENANCE AND SERVICE

| Sr No. | Specification |
| --- | --- |
| 1. | In accordance with cGMP guidelines the units must be easy to clean and where necessary. |
| 2. | The Supplier should guarantee that, if required, a service team can be on site within one working day. |
| 3 | The design should be such as to allow mechanical cleaning of the surface and that the cleanliness of the surface can be checked easily. |
| 4 | All machine parts, in particular instrumentation, should be constructed so that they can be easily removed and calibrated. |
| 5 | All special tools required for running and maintenance should be best. |
| 6 | A spare parts delivery guarantee with in time. |

Verified By & Date:

9.0 RULES AND REGULATION :

These standards, recommendation and requirements are considered  the minimum. Specifications that are more stringent or expansive take the precedence. In case of conflict between published requirements, final determination is the responsibility of the Owners Representative.

10. SCOPE OF DELIVERY

| Sr No. | Specification | Requirements |
| --- | --- | --- |
| 1 | Units described in the specific system requirements including all necessary controls and instrumentation. | YES |
| 2 | The complete mechanical and electrical installation. | YES |
| 3 | The Connections to all the necessary utilities, exhaust, and waste lines necessary for its operation. | YES |
| 4 | All piping and cabling of the units itself. | YES |
| 5 | Wiring and cable run: all wiring and cable run is part of the supply. Alkem haelth science will supply the main power switches to be located in correspondence to the electrical and control cabinets delivered by the equipment supplier. | YES |
| 6 | All internal contacts of the supplied equipment for the required utilities. | YES |
| 7 | Unload on site of the equipment: the supplier is required to define all the necessary handling devices required to the unloading operation. The supplier will inform at least 4 weeks in advance the day of delivery and the list of required handling devices. | YES |
| 8 | Assembling operation: the required consumable, the internal transportation, the assembling tools and the required personal are part of the supply. | YES |
| 9 | A complete set of commissioning spare parts. | YES |
| 10 | All special tools necessary for use and maintenance of the supplied equipment. | YES |
| 11 | A complete set of two years spare parts should be listed quoted and offered as option. | YES |
| 12 | All test activities as specified in this document. | YES |
| 13 | Training in the use and maintenance of the equipment. | YES |
| 14 | A complete set of documentation as specified In this document. | YES |

Verified By & Date:

11.0	 INSTALLATION, COMMISSIONING AND TESTS

11.1    General

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The Contractor must specify for each piece of equipment the Guaranteed performance and the guaranteed system performance. These values will be tested during the acceptance tests. | YES |
| 2 | In addition the functionality described in the user requirements and detailed in the system specifications will be tested. | YES |

11.2 INSTALLATION, COMMISSION

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1 | The commissioning tests will be carried out in accordance with a written test plan developed by the supplier with clearly stated test procedures and acceptance criteria. | YES |
| 2 | The contractor will approve successfully completed tests and will specify items requiring additional work. Representatives from Alkem haelth science. Will attend and participate in the commissioning tests as required. | YES |
| 3 | The installation and commissioning of the system will be performed at the Alkem haelth science. Facility by the contractor. | YES |
| 4 | The commissioning can only start once all the foreseen documents have been delivered by the supplier to Alkem haelth science. | YES |
| 5 | All equipment should be properly installed, adjusted, leveled, tagged, and connected with utilities. | YES |
| 6 | Point to point checks on wiring and pneumatic should be performed. | YES |
| 7 | All instruments should be properly calibrated. | YES |
| 8 | A equipment ( instrument) used for qualification must be listed and approved by Alkem haelth science. | YES |
| 9 | The calibration equipment must have all the necessary documents to demonstrate their maintenance & use. | YES |
| 10 | The last calibration of all this equipment must be less than 6 months old, and evidenced by certificate. | YES |
| 11 | Verification that the interior surfaces of equipment are free of practices and dirt and all points of product contact meet the specified material requirements. | YES |
| 12 | All the clearances and tolerances specified in the drawing or recommended by component manufacturers are correct. | YES |
| 13 | On site verification that valves and other equipment with moving parts are in their normal position if in a power down condition and move in the correct direction with the correct speed and precision. | YES |
| 14 | Verification that all the Input and Output points are connected and labeled according to the documentation and that all the along the input values have been scaled in accordance with the system specification and process requirements. That all equipment components requiring configuration | YES |
| 15 | The commissioning should demonstrate that the system supplied by the contractor has been properly installed and that the functions are in accordance with Alkem haelth science. User Requirements specifications, Vendors System specifications Manuals and other Documentation. | YES |

11.3 Site Acceptance Test   (SAT)

| Sr. No. | Specification | Requirements |
| --- | --- | --- |
| 1. | This test will be carried out once the commissioning will be completed. The scope will be to verify the performance and the functionality of the system integrated with the other factory systems | YES |
| 2 | The test will be carried out to verify the system response with the expected productivity of the system. | YES |
| 3 | Details on the test realization will be defined during the project Phase. The supplier is asked to specify the proposed duration for SAT and the standard procedure proposed. | YES |
| 4 | During SAT the required functionality, performances and system reliability are met. | YES |
| 5 | The Functionality described in the User Requirements Specification and in the System Specifications are verified and met. | YES |
| 6 | All the documentation agreed has been delivered. | YES |

Verified By & Date:

12.0 QUALIFICATION / VALIDATION

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The maintenance Qualification is responsibility of the customer. However, the supplier is responsible for delivering the basic documents for maintenance qualification. | YES |
| 2. | This includes all side costs such as : calibration measuring equipment and instruments: manpower (IQ and OQ will take place completely on Alkem haelth science.) | YES |
| 3 | Time Schedule for IQ/OQ execution will be developed by Alkem haelth science. With the supplier. | YES |
| 4 | Suppliers personnel used for IQ/OQ must be well trained and experienced. This should be documented. | YES |
| 5 | The onsite test run performed by the supplier might become part of the IQ. | YES |
| 6 | Main IQ/OQ steps such as calibration must be performed and documented in accordance to a SOP approved by Alkem haelth science. | YES |
| 7 | All equipment used for qualification must be listed and approved by Alkem haelth science. The calibration equipment should be well documented. | YES |
| 8 | The last Recalibration of all this equipment should be less than 06 month old. Proofed by Certificate. | YES |
| 9 | OQ can only start after IQ approved by Alkem haelth science. | YES |
| 10 | IQ will be carried out by Alkem haelth science. During Installation phase. IQ will include the tests performed by the contractor. | YES |
| 11 | Part of the OQ will be carried out by Alkem haelth science. During commissioning and SAT phase. OQ will include the tests performed by the contractor. | YES |
| 12 | After installation of the equipment at customers site. Complementary IQ & OQ tests will be performed by the Customer and may be supervised by a member of Technical staff. | YES |
| 13 | Qualification documents (In case of equipments/Instruments) | IQ, OQ |

Verified By & Date:

13.0 GAURANTEE/WARRANTEE

| Sr.No. | Specification | Requirements |
| --- | --- | --- |
| 1. | The System must be guaranteed including all the sub- system and components for a period of 12 months from the date of the system acceptance for a 03- shift operation. | YES |
| 2 | The servicing companies involved for the Sub- systems maintenance must be declared and the maintenance group organization described. Furthermore, the contractor will be directly responsible of the system assistance and the required operation will be co- ordinate by him. | YES |
| 3 | In case of failures, the intervention will be guaranteed by the contractor within a maximum time limit. The contractor is asked to specify the maximum time limit. | YES |
| 4 | The supplier is asked to propose as option maintenance and assistance contract after the guarantee expiration. | YES |

Verified By & Date:

# 14.0 Deviation

# 15.0 Annexure

# 16.0 Summary and Conclusion

# 17.0 Approval of Design Qualification.

| Functional area | Name | Signature | Date |
| --- | --- | --- | --- |
| Head Engineering |  |  |  |
| Head Manufacturing |  |  |  |
| Head Quality |  |  |  |

# 18.    Acceptance By vendor

# Name of Vendor:

# Sign/Date: