1.0   Revision History

| Rev. No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for storage of sterilized aluminium seals.

3.0    Scope

This SOP is applicable for storage of sterilized aluminium seals in aseptic area of C Block.

4.0	Responsibility

Officer and Above Production/ IPQA Officer	:	Implementation of Procedure.

5.0    Accountability

Production Head.

6.0    Procedure

6.1    Ensure the LAF is in working condition and pressure differential is within the specified

Limit (8 to 15 mm of wc) prior to unloading.

6.2    Ensure the cleanliness of cool zone.

6.3    Ensure the availability of pre sterilized storage containers for storage of sterilized aluminium seals.

6.4     Sanitize your hand with 70% IPA before any contact as per SOP No. PC/057.

6.5     After completion of sterilization of aluminium seals, open the door of autoclave towards aseptic side.

6.6     Fix the autoclave floor trolley to the rails of the autoclave, lock it, and pull the autoclave rack trolley on the rails of the floor trolley under LAF.

6.7     Lock the autoclave rack trolley with the floor trolley. Bring the storage containers under the LAF.	6.8     Remove the autoclave rack trolley from locking plate by unscrewing the bolt.

6.9     Remove storage nylon bag one by one, open the storage nylon bag and unload aluminium seals as instructed in 6.10.

6.10  Open the lid of storage containers and unload sterilized aluminium seals from nylon bag under

LAF.

6.11  Check for the shape, cleanliness & dryness of the sterilized aluminium seals.

NOTE: If the aluminium seals are not clean or dry then inform to the production executive/ manager to investigate the reason for failure.

6.12   Close the lid of storage containers and store on pallets under LAF.

6.13   Load the autoclave rack on the frame, unlock the autoclave rack trolley from the floor trolley and push the trolley to the autoclave.

6.14   Close the door of the autoclave and inform washing area personnel.

6.15   Sterilized aluminium seals can be stored in storage containers under LAF maximum up to 72

Hours.

7.0   Annexure (S)

NA

8.0   Reference (S)

SOP NO. - PC/057

9.0   Glossary

SOP   	: 	Standard Operating Procedure.

LAF	: 	Laminar air flow.

NO      :	Number

PC	: 	Production (C-Block)