- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | Increase the hold time study of freshly prepared 70% IPA. | Due to change hold time study from 24 hours to 72 hours of freshly prepared 70% IPA. |

- Objective

To lay down a procedure for receiving and fabrication of sterile raw material.

- Scope

This SOP is applicable for receiving and fabrication of sterile raw materials in C Block.

- Responsibility

Officer and Above Production/IPQA officer	:	Implementation of Procedure

- Accountability

Production Head.

- Procedure

  - Prestart Up

    - Ensure that BPCR of the product is available along with BOM of Raw material, duly checked and signed by production, QA person and warehouse person.

    - Ensure that dynamic pass box is in working condition and differential pressure is within the

Specified limit.

    - Ensure that temperature and humidity of the room is within specified limit as per the BPCR.

  - Operation

    - During receiving of sterile raw materials check the physical condition of the containers and ensure that the approved labels on the raw material containers are affixed. Check and verify the following details on approved label.

- Name of raw material

- Item code

- B.No

- A.R.No. of API

- Manufacturing Date of API

- Expiry date of API

- Retest date of API

- Potency

    - After verification of the weight transfer the RM containers to the RM storage area in the respective injectable lines.

    - Verify the weight and enter the values in the BPCR of the product for which the material was dispensed. Get it verified by QA personnel.

    - Mop the outer surface of container with the help of lint free mop using 70% IPA solution thoroughly. Place the container in dynamic pass box, close the door, place three to five container or as per space available in dynamic pass box allow them to stay under UV for 10 minutes then ask the person in aseptic area to remove the container under LAF.

    - In aseptic area mop the container with the help of sterile lint free mop using freshly prepared 70% IPA (Use before 72hrs).

    - Weigh each container.

    - Transfer RM container to the vial filling room of the aseptic area, mop the container with 70% IPA using lint free sterile mop.

    - Open the lid of the container under LAF and charge the sterile raw material in the hopper of the vial-filling machine.

    - Adjust the weight and start the filling of vial as per SOP No. PC/094.

    - Continue the filling operation till the entire RM gets utilized.

    - For blending operation the above operation remain same up to 6.2.8 then open the RM under LAF in blending room, weigh the required quantity of raw material under LAF in pre sterilized container.

    - Charge the material in sterile blender and start the blending operation as per SOP No. PC/104.

- Annexure (S)

NA

- Reference (S)

SOP. NO. - PC/094

PC/104

- Glossary

SOP   	: Standard Operating Procedure

LAF	: Laminar air flow			                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                     IPA	: Isopropyl Alcohol								                 NO     	: Number

QA	: Quality Assurance

BPCR	: Batch production & control record

BOM	: Bill of Material

RM	: Raw Material

PC	: Production C-Block

A.R.No.	: Analytical Report Number

B.No. 	: Batch number.