- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for washing and sterilization of machine parts and accessories.

# 3.0    Scopes

This SOP is applicable for the persons working in the washing and sterilization area and for the   machine parts and accessories to be used for aseptic processing of C Block.

# 4.0	Responsibility

Officer and Above Production/ IPQA Officer	:	Implementation of procedure.

5.0	   Accountability

Production Head.

6.0    Procedure

6.1    Collect the used machine parts and accessories from the Exit Dynamic pass box and bring the same in a hospital trolley to the static pass box of material transfer area.

6.2   Transfer the machine parts and accessories to the unit preparation area through static pass box.

6.3	Keep the machine parts and accessories in the sink of the unit preparation area one by one and ensure that all the parts are available.

6.4	Wash the components one by one initially with purified water followed by final rinse with WFI  and then store them under the Laminar airflow on the SS table.

6.5	Ensure that after final rinsing machine components should not be kept more than 4 hours.        If the hold time is more than 4 hours then again go for rinsing with WFI before sterilization.

6.6    Check and transfer the components to the autoclave in a hospital trolley and sterilize them as  per the specified Load pattern No. as per SOP No. PC/103.

7.0    Annexure (S)

NA

8.0   Reference (S)

SOP NO. - PC/103

9.0   Glossary

SOP   	:    Standard Operating Procedure.

LAF	:    Laminar air flow.

NO.     :   Number

WFI	:    Water for injection

SS	:    Stainless steel

PC	:    Production (C-Block)