- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for material movement in aseptic area.

# 3.0    Scopes

This SOP is applicable for the material movement in aseptic area of C Block.

# 4.0    Responsibility

Officer and Above Production/ IPQA Officer	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

6.0   Procedure

6.1	Material movement in aseptic area starts from the dispensed Raw material store & dispensed Primary packing material store for Injectable section.

6.2 Dispensed sterile raw material moves to the blending area through dynamic pass box after sanitization, should be stored under UV light for NLT 10 min, to be taken inside by aseptic area personnel and sanitization to be done before keeping it on the hospital trolley.

  - Then the material to be shifted to the Vial filling area for starting of vial filling operation.

6.4    Primary packing materials to be shifted from the dispensed primary packing material store   through static pass box for rubber plugs & aluminium seals and to the decartoning area for the glass vials.

6.5   Glass vials starts moving through pneumatic lifting device, pre-inspection, vial washing machine, sterilization tunnel and to the vial-filling machine.

6.6   Rubber plug and aluminium seal are transferred to the unit preparation area or garment washing area through static pass box and then transferred for autoclaving to the autoclave and then unload it after sterilization in the cool zone area. Then the sterilized rubber plugs are shifted to the vial filling & bunging room. Sterilized seals are transferred to the vial sealing room.

6.7   Filled vials are transferred to the bunging unit and after bunging vials move into the sealing room for sealing and then passed to the optical inspection area in packing hall.

- Annexure (S)

NA

8.0 Reference (S)

NA

9.0 Glossary

SOP   	: Standard Operating Procedure.

NLT		: Not less Than

NO.     : Number

PC	: Production (C-Block)