- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for monitoring of pressure differentials.

# 3.0    Scopes

This SOP is applicable for the Procedure for monitoring of pressure differentials of C Block Injection line.

# 4.0	 Responsibility

Officer and Above Production/ IPQA Officer	:	Implemented the Procedure.

5.0    Accountability

Production Head.

6.0    Procedure

  - Before start of any production operation, check the Pressure Differential of the areas.

6.2   Monitoring of Pressure differentials in Injection line I & II are done in daily basis at the interval of two hours during operation and at the interval of four hours during non operation.

6.3   The limits are as mentioned in respective formats.

6.4   For monitoring of Pressure Differentials, Calibrated Magnehellic Gauges are to be used and  recording is to be done as per Annexure –I.

6.5   The Pressure differentials for Aseptic area passage are negative pressure as compared to other areas. The Pressure Differential limits have been written on the respective magnehellic gauges.

6.6   If the pressure differential is disturbed stop the production and inform Production Executive/Officer to take required steps and start the activity only when the pressure differential is maintained.

7.0   Annexure (S)

Annexure–I    :    Procedure for Monitoring Of Pressure Differentials.

8.0  Reference (S)

NA

9.0 Glossary

SOP   	: Standard Operating Procedure.

NO     	: Number

PC		: Production C-Block