- Revision History

| Rev. No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. change in sop format 2. change in font type from time Roman to Tahoma & font size from 12 to 10 | 1. Periodic Review 2. to make the document user friendly. |

2.0	Objective

To lay down a procedure for operation and cleaning of CONDONT printer.

3.0	Scope

This SOP is applicable for operation and cleaning of CONDOT printer in B block.

4.0	Responsibility

Officer and Above Production/ IPQA Officer	:	Implementation Of Procedure.

Operation and cleaning                                     :         Technician

5.0	Accountability

Production Head.

6.0      Procedure

6.1	operation

    - Visually inspect the cleaning status of printer & surrounding area.

6.1.3    Switch on power supply of computer & edit relevant password.

6.1.4    Click the folder OBJ INKDRAW & fix the flash card in card reader.

6.15 Click file &edit message as per product specifications.

6.1.6 Set the parameter of message for proper printing of message on window.

6.1.7 Select TCU option, open & save the message.

6.1.8 Click KE option & select TCU-IT-TCU and save it.

6.1.9 Close the file & return to desktop menu.

6.1.10 Double click on my computer, select removable Disc 1

6.1.11 Click right button of mouse & click left button on load the message in flash card and remove it.

6.1.12 Insert flash card in TCU machine (HSAJET)

6.1.13 Press BLACK KEY followed by RED KEY.

6.1.14 Load label roll on dancing roll.

6.1.15 Adjust coding on label window with help of sensor.

6.1.16 Press RUN KEY followed by START KEY on MMI.

6.1.17 Take coded label, affix on BPCR & inform IPQA person for line clearance.

6.1.18 After verification of overprinted text matter on label, by IPQA and production person, start operation.

6.2 Cleaning

6.2.1 Switch off the power supply of machine

6.2.2 Remove the flash card.

6.2.3 Clean the external surface with lint free dry duster.

6.2.4   For cleaning of cartridge remove it from hosing.

6.2.5 Clean its nozzle with tissue paper with soft hand

6.2.6 Again fix the cartridge in hosing.

7.0    Annexure (S)

NA

8.0   Reference (S)

NA

9.0   Glossary

SOP	 	: Standard Operating Procedure.

NO.   	: Number

PC	            : Production (B-Block)

IPQA	: in Process Quality Assurance