| Product Name | : | Nodimox Plus 500 Capsules |  |  |
| --- | --- | --- | --- | --- |
| Generic Name | : | Amoxicillin & Cloxacillin Capsules IP |  |  |
| Label Claim | : | Each capsule contains: Amoxicillin Trihydrate IP equivalent to Amoxicillin …………250 mg Cloxacillin Sodium IP equivalent to Cloxacillin…………….250 mg |  |  |
| Product Code: | : | FC5451SF |  |  |
| Storage Condition | : | Store in a cool dry place, Protected from light. |  |  |
| Product Appearance | : |  |  |  |
| Mfg. Lic. No | : | MB/11/846 |  |  |
| Reference MPR No | : | MFA/226/07-01 | Supersedes No: | NIL |
| BMR No: | : | BMR/FC5451SF-00 | Supersedes No: | NIL |
| Effective Date | : |  |  |  |
| Standard Shelf Life | : | 24 Months | Actual Shelf Life |  |
| Standard Batch Size | : | 4,00,000 Capsules | Actual Batch Size |  |

| Batch No. | : |  |
| --- | --- | --- |
| Mfg. Date | : |  |
| Exp. Date | : |  |
| Process Order No. | : |  |
| Issued By (QA) Sign/Date | : |  |
| Received By(Production) Sign/Date | : |  |
| Batch Commenced On | : |  |
| Batch Completed On | : |  |
| % Batch Yield | : |  |

|  | Prepared By | Checked By | Approved By | Authorized By |
| --- | --- | --- | --- | --- |
|  | Production | Quality Assurance | Unit - Head | Head – Quality |
| Name |  |  |  |  |
| Sign |  |  |  |  |
| Date |  |  |  |  |

INDEX

| Sr. no. | PAGE No. | DESCRIPTION |
| --- | --- | --- |
| 1 | 1 – 1 | BMR Information |
| 2 | 2 – 2 | Index |
| 3 | 3 – 3 | General instructions |
| 4 | 4 – 7 | Dispensing of raw materials |
| 5 | 8– 14 | Granulation Stage |
| 6 | 15 – 28 | Filling and Polishing Stage |
| 7 | 29 - 29 | Weight record of filled capsules |
| 8 | 30 - 30 | Reconciliation |
| 9 | 31 - 31 | Batch Manufacturing Summary |
| 10 | 32 – 32 | Document Checklist and Approval |
| 11 | 33 - 33 | Batch Release for Packing |

General Instructions

- Before starting the batch, check dispensing and manufacturing area and equipments for cleanliness as per the SOP No. BP/096.

- All personnel present in the production area must use required gowning and gloves.

- Temperature of the dispensing area should not be more than 24C and relative humidity should not be more than 45%.

- Temperature of the processing area should not be more than 24C and relative humidity should not be more than 45%.

- Preserve all dispensing labels/tags in poly bag or label docket and attach with BMR.

- Store the Raw materials, In-process materials, semi-finished product in the designated area with proper label and mention the status of the material or batch detail.

- Equipments must be with proper label indicating the status and date.

- In-process control must be strictly ensured and the data must be recorded at regular intervals in the Batch Manufacturing Record.

- All entries have to be made in legible black ink and all signatures should have their corresponding dates.

Stage 1.0 Dispensing of Raw Materials

- Check the environmental conditions of dispensing area before start, during and end of dispensing & record in Table No.1.1 A and Table No. 1.1 B as per SOP no. BP/003.

- Check the Approved status of all the raw material containers before transferring into Dispensing area.

- Check the cleaning status of equipment and area and record in Table No.1.2.1 and 1.2.2.

- Before start dispensing, check the calibration of weighing balances.

- Take raw Excipient to be dispensed under RLAF of dispensing booth – I in double polyethylene bags with proper label.

- Take raw Active to be dispensed under RLAF of dispensing booth – II in double polyethylene bags with proper label.

- Record the dispensing operation detail in Table No.1.3 and Table No. 1.4 accordingly.

- Load the dispensed raw material in Pallate and transfer it into Dispensed Material Quarantine.

1.1 Environmental Monitoring Record

- For Excipient

Table No. 1.1 A

| Date | Time | Differential Pressure (1.0 – 1.5 mm of WC) | Pressure Differential Across RLAF HEPA (Limit…………) | Temp. (ºC) | % RH | Checked by/Date (WH) | Verified by/Date (IPQA) |
| --- | --- | --- | --- | --- | --- | --- | --- |

- For Active

Table No. 1.1 B

| Date | Time | Differential Pressure (1.0 – 1.5 mm of WC) | Pressure Differential Across RLAF HEPA (Limit…………) | Temp. (ºC) | % RH | Checked by/Date (WH) | Verified by/Date (IPQA) |
| --- | --- | --- | --- | --- | --- | --- | --- |

1.2 Equipment & Area Clearance

Table No. 1.2.1

| Previous Product: |  |  |  | Batch No.: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sr. No. | Equipment/ Area Name | Equipment ID./Room No. | Cleaned as Sop No. |  | Qualified | Checked By/Date (WH) | Verified by/Date (IPQA) |
| Dispensing Exicipient-Raw Materials |  |  |  |  |  |  |  |
| 1. | Dispensing area |  | BP/096 |  | Yes / No |  |  |
| 2. | Weighing Balance Capacity………… |  | BP/026 |  | Yes / No |  |  |
|  | Weighing Balance Capacity………… |  |  |  | Yes / No |  |  |

Table No. 1.2.2

| Previous Product: |  |  |  | Batch No.: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dispensing Active-Raw Materials |  |  |  |  |  |  |  |
| 1. | Dispensing area |  | BP/096 |  | Yes / No |  |  |
| 2. | Weighing Balance Capacity………… |  | BP/026 |  | Yes / No |  |  |
|  | Weighing Balance Capacity………… |  |  |  | Yes / No |  |  |

iiring and end of dispensing 25rom light.CALCULATION SHEET

Quantity of Amoxicillin Trihydrate (Compacted) (in Kg) for 8,00,000 capsules 200 x100 = ________________________________________________________________ % Assay of Amoxicillin Trihydrate (Compacted) on ‘as is ’ basis as Amoxicillin =________________ Kgs.

Quantity of Cloxacillin Sodium (in Kg) for 8,00,000 capsules 200 X 100 =____________________________________________________ % Assay of Cloxacillin Sodium on ‘as is ’ basis as Dicloxacillin =________________ Kgs.

| ##### Calculation Done By/Date (Production) | ##### Verified By/Date (IPQA) |
| --- | --- |

##### 1.3 Dispensing Operation Details

Table No. 1.3

| Raw Material | Dispensing Started At |  | Dispensing Completed At |  | Dispensed By (Sign & Date) | Checked By (Sign & Date) | Verified By (QA) (Sign & Date) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date | Time | Date | Time |  |  |  |
| Excipient |  |  |  |  |  |  |  |
| Active |  |  |  |  |  |  |  |

Bill of Raw Materials    (For 8.0 Lac Cap)

Table No. 1.4

| Sr. No | Item Code | Ingredients | Std. Wt (kg) | Qty to be dispensed (kg) | A.R. No./No. of container | Weight (Kg) |  |  | Weighed By Date | Ckd By/Date(Prod) | Verified By/Date(IPQA) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Gross | Tare | Net |  |  |  |
| 1. | RA0009 | Amoxycillin Trihydrate IP (Compacted) | 232.558 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Total Weight |  |  |  |  |  |  |
| 2. | RD0016 | Cloxacillin Sodium IP (Compacted) | 218.818 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Total Weight |  |  |  |  |  |  |

| 3. | RL0002 | Lactose IP |  | 16.002 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. | RM0003 | Magnesium Stearate IP |  | 4.800 |  |  |  |  |  |  |  |  |
| 5. | RA0003 | Aerosil –200 (Colloidal Silicon Dioxide) IP |  | 2.400 |  |  |  |  |  |  |  |  |
| 6. | RT0001 | Talc IP |  | 5.422 |  |  |  |  |  |  |  |  |
| Total weight |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. | R50005 |  | E.H.G. Capsules Grey/white Size “0” IP | 808,000 Nos |  |  |  |  |  |  |  |  |

Standard quantity 232.588 Kg is calculated based on 86.00 % assay of Amoxicillin Trihydrate (compacted) on ‘as is’   basis as Amoxicillin.

Standard quantity 218.818 Kg is calculated based on 91.40 % assay of Cloxacillin Sodium (compacted) on ‘as is’ basis as Cloxacillin.

Total qty of Amoxicilin Trihydrate (Compacted), Cloxacillin Sodium (Compacted) and Lactose should be equal to 467.378. Any decrease or increase in the of Amoxicillin Trihydrate (compacted) and Cloxacillin Sodium (Compacted) Qty. should be adjusted with Lactose.

| ### Material Transferred By (Warehouse) (Sign & Date) | ### Material Received By (Production) ### (Sign & Date) |
| --- | --- |

Stage 2.0 Granulation stage

Raw Material Verification

Sifting

Mixing and Blending

Yield reconciliation

Instructions for Data recording

- Check the cleaning status of equipment and area and record in Table No.2.1. and inform QA for area clearance.

- Prior to start the granulations verify the weight of dispensed raw materials in table no. 2.2.

- Check the environmental conditions of Granulation area & record in Table No.2.3

- Before start the operation check the calibration of weighing balances and equipments.

- Record the granulation operation detail in Table No.2.3 and Table No. 2.4 accordingly.

2.1 Equipment & Area Clearance

| Previous Product: | Batch No.: |
| --- | --- |

Table No. 2.1

| Sr. No. | Equipment / Area Name | Equipment / Area. No. | Cleaning Type A/B | Operation and cleaning as per SOP number | Cleaned by Sign & Date |
| --- | --- | --- | --- | --- | --- |
| 1. | Granulation area | BAB/039 BAB/040 |  | BP/096 |  |
| 2. | Sifter | BPD/027 BPD/035 |  | BP/029 |  |
| 3. | RCVD | BPD/025 BPD/029 |  | BP/027 |  |
| 4. | Balance Capacity…………… |  |  | BP/026 |  |
| 5. | Balance Capacity…………… |  |  | BP/026 |  |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

2.2	Verification of Materials

- Before using the material, production officer in presence of QA shall compare & checked the product details, item code, material description, batch number and dispensed quantities, expiry date of active materials  as specified in Process Order with signed in material issued slips & raw material verification sheet. , QA officer shall verify the details and signed in good issue request.

Table No. 2.2

| Sr. No | Item Code | Ingredients | Std. Wt for 8.0 Lac Cap(kg) | A.R. No./No. of container | Gross Weight (kg)/Nos | Weighed By Sign/Date | Checked by Sign/ Date | Verified by Sign/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | RA0009 | Amoxycillin Trihydrate IP (Compacted) | 232.558 |  |  |  |  |  |
| 2. | RD0016 | Cloxacillin Sodium IP (Compacted) | 218.818 |  |  |  |  |  |
| 3. | RL0002 | Lactose | 16.002 |  |  |  |  |  |
| 3. | RM0003 | Magnesium Stearate IP | 4.800 |  |  |  |  |  |
| 4. | RA0003 | Aerosil –200 (Colloidal Silicon Dioxide) IP | 2.400 |  |  |  |  |  |
| 6. | RT0001 | Talc IP | 5.422 |  |  |  |  |  |
| 5. | R50005 | E.H.G. CAPSULES BLUE/BLUE SIZE “0” (BDCM) IP | 808,000 |  |  |  |  |  |

2.3	 Environmental Monitoring Record (Frequency: Start/Every 2 hours/End)

Table No. 2.3

| Date | Time | Temperature ( NMT 24C) | Relative Humidity (NMT 45%) | Differential Pressure ( 1.0 – 1.5 mm of WC ) | Done By/ Date | Checked By/ Date |
| --- | --- | --- | --- | --- | --- | --- |

2.4 Sifting

- Check the sieve ID No., integrity of the sieve before and after sifting of materials and record in table no. 2.4.1 and table no. 2.4.2 accordingly.

- Start the sifting of the materials using Mechanical Vibratory Sifter as per SOP No. BP/029.

Sifting Details

Table no.: 2.4.1

| Date | Started On | Completed On | Done By | Checked By |
| --- | --- | --- | --- | --- |

Table no.:2.4.2

| Ingredients | Qty. Kg | Mesh size & No. |  | Sieve Integrity Check Before | Checked By Prod. | Verified By IPQA | Sieve Integrity Check After | Checked By Prod. | Verified By IPQA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ampicillin Trihydrate (Compacted) |  | # 20 |  |  |  |  |  |  |  |
| Colloidal Silicon Dioxide |  | # 40 |  |  |  |  |  |  |  |
| Talc |  | # 40 |  |  |  |  |  |  |  |
| Magnesium Stearate |  | # 40 |  |  |  |  |  |  |  |

Note: Sift Magnesium stearate separately and collect it into a double poly bag lined S.S container and keep aside.

Check for the residue if any from each material for appearance and quantity and record in remarks.

Note: Always store the in-process material, semi-finished product in the labeled air tight S.S. in process containers.

Remarks:______________________________________________________________________________________________________________________________________________________

  - Mixing

2.5.1	Load the sifted materials of Ampicillin Trihydrate, Colloidal Silicon Dioxide and Talc into 			the RCVD as per SOP No. BP/027.

2.5.2	Mix the materials for 15 Minutes  at 8 RPM and record the detailed in the following table

Loading Details:                            							Date

Table No. 2.5.1

| Material | Qty. (In Kgs) | Time |  | Loaded By | Checked By (Prod) |
| --- | --- | --- | --- | --- | --- |
|  |  | From | To |  |  |
| Ampicillin Trihydrate (Compacted) |  |  |  |  |  |
| Colloidal Silicon Dioxide |  |  |  |  |  |
| Talc |  |  |  |  |  |

Mixing Details										Date

Table No. 2.5.2

| Operation | Time |  | Done By | Checked By (Prod) |
| --- | --- | --- | --- | --- |
|  | From | To |  |  |
| Mixing Time: 15Mins. Before Addition of Magnesium Stearate* |  |  |  |  |

  - Blending

    - Load the sifted materials of Magnesium Stearate into the RCVD as per SOP No. BP/027.

    - Blend the material for 10 Minutes at 8 RPM and record the detailed in the following table

Loading Detail of Magnesium Stearate      					Date

Table No. 2.6.1

| Material | Qty. (In Kgs) | Time |  | Loaded By | Checked By (Prod) |
| --- | --- | --- | --- | --- | --- |
|  |  | From | To |  |  |
| Magnesium Stearate |  |  |  |  |  |

Blending Details									Date

Table No. 2.6.2

| Operation | Time |  | Done By | Checked By (Prod) |
| --- | --- | --- | --- | --- |
|  | From | To |  |  |
| Blending Time: 10 Mins. After addition of Magnesium Stearate. |  |  |  |  |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

Intimate the Quality Assurance Department to withdraw the sample of blend as per SOP No. QA/006 for analysis, as per current specification.

  - Q.A.  Sampling details

Table No. 2.7

| Quantity Withdrawn | Date of Sampling | Time of Sampling | Sampled By IPQA / Date |
| --- | --- | --- | --- |

Send rejections for destruction as per SOP No. QA/007 and record the quantity.

After completion of blending, unload the blend into labeled S.S. in process containers & record the weight details.

  - Non-Recoverable Losses Destruction Record

Table No. 2.8

| Qty of non recoverable | Destroyed by | Checked by | Verified by Q.A |
| --- | --- | --- | --- |

2.9 Weight Record of Lubricated Granules                                    Date______________

Balance ID No._____________

Table No. 2.9

| IPC / Container No. | IPC/Container Cleanliness Checked By | Weight in kg |  |  | Done By |
| --- | --- | --- | --- | --- | --- |
|  |  | Gross Weight | Tare Weight | Net Weight |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| Total Weight |  |  |  |  |  |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

  - Yield and Reconciliation

Table No. 2.9

| Particulars |  | Actual | UOM |
| --- | --- | --- | --- |
|  | Theoretical Batch Size |  | Kg |
|  | Actual Quantity of Blend Produced |  | Kg |
|  | Rejection Non Recoverable |  | Kg |
|  | In-process QA Sample |  | Kg |
|  | Other samples: 1 |  | Kg |
|  | Other samples: 2 |  | Kg |
|  | Other samples: 3 |  | Kg |
|  | Other samples: 4 |  | Kg |
|  | % Batch Yield = (2+4+5) x100 Theoretical Batch Size |  | % |
|  | Actual Yield:2X100/ Theoretical Batch Size |  | % |
|  | Total accountability (2+3+4+5) = (A) |  | Kg |
|  | % Reconciliation = (A) x 100 Theoretical batch size |  | % |

UOM = Unit of Measurement

Limits: Standard Batch Yield: NLT 99.00%

Reconciliation: 100 % ± 0.5%

If reconciliation is less or more than the standard record the reason in the following space.

Reason:___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Yield checked by (Prod.)/Date: _____________ Yield certified by (IPQA.)Date: ______________

2.11 Blend Release

Analytical Reference No.: _______________________

Approved / Rejected (Enclose blend analysis report)_________________

IPQA (Sign / Date): _________________

Hold Time Period

Blend may be stored for not more than 30 days from the date of blending at temperature NMT 24C and relative humidity NMT 45%. If the date exceeds the specified time, then report to Quality Assurance for further action.

Date of Blending (A) ____________________________

Date of compression should not be later than___________________________ (A+30 Days)

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

Stage 3.0 Filling and Polishing stage

Instructions for Data recording

  - Check the cleaning status of equipment and area and record the Details.

- Prior to start the activity verifies the weight of blend material record in 3.2.1.

- Check the environmental conditions of Processing area & record in Table No.3.3.1

- Before start the operation check the calibration of weighing balances.

3.1 Equipment & Area Clearance		Date

| Previous Product: | Batch No.: |
| --- | --- |

Table No. 3.1

| Sr. No. | Equipment Name | Room/ Equipment No. | Operation & Cleaning SOP No. | Qualified | Done By |
| --- | --- | --- | --- | --- | --- |
| 1 | Area Clearance |  | BP/096 | Yes / No |  |
| 2 | Weighing Balance |  | BP/026 | Yes / No |  |
| 3 | Weighing Balance |  | BP/026 | Yes / No |  |
| 4 | Capsule Filing M/C. (AF – 150T) |  | BP/034 | Yes / No |  |
| 5 | Dedusting & Polishing M/C |  | BP/034 | Yes / No |  |
| 6 | Online check weigher |  | BP/034 | Yes / No |  |
| 7 | Mini Capsule Sorter |  | BP/034 | Yes / No |  |
| 8 | Empty Capsule Sorter & elevator |  | BP/034 | Yes / No |  |
| 9 | Metal Detector |  | BP/034 | Yes / No |  |
| 10 | Vernier caliper |  | QA/018 | Yes / No |  |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

3.2 Process Equipment / Instrument (IPQA)

Table No. 3.2

| Sr. No. | Name of In Process Equipment / Instrument (IPQA) | Identification No. |
| --- | --- | --- |
| 1 | Electronic Balance Capacity_________ |  |
| 2 | Vernier Calipers |  |
| 3 | Disintegration Test Apparatus |  |

3.3 Verification of Blend

Table – 3.3

| Container No. | Quantity (Kgs) | Weighed By |
| --- | --- | --- |
|  | Gross |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

3.4 Environmental Monitoring Record (Frequency: Start/Every 2 hours/End)

Table – 3.4

| Date | Time | Temperature ( NMT 24C) | Relative Humidity (NMT 45%) | Differential Pressure ( 1.0 – 1.5 mm of WC ) | Done By/ Date | Checked By/ Date |
| --- | --- | --- | --- | --- | --- | --- |

3.5 Machine Setting

Set the AF – 150 and line machines as per SOP No. BP/034

Transfer the BLEND from quarantine area to capsule filling room (BAB/044).

3.5.1	Empty Hard Gelatin Capsule Details

Table – 3.5.1

| Particulars | Specifications | Done By Sign/Date |
| --- | --- | --- |
| Capsule size | “0” |  |
| Capsule Colour | Cap: Blue Body: Blue |  |
| Printing on capsule | BROADICILIN and ALKEM on body Alkem logo and 500 on cap |  |

3.5.2	Setting Parameters for Capsule Filling

Adjust the filling parameter as per given below table.

Table – 3.5.2

| To Be Set Parameters |  | Observation | Done By Sign/Date |
| --- | --- | --- | --- |
| Theoretical wt. of empty capsule(Avg. wt. of 20 caps.) | 96 mg |  |  |
| Individual wt. per capsule (Filled) | 702.5mg 5.0 % [Limit: 667.375 mg – 737.625 mg] |  |  |
| Wt. Of 20 filled capsules | 20 X [606.5mg+96.0mg]= 14.050gm Limit: ±2% [13.769 gm – 14.331 gm] |  |  |
| Capsule Lock Length | 21.3 – 21.9 mm |  |  |
| Machine Speed | ___________ Strokes per minute |  |  |
| Disintegration time | NMT 15 minutes |  |  |

    - Empty Hard Gelatin Capsule A. R. No. Details

Check A.R. No. of the empty capsules and record in the following table. Weigh 20 empty capsules at the start of batch and for every change in A.R No. of empty capsules. Calculate the average weight of 20 empty capsules, so as to determine the average weight of 20 filled capsules. Record this observation in the same table.

Table – 3.5.3

| A. R. No. | Start Date/Time | Average Weight of 20 Capsules |  | Done By Sign/Date |
| --- | --- | --- | --- | --- |
|  |  | Empty Capsule | Filled Capsules |  |

Note: Whenever A.R. No. of the EHG capsule is changed, perform the In-process checks again.

    - Challenge tests

- Metal Detector Initial Checks and after machine break down

Table – 3.5.4 A

| Date/ time | Pre Check (Pass / Fail) |  |  | Done By/Date (Prod.) | Remarks (Test Passes/ Fails) |
| --- | --- | --- | --- | --- | --- |
|  | Ferrous | Nonferrous | Stainless Steel |  |  |

(B)ECS Initial Checks and after machine break down

Pass 100 filled plus 10 empty capsules and set the voltage at which all the empty capsules get rejected.                                       Table – 3.5.4 B

| Date/ time | Voltage at all the empty capsule get rejected | Done By/Date (Prod.) | Remarks (Test Passes/ Fails) |
| --- | --- | --- | --- |

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

3.5.5 Initial Checks to be performed:

Date: _____________                                                                              Time: __________

Table – 3.5.5

| Parameters | Standard Limit | Observations |  |  |  |  |  | Done By Sign/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capsule size & Color | Size: ‘0’ Cap / Body : Blue /Blue colored |  |  |  |  |  |  |  |
| Printing on capsule | BROADICILIN and ALKEM on body Alkem logo and 500 on cap |  |  |  |  |  |  |  |
| Gross Wt. Of 20 filled capsules | 14.050 gm±2% Limit:13.769gm –14.331 gm |  |  |  |  |  |  |  |
| Individual wt. of18 filled capsule (mg) | 606.5mg+96mg=702.5mg±5% (667.375-737.625) |  |  |  |  |  |  |  |

| Average Wt. of 18 filled capsules | 702.5mg ± 2% = 688.45mg to 716.55mg |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Net content wt. of 18 capsule (mg) | 606.5 ± 5 % = 576.175 mg to 636.825 mg |  |  |  |  |  |  |  |
| Locked length of 18 filled capsules (mm) | 21.3 mm to 21.9 mm |  |  |  |  |  |  |  |
| Disintegration Time | NMT 15 minutes |  |  |  |  |  |  |  |
| Physical Appearance of 100 capsules (Put OK /NOT OK for observation) |  |  |  |  |  |  |  |  |

Appearance: Denting, Telescopic, V-notch, Brittle.

    - Record of Weight of Individual Capsules during Initial Checks and after machine break down

Table – 3.5.6

|  | Weight (mg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Individual Weight of filled capsules | Weight of empty capsule | Net content weight per capsule | Individual Weight of filled capsules | Weight of empty capsule | Net content weight per capsule |
| Date |  |  |  |  |  |  |
| Time |  |  |  |  |  |  |
| No. |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| Avg. Wt. |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |
| Done by |  |  |  |  |  |  |
| Checked by |  |  |  |  |  |  |

    - In Process Control Parameters during Capsule Filling

Table – 3.5.7

| Sr. No. | Parameters | Limit/Description | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Production | IPQA |  |
| 1 | Description | White or off white colored granular powder filled in size “0” blue colored cap & blue coloured body printed. BROADICILIN and ALKEM on body Alkem logo and 500 on cap | Start/Every 1 hour/End | Every 2 hour |  |
| 2 | Group wt. of 20 filled capsules | 14.050 gm ± 2 % 13.769 gm to 14.331 gm | Start/Every half an hour/End | Every 2 hours |  |
| 3 | Avg. wt. of 20 filled capsules | 702.5mg ± 2% = 688.45mg to 716.55mg |  |  |  |
| 3 | Individual wt. of 20 filled capsule | 702.5 mg ± 5 % 667.375 mg to 737.625 mg | Start/Every 1 hours/End | Every 2 hours |  |
| 4 | Disintegration time of 6 filled capsule | NMT 15 min. | Start/Every 2 hours/End | Every2 hours |  |
| 5 | Locked length of 18 filled capsules | 21.3 mm to 21.9 mm | Start/Every 2 hours/End | Every2 hours |  |
| 6 | Machine speed (SPM) |  | Start/Every 2 hours/End |  | NA |

3.5.8. A    In-process Checks for Capsule filling (to be filled separately for each shift if machine stopped during shift changed)

Table – 3.5.8 A

| Date/Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Limit: 716.55 mg | 717.5 712.5 707.5 702.5 697.5 692.5 687.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Target Av. Weight = 702.5mg Of 20 units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scale: 1division =0.5 mg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower Limit: 688.45 mg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group wt. of 20 filled capsules (g) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avg. wt. of 20 filled capsules (mg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Machine Speed (SPM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Done by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3.5.8. B   In-process Checks for Capsule filling (to be filled separately for each shift if machine stopped during shift changed)

Table No. – 3.5.8 B

| Date / Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper limit 716.55 mg | 717.5 712.5 707.5 702.5 697.5 692.5 687.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Target Av. Weight = 702.5mg Of 20 units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scale: 1division =0.5 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower Limit: 688.45 mg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group wt. of 20 filled capsules (g) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avg. wt. of 20 filled capsules (mg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Machine Speed (SPM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Done by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

    - C In-process Checks for Capsule Filling (to be filled separately for each shift if machine stopped during shift changed)

Table No. – 3.5.8 C

| Date/Time Date: Time: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Limit: 716.55 mg | 717.5 712.5 707.5 702.5 697.5 692.5 687.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Target Av. Weight = 702.5mg Of 20 units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scale: 1division =0.5 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lower Limit: 688.45 mg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group wt. of 20 filled capsules (g) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avg. wt. of 20 filled capsules (mg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disintegration Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Machine Speed (SPM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Done by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3.5.9 A Record of Individual Weight of 20 Capsules (mg)                                                                                                                       Table No. – 3.5.9 A

| Limit ► |  |  |  |  |  | Minimum weight =667.375mg |  |  |  |  | Average weight = 702.5mg |  |  |  |  |  | Maximum weight = 737.625mg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capsule No. 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avg. wt. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Done By |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked By |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3.5.9 B Record of Locking Length of 18 Capsules (mm)                                                                                                                           Table No. – 3.5.9 B

| Limit |  | Minimum length (mm) =21.3 |  |  |  |  |  | Average length (mm) = 21.6 |  |  |  |  | Maximum length(mm)=21.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Capsule No. 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Appearance# |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D.T. tTime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Done by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checked by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# White or off white coloured granular powder filled in size “0” blue colored cap and body

Tick mark ‘√’ if OK, ‘X’ if Not OK Appearance- Denting, Telescopic, V-notch, Brittle.

  - Post Filling Standard Sample Check Report of the Metal Detector

Table No. – 3.6

| Date | Post Check (Pass / Fail) |  |  | Done By Sign/Date | Checked By Sign/Date | Remarks ( Passes /Fails) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ferrous | Nonferrous | Stainless Steel |  |  |  |

  - Filling Output Details

Table No. – 3.7

| Date | M/C Start Time | M/C Stop Time | Output (Nos) | Output Cumulative (Nos) | Done By | Checked By |
| --- | --- | --- | --- | --- | --- | --- |

3.8 QA Sampling of Finished Product:

Intimate Quality Assurance Department to draw the sample of Filled capsules as per SOP No. QA/006 for analysis as per current specification.

Table No. – 3.8

| Quantity Withdrawn | Date of Sampling | Time of Sampling | Sampled By IPQA / Date |
| --- | --- | --- | --- |

3.9	Weight Record of Filled Capsules

Table No. 3.9

| Cont. No. | Container Cleanliness Checked By | Weight in Kgs. |  |  |  | Done By Sign/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Gross Wt. | Tare Wt. |  | Net Wt. |  |
| Total Wt. |  |  |  |  |  |  |

Transfer the filled capsules to quarantine for storage and enter details in quarantine log.

| Checked by (Prod. Sign/date) | Verified By (Q.A. Sign/date) |
| --- | --- |

3.11 RECONCILIATION

3.11.1   Reconciliation of Empty Hard Gelatin capsules

Table No. 3.11.1

| Particulars |  | Nos. |
| --- | --- | --- |
| 1. Quantity Issued for the Batch |  |  |
| 2. Additional Quantity Issued |  |  |
| 3. Total Quantity Issued (1+2) |  |  |
| 4. Quantity Consumed in Filling |  |  |
| 5. Quantity of on line Rejection Destroyed (Av. Wt. per Capsule ________mg) |  |  |
| 6. Left over EHG capsule |  |  |
| 7. Capsules Returned to Stores MRN No._____________ Date |  |  |
| 8. Total Accountability (4+5+6+7) |  |  |
| 9. Variance (3-8/3 X 100%) limit: ± 0.2% |  |  |
| Checked By/date: | Verified By/date: |  |

3.11.2   Reconciliation of filled capsules:

Table No. 3.11.2

| Sr. No. | Particulars | Actual | UOM |
| --- | --- | --- | --- |
| 1 | Theoretical Batch Size | 242.60 | Kg |
| 2 | Theoretical Filled Capsule Wt. | 702.5 | mg |
| 3 | Avg. wt. of filled capsules |  | mg |
| 4 | Theoretical Batch Size [(1) / Net Fill Weight 606.5 mg) | 400000 | Units |
| 5 | Quantity of Blend Issued For Filling [_____ kgx1000x1000]/net fill wt 606.5 mg) |  | Units |
| 6 | Quantity After Filling |  | Kg |
| 7 | No. of Units Produced [(6)/(3)*1000*1000] |  | Units |
| 8 | Hopper left over (_______ kg) |  | Units |
| 9 | Non Recoverable Rejection During Operation |  | Units |
| 10 | In-process Samples |  | Units |
| 11 | QA Samples for Validation |  | Units |
| 12 | Other Samples: 1 |  | Units |
| 13 | Other Samples: 2 |  | Units |
| 14 | Other Samples: 3 |  | Units |
| 15 | Total accountability:( 7+8+9+10+11+12+13+14) |  | Units |
| 16 | Batch yield (7+11+12+13+14)x100/4 |  | % |
| 17 | % Reconciliation: 100 x (15) / (5) |  | % |

UOM = Unit of Measurement

Limits:

Batch Yield:___________________                    % Reconciliation: 100 ± 0.5%

If reconciliation is less or more than the standard record the reason below.

Reason:_______________________________________________________________________________

______________________________________________________________________________________

Yield checked by (Prod.)/Date: ______________ Yield certified by (IPQA.)/Date: _____________

Stage 5.0: Batch Manufacturing Summary

Comments on Raw Materials:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

In-process Checks & Results (any deviations):

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Process Deviations (if any):

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Change Control (if any):

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| Production Officer Sign & Date | Production Manager Sign & Date | Reviewed by QA: Sign & Date |
| --- | --- | --- |

Stage 14: Document Check List and Approval

| Page no. | Operation | Tick mark |
| --- | --- | --- |
| 1 – 1 | BMR Information |  |
| 2 – 2 | Index |  |
| 3 – 3 | General instructions |  |
| 4 – 7 | Dispensing of raw materials |  |
| 8– 14 | Granulation Stage |  |
| 15 – 30 | Filling and Polishing Stage |  |
| 31 – 31 | Batch Manufacturing Summary |  |
| 32 - 32 | Document Check List and Approval |  |
| Attachments |  |  |
| -- | Deviation reports (if any) |  |
| -- | Material return note (if any) |  |
| -- | Certificate of analysis for finished product |  |

Note : Put tick mark ‘’  in respective brackets after completion of stage. Put ‘X’ wherever not applicable.

Batch Released

QA Manager:

Date: