- Objective

To lay down the Procedure for storage of material in blend and bulk store.

- Scope

This SOP is applicable for storage of material in blend and bulk store in Area of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP.

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure:

  - Storage of dispensed raw materials

  - Check that the environmental conditions are within limits. (Temp NMT 25°C and RH NMT45 )

  - Bring the Dispensed raw material by a clean SS pallet to the material air lock. Weigh it and record the weight.

  - Put the weighed Dispensed raw material back on the SS pallet.

  - Keep different products or different batches of same products separately in different SS pallet and wrap it by shrink film.

- Storage of tablets/capsule/granules in blend store.

    - Check that the Environmental conditions shall be within limits. (Temp: NMT 25 0C and RH: NMT 45%)

    - Transfer the container containing tablets/capsule/granules to blend store.

    - An “UNDER TEST” Label shall be affixed by IPQA Officer.

    - After getting the release status from Quality Control Dept. affix the “APPROVED” Label over the “UNDER TEST” Label by IPQA person.

    - Record the environmental condition every four hour in the Environmental Control Record.

    - Keep different products or different batches of same products separately on SS pallets.

    - Check the Product name, Batch no, Qty., Stage, Total no. of container/IPC transferred, issued and received and mention it in Annexure-1

    - If the product shall be stored for a longer time, then after one month it shall be reanalyzed by Quality Control.

- Storage of tablets and capsules in bulk store

    - Check that the Environmental conditions shall be within limits. (Temp: NMT 25 0C and RH: NMT 45%)

    - Transfer the container containing tablets and capsules to bulk store.

    - An “UNDER TEST” Label shall be affixed by IPQA Officer.

    - After getting the release status from Quality Control Dept. affix the “APPROVED” Label over the “UNDER TEST” Label by IPQA person.

    - Record the environmental condition every four hour in the Environmental Control Record.

    - Keep different products or different batches of same products separately on SS pallets.

    - If the product shall be stored for a longer time, then after one month it shall be reanalyzed by Quality Control.

- Annexure (S)

Annexure-1 Storage of material in blend and bulk store.

- Reference (S)

NA

- Glossary

SOP	:	Standard Operating Procedure

No.	:	Number

S.S	:	Stainless Steel

WH	:	Ware house

IPQA    	:      	In process quality assurance

BPR   	:       	Batch Packing Record

IPC     	:       	In process quality control

TEMP  	 :          Temperature

RH      	:      	Relative humidity

NMT   	:           Not more than

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |