- Objective

To lay down the cleaning procedure of Utensils used in oral manufacturing area.

- Scope

The Procedure is applicable for cleaning of Utensil (Scoops, Long Spatula, Big Spoons, S.S Buckets, and S.S Containers) used in manufacturing area of Alkem laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Clean the Utensil after batch changeover/ product changeover.

  - Put the Utensil in a polybag, affix the polybag with "TO BE CLEANED" label.

  - Bring this polybag to the equipment wash area.

  - Take out the Utensil and wash it thoroughly with potable water and a nylon scrubber. Finally, rinse with purified water.

  - Keep the cleaned Utensil under IR drier.

  - After drying, store the Utensil, cover with polythene bag with "CLEANED" label duly signed by Production Officer/Executive.

  - If the cleaned utensil remains unused for 72 hours, it will be cleaned again before use.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary

SOP		:	Standard Operating Procedure

No.		:	Number

IR		:	Infrared dryer

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |