- Objective

To lay down the procedure for handling of scrap.

- Scope

The SOP is applicable to lay down the procedure for handling of scrap in Alkem Laboratories Limited, Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Scrap transfer / disposal from core area solid dosage forms

    - Scrap generated in core area are uncoated, coated tablets, rejected filled capsules, rejected empty hard gelatin capsules, non-recoverable powders, rejected bottles, rejected ROPP caps, rejected measuring cups, empty blisters / strips, de-foiled blisters / strips, rejected foils / PVC films, polythene bags, cable tie, papers, plastic bottles of ink and thinner etc.

    - Collect the leftover powder, tablets and capsules in a poly bag and transfer to respective washing area, decontaminate with 2.0% NAOH and transfer to ETP through drain.

    - Collect and inspect rejected empty or de-foiled blisters / strips in black coloured poly bag from primary packing, verify that no tablets / capsules presence in it. Transfer this scrap to scrap room.

    - Collect and segregate rejected glass bottles from wash bins of filling / sealing room of dry syrup line in a poly bag, label and transfer to the respective scrap room after verification.

    - Collect and segregate rejected HDPE bottles, seals, ROPP caps, inner plugs, measuring cups and droppers etc in a poly bag and transfer them to scrap room after verification.

    - The scrap shall be weighed before transfer and quantity shall be entered in scrap transfer and verification format.

    - Before transfer the scrap shall be checked, labelled and verified by production and QA personnel respectively.

    - Transferred the scrap in between the lock facilities passage .Once this passage is filled with scrap, the inside house keeping personal lock 1st door and the outside house keeping personal open the outside (2nd) door and transferred the scrap to central  scrap yard.

  - Scrap transfer / disposal from injectable area

    - In injectable area scrap generated are leftover powders, rejected vials, rubber plugs, aluminium seals, polythene bags and aluminium containers.

    - Powder generated in aseptic area shall be collected in stainless steel containers, vials, rubber plugs and aluminium seals shall also be collected in separate stainless steel containers and to be taken out side through dynamic pass box.

    - In washing area scrap generated are vials, rubber plugs and aluminium seals to be collected in polythene lined waste bin.

    - Aluminium containers to be decontaminated with 4.0% NAOH in scrap room before transfer to scrap yard.

    - Powders to be decontaminated in washing area with 4.0% NAOH and finally shall be transferred to ETP through drain.

    - Before transfer, the scrap shall be checked, labelled and verified by production / QA personnel respectively.

  - Scrap transfer / disposal from packing area

    - In packing area scrap generated are rejected labels, cartons, WFI ampoules, vials, shippers, polythene bags, catch covers, BOPP caps, paper, gum container and ink bottles etc.

    - All the printed packing materials to be disposed after torning the same so that they remain un-useable.

    - WFI ampoules shall be cut before disposal.

    - Rejected vials to be opened and decontaminated in a scrap room before disposal.

    - Approved and under test labels pasted on the shippers and cartons, WFI ampoule or any ware to be destroyed or cut before disposal.

    - Papers and polythene bags to be torned before disposal.

    - Before transfer the scrap shall be checked, labelled and verified by production and QA personnel respectively.

    - No intact pack and printed packing material should be present in the scrap received from the production blocks.

  - Scrap transfer / disposal from warehouse

    - Scrap generated in warehouse are rejected bottles, rejected ROPP caps, rejected measuring cups, rejected aluminium  / PVC foils, rejected catch boxes, cartons, 5 ply / 7 ply shippers and other corrugated boxes, polythene bags, cable ties, paper, plastic bottles and vials etc. shall be transferred to scrap room by warehouse officer.

    - The scrap shall be weighed before transfer and quantity shall be entered in scrap transfer and verification format.

    - Before transfer the scrap shall be checked, labelled and verified by warehouse and QA personnel respectively.

  - Procedure for preparation of 2% w/v Sodium hydroxide solution

    - Take 20 litres of purified water in a container and then add and dissolve 0.400 kg Sodium hydroxide pellets with continuous stirring. During the preparation wear hand gloves and goggles for safety purpose.

  - Procedure for preparation of 4% w/v Sodium hydroxide solution

    - Take 20 litres of purified water in a container and then add and dissolve 0.800 kg Sodium hydroxide pellets with continuous stirring. During the preparation wear hand gloves and goggles for safety purpose.

  - Decontamination procedure

    - In case of B-Block take leftover powder (non-recoverable) in a container, add potable water (1:2 ratio) (for 1kg of powder 2 litres of water required) and mix it properly with stirring rod (manually) then pour in wash area drain.

    - In case of warehouse take leftover powder (non-recoverable) in a container, add potable water (1:2 ratio) (for 1kg of powder 2 litres of water required) and mix it properly with stirring rod (manually) then pour in wash area drain.

- Annexure (S)

Annexure 1- Scrap transfer/disposal and verification format.

- Reference(S)

NA

- Glossary

SOP		:   Standard Operating Procedure

No.		:   Number

NA		:   Not applicable

BMR/BPR	:   Batch Packing and Control Record

QA		:   Quality Assurance

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |