- Objective

To lay down the procedure for cleaning of return air filter.

- Scope

This SOP is applicable to for cleaning of return air filter in Area of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP.

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Open the cover plates of filter housing.

  - Remove the filters and put it into the poly bag and put “TO BE CLEANED” label.

  - Transfer the filters to Equipment wash area.

  - Clean the Filter Assembly by dry duster and put back the cover plates.

  - Remove the filters from poly bag and suck the dust from filters by using vacuum cleaner.

  - Clean the filter using vacuum cleaner in the direction to normal flow.

  - After cleaning filters put it into the fresh poly bag and put “CLEANED” label.

  - Open the cover plate of filter housing.

  - Remove the filter from polybags and fix it back to its correct position and ensure that there is no gap between filter and frame.

  - Tighten all nuts and bolts.

  - Put back the cover plates.

  - Cleaning to be Recorded in Annexure-I.

NOTE:   Cleaning of Return Air Filter to be done on every Product change over.

- Annexure (S)

Annexure-I Return Air Filter Cleaning Record

- Reference (S)

NA

- Glossary

SOP    :	  Standard Operating Procedure

No.     : 	  Number

BP      :             Beta production

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |