- Objective

To lay down a procedure for Operation and Cleaning of Roto Cone Vacuum Drier.

- Scope

This SOP is applicable for Operation and Cleaning of Roto Cone Vacuum Drier in Production Areas of Alkem Laboratory Ltd., Unit-II, Baddi.

- Responsibility:

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability:

Head Production

- Procedure:

  - Precaution :

    - Ensure that the discharge valve, charging lid is securely tighten and then fix and tighten the discharging lid on the opening securely.

    - Ensure that no operation is inside working area and there is no obstruction to the RCVD and close the front.

  - Operation :

    - Switch ON the “Main Switch” and thereafter switch on the “Machine Main Switch”

    - Switch ON the “BLENDER MOTOR OFF”; Red switch.

    - For Drying:

      - Set the temperature of the RCVD by temperature controller

      - Open the Hot water circulation valve and then start the water circulation by taking “SV” Switch “ON” position.

      - Set the “TIMER” switch in “MANUAL” mode and start the blender motor by pressing “BLENDER MOTOR ON” and set the speed by “SPEED SET” knob and wait for achieve the required temperature.

      - Take the charging port to the top position by inching operation.

      - Switch off the “Machine Main Switch” and “Main Switch”.

      - Open the front railing and then open the Charging Lid manually.

      - After the completion of charging close the charging lid tightly.

      - Ensure that no operation is inside working area and there is no obstruction to the RCVD and close the front railing.

      - Set the time of drying by taking the “TIMER” in “AUTO” mode and then set the required time.

      - Start the blender motor by pressing “BLENDER MOTOR ON” and set the RPM by “SPEED SET” knob.

      - Close the “Vacuum Releasing Valve” and then switch on the “VACUUM” switch.

      - When drying is completed switch off the “Vacuum” switch, open the “Vacuum Releasing Valve” and release the vacuum by “Vacuum    Releasing button”.

      - Close the hot water circulation and start the normal water circulation by using the water circulation valve.

      - Keep the “TIMER” in “MANUAL” mode and start the blender motor By “BLENDER MOTOR ON” to maintain the dried materials in room in temperature.

      - After maintaining the temperature  bring RCVD to discharge position by inching switch so to ensure that  discharge valve is facing downwards .

      - Switch off the “Machine Main Switch” and “Main Switch”.

      - Bring the labelled IPC below discharge valve then open the valve slowly and fill the blend in double lined polythene bag in IPC and close the valve after completion of activity.

    - For Blending:

      - Set the required temperature by temperature controller.

      - Open the normal water circulation valve and then start the water circulation by taking “SV” switch “ON” position. Set the “TIMER” switch in “MANUAL” mode and start the blender motor by pressing “BLENDER MOTOR ON” and set the speed by “SPEED SET” knob and wait for maintain the temperature.

      - Take the charging port to the top position by inching operation.

      - Switch off the “Machine Main Switch” and “Main Switch”.

      - Open the front railing and then open the Charging Lid manually.

      - After the completion of charging close the charging lid tightly.

      - Ensure that no operation is inside working area and there is no obstruction to the RCVD and close the front railing.

      - Set the time of blending by taking the “TIMER” in “AUTO” mode and then set the time by “TIMER” and set the speed by “SPEED SET” knob.

      - Start the blender motor by pressing “BLENDER MOTOR ON”.

      - When blending is completed keep the “TIMER” in “MANUAL” mode.

      - To discharge the RCVD bring RCVD to discharge position by inching switch so to ensure that discharge valve is facing downwards.

      - Switch off the “Machine Main Switch” there after “Main Switch”.

      - Bring the double lined polybag in IPC below discharge valve then       open the valve slowly and filled the blend in double lined polythene bag and close valve after completion the activity and close the polybag tightly.

  - Cleaning and change over :

    - Cleaning and change over during new batch of the same / similar product with same or ascending strength is to be taken, provided colour is same (Type A cleaning).

      - Switch ‘OFF’ the machine.

      - Remove “IN PROCESS” / “STATUS” label and affix “TO BE CLEANED” label.

      - Clean the RCVD using dry lint free cloth.

      - Remove “TO BE CLEANED” label and affix “STATUS” label duly signed by production officer/ Executive after verification from IPQA personnel.

    - Cleaning and change over during different active ingredients, colour, flavour and descending strength (Type B cleaning).

      - Switch ‘OFF’ the machine.

      - Remove “IN PROCESS” / “STATUS” label and affix “TO BE CLEANED” label.

      - Dry clean the equipment using lint free cloth.

      - Dismantle the parts (Charging Lid and Filter Assembly) and take it to wash.

      - Clean all the parts using potable water and finally rinse with purified water.

      - Dry clean the parts with the help of dry lint free cloth.

      - Fix the all dismantle parts to equipment.

      - Ensure that the final swab / rinse sample, if required is to be collected by IPQA personnel.

      - Remove “TO BE CLEANED” label and affix “CLEANED” label duly signed by production officer/ Executive after verification from IPQA personnel.

      - If cleaned equipment remains unused for 72hrs, then it shall be clean as per type “B” Cleaning.

      - After preventive maintenance, all the contact parts of equipment shall be cleaned as per type “B” cleaning before use.

      - After four consecutive batches of same / similar product “B” type cleaning shall be performed.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary:

SOP		:		Standard Operating Procedure

No.		:		Number

IPQA		:		In Process Quality Assurance

RCVD		:		Roto Cone Vacuum Drier

NA		:		Not Applicable

- Revision history

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |