- Objective

To lay down the procedure for Operation and Cleaning of Powder Deduster.

- Scope

The SOP is applicable for Operation and Cleaning of Powder Deduster in Alkem Laboratories Limited, Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Pre-start up

    - When the same batch of the same product is to be continued.

      - Visually inspect the equipment and area for cleanliness.

      - Ensure that the previous run was that of the same product and of the same batch number.

      - Ensure that the environmental conditions are as specified in BMR and record it in the Environment Monitoring Record.

      - Ensure that the brushes of the machine rotate in proper direction.

      - Affix the “status” label on the equipment.

    - When a new batch of the same product is to be taken.

      - Visually inspect the Dedusting machine and area for cleanliness.

      - Ensure that there are no remnants of the previous batch.

      - Ensure that the environmental conditions are as specified in BMR and record it in the Environment Monitoring Record.

      - Ensure that the brushes of the machine rotate in proper direction.

      - Remove “cleaned “label and affix “status” label.

    - When a different product is to be taken.

      - Visually inspect the area and equipment thoroughly so as to ensure that the same is free from the previous product.

      - Ensure that the final swab/ rinse sample release report is available in the BMR.

      - Ensure that the environmental conditions are as specified in BMR and record it in the Environment Monitoring Record.

      - Ensure that the brushes of the machine rotate in proper direction.

      - Get the line clearance for the equipment as well as the area from a QA Officer/ Executive.

      - Remove “cleaned” label and affix “status” label to the Dedusting machine.

    - Start up

      - Take a blank trial for one minute of Dedusting machine to ensure proper functioning.

      - Place an empty, clean, pre labelled HDPE container lined with polybag beneath the discharge chute of Dedusting machine.

      - Place the Dedusting machine along with HDPE containers beneath the discharge chute of compression machine

    - Operation

      - Switch on the machine.

      - Allow the compressed tablets to pass through the spiral sieve of Dedusting machine.

      - Collect the dedusted tablets in HDPE container.

    - Shutdown

      - Switch off the Dedusting machine.

      - Dry clean the Dedusting machine with vacuum cleaner and Dedusting machine.

    - Cleaning Procedure

      - At the end of shift/ batch change over of same on similar product with ascending strengths (provided colour is same. Type A cleaning).

        - Switch off the machine.

        - Remove “status” label and affix “to be cleaned” label.

        - Dry clean the Dedusting machine using vacuum cleaner and lint free cloth.

        - Dismantle the parts (lid and spiral sieve) and dry clean them with the help of dry lint free cloth.

        - Fix the dismantled parts to Dedusting machine.

        - Remove “to be cleaned” label and affix “cleaned” label duly signed by production Officer/Executive.

      - Cleaning during changeover of product with different active ingredients, colour, descending strengths.(Type B cleaning)

        - Switch off the Dedusting machine. Remove “status” label and affix “to be cleaned” label. Dry clean the Dedusting machine using vacuum cleaner and Dedusting machine.

        - Cover the electrical switch and motor using poly bags. Dismantle the parts (lid and spiral sieve) and take it to equipment wash area using SS trolley.

        - Wipe the body of Dedusting machine using wet lint free cloth. Clean the dismantled parts (lid, spiral sieve) using jet of potable water and finally rinse with purified water.

        - Dry clean the dismantled parts using dry lint free cloth. Remove the covered polythene bags from electrical switch and motor.

        - Fix the dismantled parts to Dedusting machine, Ensure that the final swab/rinse sample is collected by IPQA Officer/Executive.

        - Remove “to be cleaned” label and put “cleaned” label duly signed by Production Officer/Executive.

- Annexure (S)

NA

- Reference(S)

NA

- Glossary

SOP		:   Standard Operating Procedure

NA		:   Not applicable

BPR		:   Batch Packing Record

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |