- Objective

To lay down a procedure for Operation and Cleaning of Metal detector.

- Scope

This SOP is applicable to metal detector used in production of Alkem Laboratory Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Procedure for setting of metal detector

    - Connect the cables provided with metal detector to the main switch.

    - Switch on the power supply; wait for few minutes to stabilize the metal detector for auto check.

    - Press key “lock on” display panel, display shows “*SECURITY LEVEL NO 1 *’’.

    - Press “right arrow key” display show “* SECURITY LEVEL NO. 2*”.

    - Press “ENTER” key, display shows “* ENTER PASSWORD # 000”.

    - Press down arrow key till display shows password no. “ENTER PASSWORD N 952”.

    - Press ENTER key display will show “*GET PRODUCT 001*”.

    - Press “right arrow key” till display will show “*CH – 1THR: 400mv *”.

    - Press “up” arrow key to increment the threshold value till display shows “* CH1- THR: 640”.

    - Press “left arrow” key till display shows “*CH-1 THR: 400 mv*”.

    - Press “enter” key, a cursor will start blinking at “0.0” location of phase (0.1 degree resolution).

    - Press “left arrow” key, a cursor will start blinking at “00.0” location of phase. (10.0 degree resolution).

    - Press up arrow key. Increment phase by 10.0 degree display will show signal bar graph, now pass Metal block to be detected, observe signal of metal block on bar graph which shall be maximum.

    - Again press “Up Arrow” key. Increment phase by 10.0 degree display will show signal bar graph, now pass metal block on bar graph. If trend of signal level increment is observed with repetition of above step (increment phase by 610.0 degree) then continue same till maximum signal level is achieved on bar graph. For case of signal monitoring, signal level after change of parameter is kept on hold for “5” seconds, also bar graph mode of numeric value can be selected by pressing “DISPLAY” key once. If trend of signal level decrement is observed with repetition of phase increment, then do phase adjustment in reverse way. Press “Down Arrow” key to decrement phase by 10.0 degree now pass metal block to be detected, observe signal level of Metal block on bar graph. Now go on decrementing the phase angle by 10.0 degree in such a way that maximum signal level is seen on bar graph. Find out the optimum phase angle at which signal level achieved is maximum. Fine tuning of best setting can be done further by adjusting phase with 1.0 degree resolution, repeat increment and decrement procedure with 1.0 degree to get maximum possible signal for CH1. Press “Right Arrow” key till display shows “* SET UP: SAVE PRODUCT 001*. Press “ENTER” key, display will show * SAVING PRODUCT DATA WAIT*with this step whatever changes are “SAVED”, similarly CH2 PHASE setting can be done repeating all above steps. After all final adjustment press “DISPLAY” key to move to “BAR-LED MODE”.

  - Operation

    - Pass the metal blocks (FE, NFE & SS) through Metal Detector before starting the activity.

    - Allow the machine to display the bar graphs & threshold mode.

    - Pass standard Test blocks FE, NFE and SS through metal detector entry channel, observe signals of test blocks rejected by metal detector that means Metal Detector is ready for use.

    - Attach the Metal Detector to tablet discharge chute of Deduster M/C.

    - Pass the tablets to be tested and observe signals on bar leds and observe rejections. If there shall not be any rejection that means product passed and have no metal contamination.

    - Repass the rejected tablets through tablets through Metal Detector for verification.

  - Shutdown

    - Switch off the machine.

    - Dry clean the Machine using lint free cloth.

  - Cleaning procedure

    - At the end of shift/Batch changeover of same or similar product (Type A)

      - Remove “status” label. Affix 'to be cleaned' label.

      - Dry clean the machine using dry lint free cloth.

      - Remove “to be cleaned” label and affix “cleaned” label duly signed by Production Officer/Executive.

    - When the changeover to another product. (Type B)

      - Remove 'status' label and affix 'to be cleaned' label.

      - Remove all powder loosely sticking to the machine.

      - Dry clean the machine with the help of a vacuum cleaner and lint free cloth.

      - Cover the control panel with polythene bags.

      - Dismantle the parts of metal detector and take it to equipment wash using ss trolley.

      - Clean the dismantled parts of metal detector using jet of potable water and finally rinse with purified water.

      - Dry clean the metal detector using lint free cloth.

      - Dry clean the dismantled parts using lint free cloth.

      - Fix the dismantled parts of metal detector.

      - Remove the covered polythene bags of control panel.

      - Wipe the control panel with dry lint free cloth.

      - Remove 'to be cleaned' label and affix 'cleaned' label duly signed by Production Officer/ Executive.

- Annexure(s)

NA

- Reference(s)

NA

- Glossary:

SOP	:  Standard Operating Procedure

No.	:  Number

IPQA	: In process quality assurance

- Revision history

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |