- Objective

To lay down a procedure for Operation and Cleaning of Automatic Capsule Filling Machine (AF-150) and its Line Machines.

- Scope

The SOP is applicable for Operation and Cleaning of Automatic Capsule Filling Machine (AF-150) in Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is done as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

    - Ensure that the compressed air and vacuum shall not be less then the limit.

    - Do not run the machine without closing the safety door.

    - Ensure that the vacuum cut off valve shall be working properly.

    - Ensure that the line machines are working properly.

  - Pre-start up

    - Remove the “STATUS” Label and affix “CLEANED” Label duly signed by Production Officer/ Executive.

    - Ensure proper functioning of machine lubrication system.

    - Ensure that all moving parts are free by rotating the machine clockwise manually

    - Ensure correct setting of Dosing Disk assembly.

    - Ensure correct setting of powder hopper.

    - Capsule separation pressure shall be in between 15-25 inch of Hg.

    - Ensure that the gap between dosing disk and scraper plate shall be 0.05 to 0.1 mm

    - Ensure that the gap between cap bush and body bush shall be 0.1 mm.

    - Ensure that the stirrer of a powder hopper is rotating freely in the powder hopper.

    - Ensure that the empty capsule Sorter and elevator, Dedusting and Polishing machine, Mini Capsule Sorter, Empty Capsule Sorter, Filled Capsule Sorter and Elevator and Metal Detector shall be working properly.

    - Ensure that the area around the AF 150T machine and line machines shall be cleaned as per SOP in force.

  - Start up

    - Bring the HDPE container along with double poly bags having “STATUS LABEL” Label duly signed by Production Officer/Executive.

    - Bring the IPC/BIN filled with granules to the cubicle which shall have proper status label and approved label on it duly signed by IPQA Officer/ Executive.

    - Load the IPC/BIN with the Lifting and Positioning device and place it to the Hopper.

    - Open the butterfly valve of IPC/BIN and fill the hopper.

    - Set the hopper valve to allow the materials to flow into the Dosing disk.

    - Bring the Empty Hard Gelatin capsules to the cubicle and load into Empty capsule hopper.

  - Operation

    - Open the compressed air valve.

    - Turn On the PLC button of AF 150 and also switch on the line machines.

    - Press loading ON, Select mode depending on type of filled material. Select 1 for Powder Mode.

    - Press ESC to come out from the menu. Set the speed of machine by rotating the speed tuning knob.

    - Set the weight by adjusting the weight adjustment screw as follow :

      - Clockwise rotation - Increases the weight.

      - Anticlockwise rotation - decreases the weight.

    - Set the locking length.

    - Rotate the machine manually to ensure the smooth running.

    - Run the machine in auto mode.

    - Press Drive ON to run the machine.

    - Take the samples of capsules and check the parameters as per BMR.

    - After getting satisfactory results collect the filled capsules in S.S container and then place the capsule in HDPE drum and put “CONTAINER” Label on it.

  - Shutdown

    - Press drive OFF to stop the machine.

    - Dry clean the machine with lint free duster.

    - Close the valve of IPC.

    - Remove the powder from hopper.

    - Remove the empty capsules from hopper.

    - Destroy the rejected capsules by putting them into water.

    - Record the same in BMR.

    - Clean the cubicle as per SOP in force.

  - Cleaning Procedure

    - At the end of the shift/batch changeover of same product

      - Put OFF the mains.

      - Remove all the powder using vacuum cleaner.

      - Remove powder hopper by loosening the knob.

      - Remove the dosing disk.

      - Remove the brush of DP 100 and clean with vacuum cleaner.

      - Clean above parts with dry lint-free duster.

      - After five ‘Type A’’ cleaning ‘Type B’’ cleaning shall be performed.

    - Cleaning during changeover of product

      - Put OFF the mains and put 'TO BE CLEANED' Label on the machine.

      - Remove all powder using a vacuum cleaner.

      - Remove hopper by loosening the knob.

      - Remove powder dosing assembly.

      - Dismantle the magazine and finger projection.

      - Dismantle the pins.

      - Remove the brush of DP 100, Sorting Plate of Empty Capsule Sorter Elevator, Sorting Plates and sorting drum of Mini Capsule Sorter.

      - Dismantle the cap bush and body bush.

      - Take the above parts to washing area with the help of trolley having 'TO BE CLEANED' Label.

      - Clean all the parts using nylon scrubber with potable water.

      - Finally rinse the parts with purified water.

      - Dry the parts with lint-free duster.

      - Dry the DP brush under Infra red Drier.

      - Wipe the surface of machine with dry lint-free cloth.

      - Assemble all the above parts on the AF 150 machine.

      - Remove the 'TO BE CLEANED' Label and put 'CLEANED' Label duly signed by Production Officer/Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

      - After preventive maintenance, all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary

SOP		:	Standard Operating Procedure

No.		:	Number

IPQA		:	In process quality Assurance

BMR		:	Batch Manufacturing Record

IPC		:	In process container

S.S.		:	Stainless Steel

DP Machine	:	Dedusting and Polishing machine

- Revision History:

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |