- Objective

To lay down a procedure for Operation and Cleaning of lifting and positioning device.

- Scope

The SOP is applicable to lifting and positioning device installed in manufacturing area of Alkem Laboratories Limited, Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure:

  - Precautions

    - During operation if you feel any undue noise, stop the machine immediately and information should be made to engineering department.

  - Pre-start up

    - Ensure the cleanliness of mobile lifter.

    - Ensure that the level of hydraulic oil is sufficient enough to carry out the operation.

    - Ensure that the IPC/BIN lifting platform is fixed properly.

  - start up

    - Bring the lifting device platform to the site of operation.

    - Switch on the main.

  - Operation

    - Place the IPC/BIN properly with proper alignment of lifting platform.

    - Lift up the IPC/BIN by pressing the ‘UP’ button on the control panel.

    - After completion of operation lift down the IPC/BIN by pressing the ‘DN’ button on the Control panel.

  - Shutdown

    - Put OFF the mains and keep the mobile lifter in its place.

  - Cleaning procedure

    - Dry clean the equipment using a vacuum cleaner and lint free cloth.

- Annexure (S)

NA

- Reference(S)

NA

- Glossary

SOP		:   Standard Operating Procedure

No.		:   Number

IPC		:   Intermediate Product container

NA		:   Not applicable

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |