- Objective

To lay down a procedure for Operation and Cleaning of Coating machine.

- Scope

The Procedure is applicable for Operation and Cleaning of coating area in Alkem laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is done as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

    - Never stop the machine with the interlock switches, use only stop button to stop the machine.

    - Inspect interlock switches and cables regularly. Replace if damaged or defective.

    - Disconnect machine and lock out power before opening, adjusting, cleaning, servicing and changing guns.

    - Never insert body parts or foreign objects into the running machine.

    - Switch on the mains.

  - Prestart up

    - For same batch continuation, when same batch is to be continue.

      - Visually inspect the equipment and the area for cleanliness.

      - Ensure that the previous run was of the same batch.

      - Ensure that the environmental conditions shall be as specified in BMR and record it in the environmental control record.

      - Affix “Status” label on the equipment duly signed by Production Officer/Executive.

    - For new batch of same product, when different batch of same product is to continue

      - Visually inspect the Coating machine and area for cleanliness.

      - Ensure that there shall not be any remnants of the previous batch.

      - Remove 'CLEANED' label and affix “Status” Label duly signed by Production Officer/Executive.

      - Ensure that the environmental conditions shall be as specified in the BMR and record it in environmental control record.

      - Take the line clearance from IPQA Officer/Executive.

    - For new batch of different product, when different is to continue

      - Visually inspect the equipment thoroughly so as to ensure that the remnants of the previous product shall be absent.

      - Remove "CLEANED" label and affix “Status” label to Coating machine duly signed by Production Officer/Executive.

      - Ensure that the environmental conditions shall be as specified in BMR and record it in the environmental control record.

      - Take the line clearance from IPQA Officer/Executive.

      - Ensure the cleanliness of Infra Red Drier.

      - Adjust the height of Infra Red Drier as per the requirement.

  - Start up

    - Fix the silicone tube in the peristaltic pump and connect it to the spray header and return line to the tank.

    - Check the batch identity of the coating solution and attach its outlet with the silicone tubing of the peristaltic pump.

    - Ensure that the compressed air pressure is fixed at 4 kg/cm2.

    - Adjust the spray rate and spray gun distance as mentioned in the BMR.

  - Operation

    - Manual Mode

      - Load the tablets into the pan.

      - Close the window of the Coating machine.

      - Twist the “RED” switch clockwise to “ON” the machine.

      - Press “GREEN” button when display shows “COATING MACHINE” press, “Log in” and enter the password then press “OK”, again press “Log in”. Display shows “Man “Auto   “Auto Op” Film” Sugar”.

      - Press “Man then press reset to stop alarm noise. Op”

      - Press “Para” to set parameters required as per BMR.

      - Press “Pan ON” to start pan.

      - Press “Blower ON” to start blower.

      - Press “Heater sol ON” to start heater or inlet temp.

      - Press “Spray ON” to start spray.

      - Continue the coating operation as specified in BMR, and record it in respective log book.

      - Unload the tablet in the pre labelled HDPE container with the help of unloading device.

      - After completing the operation “log out” and switch off the Mains by pressing “Red” button.

    - Auto Mode

      - Load the tablets into the pan.

      - Close the window of the Coating machine.

      - Twist the “RED” switch clockwise to “ON” the machine.

      - Press “GREEN” button when display shows “COATING MACHINE” press “Log in” and enter the password and press “OK” ,again press “Log in”. Display shows “Man “Auto “Auto Op” Film” Sugar”.

      - Press “Auto   to run the machine in Auto mode.

      - Press “Para” to set parameters required as per BMR.

      - Continue the coating operation as specified in the BMR.

      - After dosing time pan stops automatically.

      - Unload the tablet in the pre labelled HDPE container with the help of unloading device.

      - After completing the operation “Log out” and switch of the Mains by pressing “Red” button.

  - Shutdown

    - Ensure that the coating solution shall not be remaining left in inlet and outlet pipeline.

    - Dry clean the Nocona using dusters.

  - Cleaning Procedure

    - At the end of shift/Batch changeover of same product with ascending strengths (provided colour is same. Type A)

      - Remove all remnants of previous batch from the surrounding area and equipment.

      - Remove “Status” Label and affix 'TO BE CLEANED' Label duly signed by Production Officer/Executive.

      - Dry clean the main inlet and exhaust plenum using duster.

      - Wash the nozzle/needle of spray gun with purified water.

      - Wash the Holding tank with potable water and finally rinse with purified water.

      - Clean the marpene tube with potable water and then rinse with purified water.

      - Remove “TO BE CLEANED” Label and affix “CLEANED” Label duly signed by Production Officer/Executive.

      - After four ‘Type A’’ cleaning ‘Type B’’ cleaning shall be performed.

    - For the changeover to another product/changeover to the product with descending strengths or changeover to a product with different colour. (Type B)

      - Remove “Status” Label and affix 'TO BE CLEANED' Label.

      - Remove all remnants of previous product from the surrounding area and equipment.

      - Remove all coating material loosely sticking to Coating machine and spray header.

      - Dry clean the Coating machine with the help of a vacuum cleaner and duster.

      - Cover the control panel with polythene bags.

      - Remove the exhaust and inlet plenum by de clamping the slide duct and lifting it upwards.

      - Dismantle the parts of Coating machine (Spray guns, Solution Inlet and Outlet tubing’s and Solution tank) and take it to equipment wash using trolley.

      - Remove the exhaust and inlet plenum by de clamping the slide duct and lifting it upwards.

      - Dry clean the Coating machine using dusters.

      - Clean the dismantled parts of Coating machine using potable water and finally rinse with purified water.

      - Dry clean the dismantled parts using dusters.

      - Clean the main exhaust and inlet plenum using potable water.

      - Dry clean the main inlet and exhaust plenum using duster.

      - Fix the main inlet and exhaust plenum to Coating machine with the help of clamps.

      - Remove the covered polythene bags of control panel.

      - Wipe the control panel with wet dusters.

      - Fix the dismantled parts to the Coating machine.

      - Remove 'TO BE CLEANED' Label and affix 'CLEANED' Label duly signed Production Officer/ Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

      - After preventive maintenance, all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

  - Troubleshooting

    - During operation if any leakage of solution droplets stops the machine by emergency switch.

    - Check & clean the spray gun.

    - If you feel any undue noise immediately stop the machine, check & repair the fault.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary

SOP		:	Standard Operating Procedure

No.		:	Number

BPR		:	Batch Packing Record

BMR		:	Batch Manufacturing Record

HDPE		:	High density polyethylene

IPQA		:	In process quality assurance

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |