- Objective

To lay out a procedure for Operation and Cleaning of Colloidal Mill.

- Scope

This SOP is applicable to colloidal mill used in coating area of production of Alkem Laboratory Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Pre start up

    - For same batch continuation, when same batch is to be continue.

      - Visually inspect the equipment and room for cleanliness.

      - Ensure that previous run was of the same batch number.

      - Put the “STATUS” Label on the equipment duly signed by Production Officer/Executive.

      - Check the temperature and relative humidity of the area and record it in the environmental control record.

    - For new batch of same product, when different batch of same product is to continue

      - Visually inspect the equipment and the room for cleanliness.

      - Ensure that there shall not be any remnants of previous batch.

      - Remove ‘CLEANED’ Label and affix “STATUS” Label on the equipment duly signed by Production Officer/Executive.

      - Check the temperature and relative humidity of the area and record it in the environmental control record.

    - For new batch of different product, when different is to continue

      - Visually inspect the equipment and the room for cleanliness.

      - Ensure that there shall not be any remnants of the previous product.

      - Remove “CLEANED” Label and affix “STATUS” Label duly signed by Production Officer/Executive.

      - Check the temperature and relative humidity of the area and record it in the environmental control record.

  - Start up

    - Switch ‘ON’ the main power supply.

    - Switch ‘ON’ the machine by pressing Green button on the panel board and check the smooth rotation.

  - Operation

    - Ensure that Hopper, Solution Recirculation loop and Discharge Valve shall be fitted properly.

    - Pass the material, which is to be milled as per the procedure given in BMR.

    - After completion of milling, rinse the hopper with some quantity of solvent used for solution preparation so as to remove the adhered material from the hopper.

  - Shutdown

    - Switch “OFF” the Colloidal Mill by pressing the Red colour Button on panel board.

    - Switch “OFF” main power supply.

  - Cleaning

    - At the end of Shift/Batch changeover of same product with ascending strengths (provided colour is same. Type A

      - Affix “TO BE CLEANED” Label on the machine duly signed by Production Officer/Executive.

      - Remove loosely stuck material from the machine by using clean duster.

      - Remove the contact parts and non-contact parts by using clean duster.

      - Remove ‘TO BE CLEANED’ Label and affix “Cleaned” Label duly signed by Production Officer/ Executive.

      - After five ‘TYPE A’ Cleaning ‘TYPE B’ cleaning shall be performed.

    - Cleaning during changeover of product with different active ingredients, colour, descending potency – Type ‘B’.

      - Affix the Label “TO BE CLEANED” on the Machine.

      - Remove loosely stuck material from the machine by using clean duster.

      - Dismantle the machine parts (Hopper, Recirculation loop with discharge valve).

      - Affix “TO BE CLEANED” to the dismantle parts and transfer them to wash area.

      - Wash the parts with Potable water, and finally rinse it with purified water.

      - Dry the dismantled clean parts.

      - Assemble the dried clean parts in back sequence.

      - Remove “TO BE CLEANED” Label and affix ‘CLEANED’ Label duly signed by Production Officer/ Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

      - After maintenance, all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

- Annexure(s)

NA

- Reference(s)

NA

- Glossary

SOP	:  Standard Operating Procedure

No.	:  Number

IPQA	: In process quality assurance

- Revision history

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |