- Objective

To lay out a procedure for Operation and Cleaning of tablet inspection machine.

- Scope

This SOP is applicable to tablet inspection machine used in production of Alkem Laboratory Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Pre start up

    - For the same batch

      - Visually inspect the machine and room for cleanliness

      - Ensure that previous run was of the same batch.

      - Ensure that the material lying in the room shall be of the same batch.

      - Ensure that the environmental conditions shall be as specified in BMR.

      - Affix ”Status” label on the equipment duly signed by Production Officer/Executive.

      - Place the HDPE container lined with double polythene bag for collection of inspected tablets and put “Status” Label on the drum.

    - For the new batch of same the Product

    - Visually inspect the equipment room for cleanliness. Ensure that there shall not be any remnants of previous batch.

    - Put the “Status” Label on the machine.

    - Check the material (which shall to be taken for Inspection) for batch identity and bring them into Inspection room.

    - Check the temperature and relative humidity and record it in the environment control record.

    - Take the line clearance from IPQA Officer/Executive.

  - For the different product

    - Visually inspect the machine and the room for cleanliness. Ensure that there shall not be any remnants of previous product.

    - Affix the “Status” Label on the machine.

    - Check the material (which is to be taken for inspection) for Product and Batch no and bring them into Inspection room.

    - Check the temperature and relative humidity as specified in BMR and record it in the environment control record.

    - Take the Line clearance from IPQA Officer/Executive

  - Start up

    - Fill the Hopper with tablets.

    - Adjust the shutter as per the size of the tablet.

    - Switch 'ON' the main electric supply.

  - Operation

    - Switch ON the Motor and Vibrator.

    - Set the frequency of Vibrator by rotating the black knob.

    - Tablet passes through the channel to 1st inspection belt , where the tube lights are fixed on stand which helps in checking following parameters:  a)Capping  b) Mottling c) Broken edge

    - The belt is having brushes which allow the tablets to pass, facing on one side and after 1st inspection belt when the tablet drops on the 2nd inspection belt it reverse from its facing on surface, so Operator can easily inspect the tablets.

    - Collect the good tablets in a HDPE container lined with double polythene bag kept below the outlet of the Inspection Belt. Remove the rejected tablets and keep them in another drum and Label it as Recovery.

    - After inspection tie all containers duly labelled and keep in quarantine area.

  - Shutdown

    - Switch off the Motor and Vibrator of the machine.

    - Dry clean using dusters.

  - Cleaning procedure

    - At the end of shift/Batch changeover of same product with ascending strengths (provided colour is same. Type A cleaning)

      - Remove all remnants of previous batch from the surrounding area and equipment.

      - Remove “Status” Label. Affix ‘'TO BE CLEANED'’ Label. duly signed by Production Officer/Executive

      - Remove all powder loosely sticking to machine.

      - Dry clean the belt and machine body with the help of a vacuum cleaner and duster.

      - Remove ‘'TO BE CLEANED “Label and affix “Cleaned” duly signed by Production Officer/Executive.

      - After five ‘Type A’’ cleaning ‘Type B’’ cleaning shall be performed.

    - When the changeover to another product/changeover to the product with descending strengths or changeover to a product with different colour. (Type B cleaning)

      - Remove “Status” Label and affix ‘'TO BE CLEANED'’ Label. Duly signed by Production Officer/Executive.

      - Remove all remnants of previous Product from the surrounding area and equipment.

      - Remove all powder loosely sticking to machine.

      - Dry clean with the help of a vacuum cleaner and duster.

      - Cover the control panel with polythene bags.

      - Remove the hopper by de clamping the slide duct and lifting it upwards.

      - Dismantle the parts and take it to equipment wash using trolley

      - Clean the dismantled parts using potable water and finally rinse with purified water.

      - Dry clean the dismantled parts using dusters.

      - Remove the covered polythene bags of control panel.

      - Wipe the control panel with wet dusters.

      - Fix the dismantled parts to the machine.

      - Final rinse water sample shall be collected by IPQA Officer/ Executive

      - Remove ‘'TO BE CLEANED'’ Label and affix ‘'CLEANED'’ Label duly signed by Production Officer/ Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

      - After preventive maintenance, all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

- Annexure(s)

NA

- Reference(s)

NA

- Glossary

SOP	:  Standard Operating Procedure

No.	:  Number

IPQA	: In process quality assurance

- Revision history

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |