- Objective

To lay down a procedure for cleaning and sanitization during batch changeover and product changeover in manufacturing area.

- Scope

This SOP is applicable to production areas of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

There are two types of cleaning to be carried out in manufacturing area beside routine cleaning and sanitization activities.					 			First is Type A cleaning, which will be carried out

- During changeover of batches between same products.

- In case of campaign batches that is same strength of same products up to fifth batch.

Second is Type B cleaning, which will be carried out

- During changeover of batches between higher strength to lower strength of same products and next molecule is different from previous one or if the formulation is same except colour / flavour.

- In case of campaign batches that is same strength of same products after completion of fifth batch.

- If the area/equipment are lying unused for more than 72 Hrs. then before start up of a new batch.

- In case of any major maintenance work carried out which may affect the product quality during production.

- In case of any major power or HVAC failure which may affect the product quality during production.

Note: In secondary packing area only Type A cleaning will be done.

  - Type A Cleaning procedure

    - For Type A cleaning the requirements are as follows:					Nylon brush, polythene bags, Lint free mops and vacuum cleaner.

    - Once the batch is over then use nylon brush to remove all the remnant powders lying in any parts of the machine and collect in a polybags and put status label on the polybags.

    - Weigh the powder and enter into the BMR in non-recoverable column for particular operation.

    - Use vacuum cleaner to remove loose dusts from all the surfaces including areas, which are difficult to access by hand.

    - Use lint free mops to clean the equipments thoroughly.

    - Put “CLEANED” label and enter cleaning detail into the Cleaning log as Annexure I.

    - Inform the housekeeping personnel and get the area cleaned as per Cleaning and sanitization SOP.

  - Type B Cleaning procedure

    - For Type B cleaning the requirements are as follows:

Nylon brush, polythene bags, Lint free mops, Vacuum cleaner, Two SS buckets with purified water and Water-jet machine.

    - Follow procedure 5.1.2 to 5.1.5 first.

    - Clean the subjected equipment using clean lint free mop with purified water.

    - Dip the lint free mop in the first bucket, squeeze the mop and clean the surfaces with the wet mop and then again dip the mop in the second bucket for cleaning of the mop and then dip in the first bucket and use for cleaning.

    - These procedure to be followed to clean the whole equipment, area, tables, trolleys, raiser grills, pendants or any other items which can contribute to cross-contamination.

    - For the areas which are difficult to clean in any equipment or areas needs to be cleaned with water-jet using water jet machine.

    - After cleaning visually verify the cleaned surfaces for remaining of any remnants.

    - The detachable parts to be dismantled as per respective machine SOPs, to be packed in polybags, and to be taken to washing area for cleaning as per cleaning SOP of production accessories.

    - Swab sample to be provided as per the requirement after cleaning is complete based on the requirements.

    - Put “CLEANED” label and enter cleaning detail into the Cleaning log as Annexure I.

- Annexure (S)

Annexure I:	Cleaning Log

- Reference (S)

NA

- Glossary

SOP		:		Standard Operating Procedure

NA		:		Not Applicable

BMR		:		Batch Manufacturing Record

HVAC		:		Heat Ventilation and Air Conditioning

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |