- Objective

To lay down the Procedure for reprocessing of recoverable tablet/capsule/dry syrup.

- Scope

This SOP is applicable for reprocessing of recoverable tablet/capsule/dry syrup in Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Recoverable Residue.

    - Recoverable Residues are those residues of the product, which are generated during following process of Manufacturing and packing operations.

    - Tablets beyond the specification of parameters like weight variation, hardness, thickness, during machine setting. Left over granules at the end of compression and   Residue generated at Inspection stage.

    - Capsules beyond the specification for parameters like weight variation, locking length. Left over powder/Pallets at the end of capsule filling and Recoverable Rejects generated after inspection stage.

    - Recoverable Rejects during Packing.

  - Storage of Recoverable Residue.

    - Recoverable Residue shall be collected in a HDPE drum lined with clean polythene bag.

    - RR shall be weight and affix the “RECOVERABLE” label duly singed by production officer and Verified by QA Officer.

    - Qty. of RR generated in Batch shall be recorded, in respective BMR as well as Annexure-I and in E.Com system.

  - Recoverable Residue Addition.

    - Recoverable Residue shall be, issued from Recovery Room, Qty shall be cross checked by officer Production/Q.A against the Qty specified in E-Com and Annexure-I

    - Recoverable Residue shall be inspected before addition for the presence of any foreign matters.

    - Details of RR shall be mentioned as per BMR in which RR is to be added and verify the same by QA.

    - RR shall be carry forward from E Com system for the batch in which RR is to be added.

    - RR should not be more than 5%w/w of total standard batch size.

    - RR which is to be added should not be more than 6 months old from the date of manufacturing.

    - If RR is kept for more then 6 months, such RR shall be analyzed by quality control department and attach QC Release Report with BMR.

    - If RR is not used before 12 months from the date of manufacturing, such RR shall be destroyed after taking approval from QA.

  - Procedure for recovery addition in dry syrup.

    - Collect the recoverable at stages of filling and packing operation.

    - Store the duly labelled recoverable blend under designated environment conditions.

    - Record the details of recoverable in Recoverable Statement of BMR.

    - In case of recoverable from packing stage, visually inspect the filled bottles for any type of damage.

    - Open the recoverable filled bottles of filling& packing stage to recover the net   content

    - Sift the total recoverable blend through the vibro sifter using the sieve as specified in BMR.

    - Add the sifted recoverable blend at the mixing stage as specified in BMR

  - Procedure for recovery addition in tablet

    - Collect the recoverable at stages of Compression, Coating and Packing operation.

    - Store the recoverable tablets duly labelled under designated environment conditions.

    - Record the details of recoverable in Recoverable Statement of BMR.

    - Take line clearance of Multi Mill and Vibro Sifter from QA Executive/ Officer.

    - In case of recoverable from packing stage, visually inspect the tablet for any pieces of foil.

    - Crush the recovered tablets using multi mill fitted with S.S. Screen. (Size of screen shall be as per specified in BMR).

    - Sift the crushed tablets using Vibro sifter equipped with sieve. (Size of sieve shall be as per specified in BMR).

    - Lubricate the granules as per BMR.

    - Send the request of the blend for Content Assay.

    - Compress the lubricated granules after approval by QC.

  - Procedure for recovery addition in capsule

    - Collect the recoverable at stages of filling and packing operation.

    - Store the duly labelled recoverable capsules under designated environment      conditions.

    - Record the details of recoverable in Recoverable Statement of BMR.

(In case of recoverable from packing stage, visually inspect the capsules for any pieces of foil.)

    - Lubricate the granules as per BMR.

    - Send the request of the blend for Content Assay.

- Annexure (S)

Annexure –I Recoverable Residue format

- Reference (S)

NA

- Glossary

RR       :		Recoverable Residue.

SOP     :		Standard Operating Procedure

No.      :		Number

QA       :           Quality Assurance

BMR     :           Batch Manufacturing Record

Qty      :            Quantity

QC       :           Quality control

SS        :           Stain less Steel

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |