- Objective

To lay out a procedure for Indent, Receipt, Storage and Handling of Sieves and Screens.

- Scope

This SOP is applicable to sieves and screens used for vibratory sifter, multimill in the production areas of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production Technician: To carry out the activity in accordance with the procedure as mentioned in the SOP.

Production Officer/ Executive: To ensure that the activity is carried out in accordance with the procedure as mentioned in this SOP.

- Accountability

Head Production

- Procedure

  - Indenting

    - The codification for sieve and screens shall be decided at the time of indent, so That party should provide the new sieve and screens with proposed code number. Party should also provide the validation certificate and lead free certificate for Sieves/Screen and non toxicology test report of silicone gasket for sieves.

  - Coding System for Sieves and Screens

    - The coding system for sieves shall be as given below

BP/SS/XXX/YY

Where “BP” indicates Beta Production, SS as sieve size, XXX as Sieve Serial Number & YY as year of purchase respectively.

    - The coding system for Screen shall be as given below

BP/SS/XXX/YY

Where “BP” for screens of Beta block, SS as screen size, XXX as Screen serial number & YY as year of purchase respectively. Check the sieve/screen stock Record and take the new Sieve/screen Code No. which will be followed by last Code No. of the Sieve/Screen.

  - On receipt of new sieve check the following

    - Check the validation certificate, lead free certificate and non toxicology test report of silicone gasket.

    - Diameter-Check with scale.

    - Physical appearance-Check with naked eye.

    - Integrity-Check with naked eye/Integrity test microscope.

    - Mesh Size-Check with Normal Aperture Calibrator.

  - Procedure for checking sieve size

    - For sieve Size 4#to 40#- The distance between adjacent parallel wires is called as Nominal Aperture size (NAS). Nominal Aperture Calibrator is used to measure the NAS. The Nominal Aperture Calibrator has an illuminated scale with a full-scale gradation of 10mm divided in 100 parts. The least count of the scale is 0.1mm or 100 microns.

    - Take measurement as follows

      - The calibrator is placed on the wire mesh & the individual opening is visualized by using the fine-tuning.

      - The distance between adjacent parallel wires is measured in both wrap & weft directions.

      - Nine such readings are taken all over the mesh.

      - The average of these readings is the Nominal Aperture Size of the wire mesh.

      - The NAS is compared with the standard sizes specified in the

      - If it falls within the specified tolerances the sieves is accepted.

    - For sieve Sizes Above 40# up to 200# Take measurement as follows

      - The Calibrator is place on the wire mesh & the no of wires in 2.5mm is counted.

      - Nine such readings are taken all over the mesh.

      - Multiplying this by 10 will give the no wires in a linear inch, i.e. 25.4mm approx.

      - The NAS of the wire mesh is calculated by using the below formula.

      - NAS= [(25.4X1000)/N]-D, Where N=Number of wires per Linear inch & D=Diameter for wire in microns.

      - The NAS is compared with the standard sizes specified in Tolerances on Aperture Widths for Sieves.

      - If it falls within the specified tolerances the sieves is accepted.

  - On receipt of new screens check the following

    - Diameter-Check with scale.

    - Physical Appearance-check with naked eye.

    - Integrity-check with naked eye/Integrity test microscope.

    - Pore Size-Check with Nominal Aperture Calibrator.

    - Welding Quality-Check with naked eye.

  - Screen Size Checking Procedure

    - The Calibrator is placed on the screen & the individual pores are visualized by using the fine-tuning.

    - Measure the diameter of the pores at 10 different locations on screen.

    - The average of these readings in Nominal Aperture Size (NAS) of the screen.

    - The NAS is compared with the standard sizes specified in the

    - If it falls within the specified tolerances the screen is accepted.

    - After compliance of new screen enter details in for Multimill screen.

  - Cleaning procedure

    - Take used sieve/screen to adjacent washroom, in case where washroom facility is not available then pack in a poly bag and transfer if to IPC washroom area for cleaning.

    - Wash the sieve/screen with potable water to remove adhered powder.

    - Scrub the sieve/screen using nylon brush and wash thoroughly with potable water.

    - Then finally rinse with purified water.

    - Dry the sieve/screen with compressed air by applying low pressure.

    - Check the cleanliness and integrity of dried sieve/screen, if cleaning is not satisfactory again clean the sieve/screen.

    - Pack the cleaned, dry, inspected sieve/screen in poly bag and take in “change part room” make Entry in Issuance, usage, cleaning record of sieves/screens Annexure-1.

  - Storage, Handling and Usage.

    - Check for cleanliness and integrity, while receiving and issuance of sieve/screen.

    - Store the cleaned, polythene wrapped, labelled sieve/screen in the sieve cabinet.

    - Designated cabinet in CHANGE PARTS ROOM, Provided for different type of sieves, screens.

    - Record the integrity of sieve/screen before and after use in Batch Manufacturing record.

    - Take the required size sieve/screen from the designated cabinet and enter In “Issuance” Usage, cleaning and Received Record of Sieves/Screens. Annexure-1

  - Destruction

    - In case any damage on sieve/screen the same shall be separated and Labelled as rejected.

    - Production Officer/Executive shall check the rejected Sieve/screen & Verified with QA.

    - Quarantine the rejected Sieve/Screen and raise destruction note collected From QA Dept. After authorization of destruction form, destruct rejected and quarantined Sieve/screen by defacing.

- Reference (S)

Sieves/screen calibration kit manual

- Glossary

SOP		:	Standard Operating Procedure

NO		:	Number

- Annexure (S)

Annexure-I	Sieve /Screen issuance cleaning and return record

- Glossary

SOP		:		Standard Operating Procedure

NA		:		General

IPA		:		Isopropyl alcohol

QTY		:		Quantity

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |