- Objective

To lay down the Procedure for Operation and Cleaning of Multimill.

- Scope

This SOP is applicable to Multimill installed in the production areas of Alkem Laboratories Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

    - Check all the contact parts are fitted properly.

    - Wait for machine to stop fully before changing direction.

  - Pre startup

For same batch continuation

    - Visually inspect the equipment and the area for cleanliness.

    - Ensure that the previous run was of the same batch No.

    - Ensure that the environmental conditions shall be as specified in BMR.

    - Affix the “Status” label on the equipment.

    - Place the empty, clean, pre-labelled product IPC under the discharge chute of the Multimill.

For new batch of the same product

    - Visually inspect the Multimill and area for cleanliness.

    - Ensure that there shall be no remnants of the previous batch.

    - Remove 'Cleaned' label and affix “Status” label.

    - Ensure that the environmental conditions shall be as specified and record it in environmental control record.

    - Place an empty, clean and pre-labelled product IPC of under the discharge chute of Multimill

    - Take the line clearance by IPQA. Officer/Executive as per SOP of line clearance.

For different product

    - Ensure that the final rinse water release report shall be available with the BMR

    - Visually inspect the equipment thoroughly so as to ensure that the remnants of the previous product shall be absent.

    - Remove the "CLEANED" label and affix “Status label” to Multimill.

    - Ensure that the environmental conditions shall be as specified and record it in the   environmental control record.

    - Place empty, clean, pre-labelled product IPC under the discharge chute of Multimill.

    - Take the line clearance by IPQA. Officer/Executive as per SOP of line clearance .

  - Start up

    - Ensure that the main electric supply shall be “ON”.

    - Ensure that all the moving parts and blades shall be fixed properly.

    - Fix the screen as per the requirement.

  - Operation

    - Ensure that the mains are "ON”.

    - Start the machine either in forward or reversed direction as per requirement

    - Charge the product gradually into the charging hopper.

    - Open the slide plate slowly to enter the product into the milling chamber and regulate the input by adjusting the slide valve.

    - Carry out the milling operation as specified in BMR, and record it in respective log book.

  - Shutdown

    - Switch “OFF” the mains after completion of activity.

  - Cleaning Procedure

    - At the end of the shift/Batch to Batch changeover/ changeover of a product with ascending strengths provided the colour to be same. (Type A cleaning)

      - Remove 'In-process' label and affix 'To be cleaned' label to Multimill.

      - Remove all remnants of previous batch/product from the surrounding area and equipment.

      - Remove all powder loosely sticking to shaft blades, S.S. screen, screen holding plate discharge chute and charging hopper with S.S. spatula.

      - Dry clean the Multimill with the help of a vacuum cleaner and lint free duster.

      - Wipe the Multimill and control panel with wet lint free dusters.

      - Remove 'To be cleaned' label and affix 'cleaned' label duly signed by Production Officer/Executive.

      - After five “‘Type A’’ cleaning ‘Type B’’ cleaning shall be performed.

    - When the changeover to another product/changeover to the product with descending strengths or changeover to a product with different colour. (Type B cleaning)

      - Remove “status label” and affix 'To be cleaned' label.

      - Remove all remnants of previous batch/product from the surrounding area and equipment.

      - Remove all powder loosely sticking to shaft blades, S.S. screen, and screen holding plate Discharge chute and charging hopper with S.S. Spatula.

      - Dry clean the Multimill with the help of a vacuum cleaner and lint free duster.

      - Cover the control panel with polythene bag.

      - Cover the Multimill with polythene bag.

      - Dismantle the parts (Shaft blades, S.S. screen, Screen holding plate, Discharge chute and charging hopper) and take it to equipment wash area using trolley.

      - Clean the Multimill with nylon scrubber using potable water and finally rinse with purified water.

      - Dry clean the Multimill using lint free dusters.

      - Clean the dismantled parts of Multimill using potable water and finally rinse 	with purified water.

      - Dry clean the dismantled parts using dusters.

      - Remove the covered polythene bags of Multimill and control panel.

      - Wipe the Multimill and control panel with wet lint free dusters.

      - Fix the dismantled parts of Multimill and set screen as per product 	requirement.

      - Final rinse water sample shall be collected by IPQA Officer/ Executive and 	send to Q.C for analysis.

      - Remove 'To be cleaned' label and affix 'cleaned' label duly signed by         	Production Officer/ Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it will be cleaned 	as per “Type B” cleaning before use.

      - After preventive maintenance, all the contact parts of the equipment to be  	cleaned as per “Type B” cleaning before use.

- Annexure (S)

NA

- Reference (S)

NA

- Glossary

SOP		:	Standard Operating Procedure

NO		:	Number

IPQA		:	In Process Quality Assurance

SS		:	Stainless Steel

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |