- Objective

To lay down a procedure for Operation and Cleaning of Sifter.

- Scope

The SOP is applicable to Sifter installed in granulation area oral manufacturing of Alkem Laboratories Limited, Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

    - During operation if you feel any undue noise, stop the machine immediately and information should be made to engineering department.

    - Vibrator should be carefully handled.

    - While running the sifter lock its wheel with the help of wheel locks.

  - Pre-start Up

    - For same batch continuation

      - Visually inspect the equipment and the area for cleanliness.

      - Ensure that the previous run was that of the same batch number and of the same product.

      - Ensure that the environmental conditions are within limit as per specification.

      - Affix the 'In-process' label on the equipment.

      - Check the sieve integrity visually.

    - For the new batch of the same product

      - Visually inspect the Vibro sifter and area for cleanliness.

      - Ensure that there are no remnants of the previous batch.

      - Remove 'Cleaned' label and affix 'In-process' label.

      - Fix the sieve as specified in BMR after checking its integrity.

      - Ensure that the environmental conditions are as specified and record it in the	environmental control record.

      - Take the line clearance and start the process.

    - For the batches of different products

      - Ensure that the final rinse water release report is available with the BMR

      - Visually inspect the equipment thoroughly, to ensure that the remnants of the previous products are absent.

      - Get the line clearance for the equipment as well as the area from a QA Officer/Executive.

      - Remove the "CLEANED" label and affix 'In-process' label to Vibro sifter.

      - Fix the sieve as specified in BMR after checking its integrity.

      - Ensure that the environmental conditions are as specified and record it in the 	   environmental control record.

      - Take the line clearance and start the process.

    - Start up

      - Take a blank trial for one minute of vibro sifter to ensure proper functioning.

      - Place an empty, clean, pre labeled IPC container lined with poly bag beneath the discharge chute of Vibro sifter.

      - Tie a poly bag to discharge chute of sifter by tag.

      - Bring the container of raw materials to be sifted near vibro sifter.

    - Operation

      - Switch ON the sifter.

      - Ensure the sieve integrity before starting the operation. Affix ‘In-process' label.

      - Remove the lid of sifter and put two scoops of raw materials which to be sifted.

      - Start the sifter and continue sifting by putting the raw material which to be sifted, with the help of scoops.

      - After completion of sifting of individual raw material tie the polybags with tag and affix the Status label with duly signed by Production Officer.

      - Ensure the sieve integrity after completion of sifting.

      - Fill equipment log as per SOP in force.

    - Shutdown

      - Switch OFF the vibro sifter.

      - Dry-clean the vibro sifter with vacuum cleaner and duster.

    - Cleaning Procedure

      - At the end of shift / Batch changeover of same product with ascending strengths (provided colour is same. Type A cleaning)

        - Switch OFF the vibro sifter.

        - Remove 'In-process' label and affix 'To be Cleaned' label.

        - Dry clean the vibro sifter using vacuum cleaner and duster.

        - Dismantle the parts (discharge hopper, sieve, gasket, C-ring and top hopper) and dry clean with the help of dry duster.

        - Fix the dismantle parts to vibro sifter.

        - Remove 'To be cleaned' label and affix 'Cleaned' label duly signed by Production Officer.

        - After five ‘Type A’’ cleaning ‘Type B’’ cleaning should be performed.

      - Cleaning during changeover of product with different active ingredients, colour, descending strengths. (Type B cleaning)

        - Switch OFF the vibro sifter.

        - Remove 'In-process' label and affix 'To be cleaned' label.

        - Dry clean the vibro sifter using vacuum cleaner and duster.

        - Dismantle the parts (discharge hopper ,sieve, gasket, C-ring and top hopper) and  take it to equipment wash area.

        - Wipe the body of vibro sifter using wet duster.

        - Clean the dismantled parts (discharge hopper, sieve, gasket, C-ring and top hopper) using 	sufficient quantity of potable water and finally rinse with purified water.

        - Dry clean the dismantled parts beneath the IR Dryer.

        - Fix the dismantled parts to vibro sifter and cover with Poly bag.

        - Ensure that the final rinse sample is collected by IPQA Officer/ Executive.

        - Remove 'To be Cleaned' label and put 'Cleaned' label duly signed by Production Officer.

        - If the cleaned equipment remains unused for 72 hrs, then it will be cleaned as per “Type B” cleaning before use.

        - After preventive maintenance, all the contact parts of the equipment to be cleaned as per “Type B” cleaning before use.

  - Troubleshooting

    - If during operation movement of sifter occur, stop the machine immediately and adjust the vibrations of the vibrator.

    - Always use the gaskets to avoid the leakage of powder during operation.

- Annexure (S)

NA

- Reference(S)

Machine manual

- Glossary

SOP		:   Standard Operating Procedure

No.		:   Number

BMR		:   Batch Manufacturing Record

S.S		:   Stainless Steel

IPQA 		:   In process Quality Assurance

IPC		:   Intermediate Product container

NA		:   Not applicable

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |