- Objective

To lay down a procedure for Operation and Cleaning of Tray Dryer.

- Scope

The SOP is applicable to Operation and Cleaning of Tray Dryer installed in manufacturing area of Alkem Laboratories Limited, Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

    - Never on the machine when door is open.

    - Never open the door when the machine is on running position.

    - Never close the inlet and outlet valve when machine is on running position

    - During operation if you feel any undue noise, stop the machine immediately and information should be made to engineering department.

  - Operation

    - Check the area for cleanliness and ensure that there shall be no remnants of previous product.

    - Ensure that all the switch in control panel is “OFF” position.

    - Open the door of the Tray Dryer and load the filled tray into the tray dryer.

    - Close the door of the tray dryer.

    - Open the Inlet and Outlet Valve corresponding Inlet Air and Outlet Air unit.

    - Switch on the main power supply by switch on the “MAIN SUPPLY ON”

    - Switch on the pump by switch on the “PUMP ON”

    - Start the exhaust blower by switch on the “EXHAUST BLOWER ON”. To get the temperature switch on the “STEAM ON”.

    - Start the main blower by pressing the green colour “MAIN BLOWER ON”.

    - Set the temperature as required by pressing the temperature controller button on “PID Controller”.

    - Set the time as required by pressing the time controller button on “Process Timer”.

    - In case of any emergency press red coloured “EMERGENCY STOP” switch.

    - When drying will complete switch off as follows: “STEAM ON”, “PUMP ON”.

    - Before unload the dried materials switch off as follows: “MAIN BLOER OFF”, “EXUST BLOWER ON”, “MAIN SUPPLY ON”.

    - Open the door of the tray dryer and unload the dried powder from trays.

  - Cleaning

    - Cleaning during new batch of the same / similar product with same or ascending strength is to be taken, provided colour is same (Type A cleaning).

      - Switch ‘OFF’ the machine.

      - Remove “IN PROCESS” / “STATUS” label and affix “TO BE CLEANED” label.

      - Clean the Tray Dryer and trays using dry lint free cloth.

      - Check cleanliness of trays and tray dryer.

      - Remove “TO BE CLEANED” label and affix “CLEANED” label duly signed by production officer/ Executive after verification from IPQA personnel.

    - Cleaning during different active ingredients, colour, flavour and descending strength (Type B cleaning).

      - Switch ‘OFF’ the machine.

      - Remove “IN PROCESS” / “STATUS” label and affix “TO BE CLEANED” label.

      - Dry clean the Tray dryer and trays using lint free cloth.

      - Dismantle the trays and take it to wash.

      - Clean inner and outer part of the tray drier and also the trays using potable water and finally rinse with purified water.

      - Dry clean the parts with the help of dry lint free duster.

      - Fix the all dismantle parts to equipment.

      - Ensure that the final swab / rinse sample, if required is to be collected by IPQA personnel.

      - Remove “TO BE CLEANED” label and affix “CLEANED” label duly signed by production officer/ Executive after verification from IPQA personnel.

      - If cleaned equipment remains unused for 72hrs, then it shall be clean as per type “B” Cleaning.

      - After preventive maintenance, all the contact parts of equipment shall be cleaned as per type “B” cleaning before use.

      - After four consecutive batches of same / similar product “B” type cleaning shall be performed.

- Annexure (S)

NA

- Reference(S)

NA

- Glossary

SOP		:   Standard Operating Procedure

No.		:   Number

IPQA		:   In Process Quality Assurance

NA		:   Not applicable

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |