- Objective

To lay down a procedure for Operation and Cleaning of Compression Machine.

- Scope

This SOP is applicable to Compression Machine installed in the production areas of Alkem Laboratories Ltd., Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is carried out as per SOP.

- Accountability

Head Production

- Procedure

  - Precautions

      - Don't rotate the machine in the anti-clockwise direction.

      - Ensure the required overload pressure shall be given in hydraulic system before operating the machine.

      - Ensure the proper machine setting before starting the operation.

      - Do not keep open the safety guard during running machine.

      - Do not touch moving part of machine during running.

  - Pre startup

    - For same batch continuation

      - Visually inspect the machine and the surrounding area for cleanliness.

      - Ensure that previous run was of the same batch.

      - Ensure that the material lying in the room shall be of the same batch.

      - Put the "Status" Label on the machine.

      - Ensure that the environmental conditions shall be as per specified in BMR and record it in environmental control record.

      - Ensure the proper setting of the machine by rotating the hand wheel.

    - For the new batch of the same product

      - Visually inspect the equipment and the room for cleanliness.

      - Ensure that there shall not be any remnants of previous batch.

      - Put the “Status” Label on the equipment.

      - Ensure that the environmental conditions shall be as per specified in BMR and record it in the environmental control record.

      - Ensure the proper setting of the machine by rotating the hand wheel.

      - Take the line clearance from IPQA Officer/Executive.

    - For the different product

      - Visually inspect the equipment and the room for cleanliness.

      - Ensure that there shall not be any remnants of previous product.

      - Ensure that the swab release report shall be available with BMR.

      - Ensure that the environmental conditions shall be as per specified in BMR and record it in the environmental control record.

      - Ensure the proper setting of the machine by rotating the hand wheel.

      - Take the line clearance from IPQA Officer/Executive.

  - Start up

    - Switch ON the mains.

    - Switch ON the dust collection system.

    - Ensure that the lubrication system shall be functioning properly.

    - Ensure that the turret guards, Force feeder assembly, 'Y' chute and discharge chute shall be fixed properly.

    - Check the status label of the bin which shall be started.

    - Ensure for the release status of the blend.

    - Place the BIN of Lubricated granules which shall be compressed over the 'Y' chute in BIN feed area.

    - Open the butterfly valve of BIN and allow the granules to flow into the hoppers of machine.

    - Set the overload pressure (approx. 6 tones).

    - Place empty, clean pre labelled HDPE containers below the discharge chute lined with double polythene bag.

  - Operation

    - Ensure that all the on line machines shall be assembled along with compression machine.

    - Ensure that the clutch shall be disengaged. Put the feeder control switch at “MANUAL” position.

    - Press the left and right feeder control switches so as to allow the feeder to get filled with the granules.

    - Rotate the compression machine manually by rotating the hand wheel provided on the right side of the machine so as to spread the powder in die holes.

    - Turn the switch of feeder control to “AUTO” position. Press the "START" button to start the main motor.

    - Engage the clutch slowly and Set the physical parameters for compressed tablets as per specified in BMR.

    - Keep the tablets collected during initial two rotation in a HDPE drum and put “NON RECOVERABLE” Label duly signed by Production Officer/Executive.

    - Set the parameters as specified in BMR and collect tablets in HDPE drum lined with polybags and put “RECOVERABLE” Label duly signed by Production Officer/Executive.

    - Ensure that all the physical parameter set as per specified in BMR then carry out the Compression operation.

    - Check the parameters during the whole process as per specified in BMR and record the same in BMR.

  - Shutdown

    - Switch “OFF” the mains.

    - Switch “OFF” the dust collection system.

    - Release the overload pressure completely.

    - Dry clean the machine using vacuum cleaner and dusters.

  - Cleaning Procedure

    - At the end of the shift/ batch changeover of same product and strength or ascending strength (provided colour is same) - Type A cleaning.

      - Switch ‘OFF’ the main electric supply.

      - Remove "Status" Label and affix 'TO BE CLEANED' Label to the equipment duly signed by production Officer/Executive.

      - Remove the Force feeders.

      - Dry clean the compression machine using vacuum cleaner and duster.

      - Dry clean the Force feeders and 'Y' chute using vacuum cleaner and duster.

      - Dry clean the hoppers using vacuum cleaner and duster.

      - Clean the upper punches and the punch holes.

      - Clean the area as per SOP in force.

      - Fix the Force feeders and 'Y' chute to the machine.

      - Remove 'TO BE CLEANED' Label and affix 'CLEANED' Label duly signed by Production Officer/ Executive.

      - After five “Type A” cleaning “Type B” cleaning shall be performed.

    - Cleaning during changeover of product with different active ingredient, colour, descending potency - Type B cleaning.

      - Switch 'OFF' the mains. Remove "STATUS" Label and affix 'TO BE CLEANED' Label to the Equipment. Duly signed by Production Officer/ Executive.

      - Dry clean the machine with vacuum cleaner and duster.

      - Dismantle the parts of the machine ('Y' chute, Hoppers, Discharge chute, Force feeder) and take it to equipment wash using trolley.

      - Wash the dismantled parts ('Y' chute, hoppers, force feeders, discharge chute) using potable water and finally rinse with purified water.

      - Dry clean the dismantled parts using dusters.

      - Ensure that the Swab sample shall be collected by IPQA Officer/ Executive Remove the turret guards by removing the screw.

      - Remove the cam piece then remove the upper punch by lifting them upward.

      - Remove the lower punch by loosening the anti turning strip then removing the nylon plug.

      - Unscrew the Die locking screw.

      - Remove the Dies by gentle tapping the Dies from below using die lever.

      - Clean the Upper and Lower punches, Dies, Die locking screws, anti-turning strips and plugs with IPA.

      - Clean the turret and machine below turret using IPA solution.

      - Clean the Upper and lower punch holes using a duster dipped in IPA with the help of bottle brush.

      - Dry clean all the above parts using dusters. Dry clean the feeder motors using duster.

      - Fix the Dies by gently tapping from the above with Die lever.

      - Ensure the tightening of die locking screws.

      - Fix the lower punches and tighten them by using plug and anti-turning strips.

      - Ensure the tightening of lower punch holding plug. Fix the upper punches.

      - Fix the dismantled parts ('Y' chute, hoppers, force feeders, discharge chute) onto the machine.

      - Remove 'TO BE CLEANED' Label and affix 'CLEANED' Label duly signed by Production Officer/ Executive.

      - If the cleaned equipment remains unused for 72 hrs, then it shall be cleaned as per “Type B” cleaning before use.

      - After maintenance all the contact parts of the equipment shall be cleaned as per “Type B” cleaning before use.

- Annexure (S)

NA

- Reference (S)

NA

- Glossary

SOP		:	Standard Operating Procedure

NO		:	Number

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |