- Objective

To lay down a procedure for operation and cleaning of Infra Red Drier.

- Scope

The Procedure is applicable for Operation and Cleaning of Infra Red Drier in Granulation area manufacturing area of Alkem laboratories Ltd. Unit-II, Baddi.

- Responsibility

Production technician: To operate and clean the machine as per procedure mentioned in SOP.

Production Officer/Executive: To ensure that operation and cleaning of machine is done as per SOP.

- Accountability

Head Production

- Procedure

  - Precaution

    - During operation if you feel any undue noise, stop the machine immediately and information should be made to engineering department.

  - Prestart up

    - Ensure the cleanliness of Infra Red Drier.

    - Adjust the height of Infra Red Drier as per the requirement.

  - Operation

    - Switch on the main.

    - Start the infra Red Drier.

    - Place the equipment which is to be dried below the drier.

    - Press” MENU” then select blower on time by pressing up and down key.

    - Press start key it shows heater on time then select heater time by pressing up & down key.

    - Display shows ‘Manual Mode Alert’ then press “BL” key to start blower on or press ”“HT”  key to start heater.

    - After completions of drying switch off the Infra Red Drier and ensure that the cooling fan is on for another five minutes.

  - Shutdown

    - Switch OFF the Infra Red Drier.

  - Cleaning Procedure

    - Wipe the body of Infra Red Drier using wet duster.

    - Dry clean the Infra Red Drier using dusters.

- Annexure(S)

NA

- Reference(S)

NA

- Glossary:

SOP		:	Standard Operating Procedure

No.		:	Number

IR		:	Infrared dryer

NA		:	Not applicable

- Revision History

| Revision No. | Details Of Changes | Reason for Change |
| --- | --- | --- |
| 00 | New SOP | NA |