- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for handling of sterile raw materials.

# 3.0    Scopes

This SOP is applicable for the handling of sterile raw materials of C Block.

# 4.0    Responsibility

Officer and Above Production	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

- Procedure

  - Ensure that LAF in aseptic area are operational and differential pressure is within specified limit (8 to 15 mm of WC).

6.2 Raw material containers to be received from warehouse on hospital trolley in the raw        material staging area.

6.3   Container of sterile raw material to be sanitized by 70% IPA (use before 72 hours) solution before transfer to the Dynamic pass box.

6.4   Allow the containers to remain under UV lights & Laminar Air Flow for NLT 10 minutes.

6.5   Open the Pass box and sanitize the containers by freshly prepared 70% IPA solution thoroughly and store under LAF on SS pallets in filling area.

  - Take the sterile raw material containers to the vial filling room using hospital trolley.

6.7   Sanitize the container by using freshly prepared 70% IPA before opening the lid of the container for the use of API.

Note: Open the lid of container under LAF only.

6.8    Remove the material from the container by using pre sterilized SS scoop and transfer to

hoppers of vial filling machine under Laminar air flow devices.

6.9    Material should not be exposed to the environment condition for long period of time after

Opening of the container. Ensure the temperature and humidity of the room is under specified

limit as per product requirement.

7.0   Annexure (S)

NA

8.0  Reference (S)

NA

9.0  Glossary

SOP   		: Standard Operating Procedure.

IPA			: Isopropyl Alcohol

NO      		: Number

LAF			: Laminar Air Flow

API			: Active Pharmaceutical Ingredient

NLT			: Not less than

UV			: Ultra violet

SS			: Stainless Steel