- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | 1. Increase hold time study of freshly prepared 70% IPA. | Due to change hold time study from 48 hours to 72 hours of 70% IPA. |

2.0    Objective

To lay down a procedure for defining aseptic area practices.

3.0    Scope

This SOP is applicable for the persons working in the aseptic area and for following the aseptic area practices while working in the aseptic areas of C Block.

# 4.0     Responsibility

Officer and Above Production/IPQA Officer	:	Implementation of Procedure.

5.0	    Accountability

Production Head.

6.0	Procedure

6.1	   Do’s

    - While entering in aseptic area person should enter one by one at a time.

6.1.2 Only one door of the area should be opened at a time.

6.1.3 Wash the hand in first air lock with disinfectant solution using sterillium.

6.1.4 Maintain proper gown control.

6.1.5 Movement in aseptic area should be minimum and talking should be minimum and when required.

6.1.6 Man & material movement of the area should be followed as per Sop’s PC/067 & PC/068.

6.1.7 Keep the LAF unit operational during operation/storage.

6.1.8 Place the sterile component under LAF in closed condition.

6.1.9 Disinfect hand in aseptic area with freshly prepared 70% IPA (use before 72 hrs) before touching           any sterile item.

6.1.10 Contact sterile materials only with sterile instruments.

6.1.11 Keep the entire body out of the path of unidirectional airflow.

6.1.12 Area should be cleaned daily as per SOP No PC/057 using filtered disinfectant solution as per schedule.

6.1.13 Remove the remaining materials after batch completion.

  - Don’t

6.2.1 Do not move fast in the aseptic area.

6.2.2 Do not gather in one room of the aseptic area.

6.2.3 Do not open two door of the area at a time.

6.2.4 Do not enter in area through 1st air lock to 4th air lock.

6.2.5 Do not expose the component outside the LAF.

6.2.6 Do not carry out production operations when temperature and humidity condition are beyond the     specified limits. (Temperature NMT 28 ºC and RH NMT 40 %)

- Annexure (S)

NA

8.0  Reference (S)

Sop No. PC/067

PC/068

PC/057

9.0  Glossary

SOP		: Standard Operating Procedure.

LAF		: Laminar air flow.

HEPA 		: High efficiency particulate air filter.

RH		: Relative humidity

NMT		: Not More Than