- Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | NA | Periodic Review |

2.0    Objective

To lay down a procedure for filtration of IPA

- Scope

This SOP is applicable for filtration of IPA used in aseptic areas of C Block.

# 4.0	  Responsibility

Technician			:	Filtration.

Officer and Above Production	:	Implementation of Procedure.

5.0	   Accountability

Production Head.

- Procedure

6.1   Ensure that availability of pre sterilized 0.22 membrane filter with filtration assembly, pressure vessels, silicon tubes and SS clamps.

6.2   Ensure that the availability of freshly prepare 70% IPA solution in pressure vessel.

6.3   Connect the silicon tube to up stream of filtration assembly and then connect the other end of tube to out let  of pressure vessel containing 70% IPA.

6.4   Connect the silicon tube to down stream of the membrane filtration assembly and connect the other end to the filtrate collection pre sterilized pressure vessel.

6.5  Connect compressed air tube to inlet of the pressure vessel containing 70% IPA to be filtered.

  - Tie the clamp at the joints to avoid spoilage during processing.

  - Ensure that all the connection clamps are firmly tight.

6.8  Open the valve of compressed air line and increase the pressure from pressure regulator up to 1.5 kg/cm2  and build up the air pressure in pressure vessel.

6.9  Open the air vent valve of membrane filtration assembly to remove the air bubbles so that the rate of filtration remains satisfactory.

6.10 Carry out the operation till all the 70% IPA solution gets filtered.

6.11 Check the filter integrity as per SOP No PC/095 before and after filtration.

  - Perform the filtration activity under LAF.

7.0    Annexure (S)

NA

8.0   Reference (S)

SOP No:  PC/095

9.0   Glossary

SOP	    :	 Standard Operating Procedure.

LAF	    :	 Laminar air flow.

IPA	    :	 Isopropyl Alcohol

No.       : 	Number

SS	    : 	Stainless Steel.

PC	    :	 Production C-block