1.0    Revision History

| Rev.No. | Details of changes | Reason for change |
| --- | --- | --- |
| 01 | NA | Periodic Review |
| 02 | Increase hold time study of Sterilized garments. | Due to change hold time study from 48 hours to 72 hours of garments. |

2.0   Objective

To lay down a procedure for storage of aseptic area garments and surgical gloves after         sterilization.

3.0   Scope

This SOP is applicable for the aseptic area garments in C Block.

4.0   Responsibility

Officer and Above Production	:	Implementation of Procedure.

5.0	Accountability

Production Head.

6.0    Procedure

6.1	Ensure the cool zone LAF and garment cubicle are in working condition and pressure differential is within the specified limit (8 to 15 mm of wc) prior to unloading.

6.2	 Ensure cleanliness of cool zone.

6.3    Sanitize your hand with 70% IPA before making any contact to sterile articles.

6.4    On completion of sterilization of aseptic area garments and surgical gloves open the door of autoclave towards aseptic side.

6.5    Fix the autoclave floor trolley to the rails of the autoclave, lock it, and pull the autoclave trolley on the rails of the floor trolley under LAF.

  - Lock the autoclave trolley with the floor trolley.

  - Bring a pre-sanitized hospital trolley under the concerned LAF.

  - Unload aseptic area garments and surgical gloves on the surgical container.

6.9    Check for dryness of the garments, If not adequately dried then load the same in the autoclave  and dry in manual mode.

  - Remove the lock of autoclave trolley, push the trolley inside the autoclave and close the door of the autoclave.

6.11  Move the surgical container to change room II through change room IV to Ist and then to IInd  and unload the garments and surgical gloves in garment cubical.

6.12	 Hang the gowns and hoods in hangers and keep the booties and sterilized gloves on the shelves			 provided in the garment cubicle.

6.13	Ensure UV light and LAF is operational during storage of the aseptic area garments and sterilized gloves.

  - Sterile garments and sterilized gloves can be stored in garment cubicle maximum up to 72 hours.

7.0    Annexure (S)

NA

8.0    Reference (S)

NA

9.0    Glossary:

SOP   	: Standard Operating Procedure.

LAF	: Laminar air flow.

NO        : Number

PC         : Production (C-Block)